Automotive Communication Systems (S1-26_AELZG513)-20260725_154102-Meeting Recording

July 25, 2026, 10:11AM

1h 59m 58s

Shree Prasad M .   0:03
I'll also allow mics for everyone so that in case if you are having any doubts, you can unmute and raise your queries. Of course, I appreciate if you unmute only when you are speaking because otherwise if you are on unmute, the background noise can actually cause disturbance in the.
Class.
OK, I think we can get started at least with the some generic discussion.
So, just you know to start with, what when we say Automotive Communication Systems, what is that that comes to your mind?
Have you worked probably anything related to Automotive Communication Systems in your domain or probably your workplace or else is it something which you are already working on or how is it?
And I appreciate if the class is interactive, because that does what makes the learning possible. It has to be a two-way traffic, not an one-way traffic. So I appreciate in case if we can answer on the chat box.

JAGADESHAN K .   0:59
Ohh.

Shree Prasad M .   1:18
Anything related to, okay, so like INCA tools, can protocol, OBD, yes, can. I think I would see, you know, a lot of answers related to CHANDAN, LIN, and maybe other sorts of protocol. Any other, you know, protocol which you think, yes, J1939, yes, that is also.
A good example. So do you think UDS, Unified Diagnostic Services, CAN Classic, CAN fdfde, yes, all good, yes. Okay, there is also one interesting answer in chat box, SPA I2C, CAN UR.
OK, and also someone says, or is this one says anything that helps subsystems communicate fine, yes?
Communication between sensors, actuators, controls, dashboard using different protocols, FlexRay and CHANDAN. OK, so I would probably consider more or less, you know, different protocols, which you know, probably our thinking is would be only between.
ECU to ECU communication. Say for example, like CHANDAN, LIN, you know, what are Ethernet and probably it's not that you know many of you are unaware. Nonetheless, you know, probably will go through these protocols in this course. So apart from this, government of India has identified the...
some technology recently to ensure there won't be any, you know, there won't be any sort of accidents or else it will reduce the number of accidents. So whatever technologies that was mentioned so far in the chat box was mainly for, you know, efficient functioning of or probably enabling.
certain functionalities to work properly. Say for example, in order to meet certain pollution norms like BHARAT stage four or five, you know, it is very imperative to have an ECU. We'll see what are actually an ECUs and then communication amongst those ECUs. So this all actually happens inside the car.
No, but for us to reduce, you know, the accidents among the vehicles, is there any technology that we can consider, probable technology, any comments in the chat box?
OK, finally, there is one answer which we call it as V to V, and many of you are saying functional safety. Of course, functional safety is something which...
You know, I would say saying, you know, whether a intended function is working correctly or not, that would be you would be shedding in your radars or whatever it is. Now, we have onboard diagnostics that is for diagnosis of the vehicle or whatever it is. We have connected cars and then there are a couple of few answers which says...
V2V, so which is vehicle to vehicle communication or V2X, which is vehicle to everything communication. And some of you are mentioning about assault rating as well. Yeah, probably there are assault A, B, C, D. I think most of you, you would be more aware of, you know, what this assault ratings are and that actually.
Says, you know what will happen if a functionality fails and so on and so forth. Of course, you know, telematics as well. OK, now.
Why, you know, we would require, you know, two vehicles to communicate when already we have a DAS features? Any comments in the chat box?
Why we would want, you know, vehicles to communicate when we already have ADAS or probably, you know, advanced driver assistance systems. Any comments in the chat box?
OK, we'll we'll can't see beyond forward vehicle. That is a good answer. OK, all vehicles running in the same rd. Yes, OK. This is also in case of AB, it is better to rear vehicle toward near collision, avoid collision. OK, Adas is safe. Yes, even V2V is also.
comes under ADAS, but in ADAS we can have two different categories. One is onboard sensors and then which we can have, you know, communication itself acting as a sensor that we will see in today's class. Okay, for 360 degree awareness or whatever it is. Okay, now before I get into the details of, you know,
specifics of this particular course, what will be discussed on what you can expect. I know we know what would be the answer for the question, but still I intend to ask this question to for a couple of reasons, which I'll say after asking this question. So how many of you have taken communication subjects earlier?
Or how many of you are aware of the basics of communication? Or else, you know, to be more generic, how many of you are from a mechanical background or how many of you are from electrical background?
Okay.
Mechanical, OK, electrical, mechanical, mechanical, OK, so even for those of you who have taken electronics, so how many of you have studied about what do you call as this Communication? Of course, you know, I do totally understand, so I assume you know the class is more or less from.
Mechanical background. OK, so then this brings to the question, so you know what topics I should be knowing, or else how is it like, you know, how to manage this course and all. See, one thing I'll make, I want to clarify in the start of the class itself, I'll not be.
expecting any prerequisites from your end.
There are no prerequisites required for this particular course. The interest or, you know, the time that you invest to learn this course are only the prerequisites that you require for this. Whatever is required in order to understand this particular course will be given as and when I consider that particular course.
topic. So for students from electronic background, it may sound a little familiar or it may be for them, for you also, it may be a little, you know, you know, maybe you may not be aware of this particular topic, but I'll have to consider, you know, what we call as the...
You know the prerequisites before I get into the any details of this particular topic, because that is very much essential for this particular course. OK, so what is that you can expect in this particular course? Let me start sharing the screen.
Just a second.
Just give me a minute.
T.
So it started sharing the screen, so this presentation will be uploaded.
So, today's class would be mainly the outline, and then probably I will try to consider a small use case through which I'll try to explain the concept, okay, as to why feedworks is important. So, this is called as a platooning application.
Okay, through use of platoning. And I don't think that, you know, we are discussing something which is futuristic, which may or may not be realised. Platoning is already implemented. I think it would be running in some of the European countries, although you may not see in India, because you will understand why it.
Why it is difficult to realise a platooning application in India? OK, so this is roughly, you know, what I intend to discuss in today's class: need for vehicle to be a vehicular communication and what actually is DSRC and what does cellular V2X. I'll be explaining what is platooning later.
And probably, I'll just give a skip for this enhanced perception or collective perception, and apart from this, I'll also be to a certain extent discussing, you know, the intra-vehicular communication.
the intra-vehicular communication, which is mainly the communication among ECUs in the vehicle. So this time I intend to actually differentiate between the two different kinds of architectures, one which is a signal-oriented architecture.
Okay, signal-oriented architecture, and then the other one is nothing but the service-oriented architecture. Okay, so all these aspects will be considered in a while. Okay, now let's go ahead. Let's consider now two different scenarios here. So, one traffic scenario is at the...
Intersection, as you can observe. Let's observe the green truck is the vehicle that you are driving, which we call it as the ego vehicle. Ego vehicle means the vehicle that you are driving. Ego is self ego vehicle, so ego is.
self. Always the vehicle that you're driving is called a self and then this vehicle is nearing the traffic junction and then its intention is to take left or else take left turn. And then there are two possibilities and then you can see the arrows here which indicates the direction in which.
the vehicles have to take turns. And at this, we can see the blue dot here indicates the pedestrian. And then at the corner of this intersection, there can be a tree or there can be a high-rise building as well.
This is scenario one. So this is a high-rise building. And then the other scenario is the box in the right, which is red box or red colour one. So this indicates an emergency vehicle, or else a vehicle which intends to break the.
We may have ADAS features on this truck installed or vehicle installed. Say for example, it can have the onboard sensor, which I'll call it as OBS. The onboard sensors here are, say for example, it would be a radar sensor. It's an onboard sensor.
Or else it can be a LIDAR sensor or it can be camera sensors as well, so there are...
three different onboard sensors on this particular ego vehicle. Now comment in the chat box whether the onboard sensors on this particular ego vehicle one can it detect.
the presence of, say, for example, the emergency vehicle or the pedestrians? And second, can it directly make out what is the intent of, or else can it convey the intent of its action to the pedestrian or else to the emergency vehicles? Comment in the chat box. So one answer is no.
Why the onboard sensors on the ego vehicle? Why it cannot detect the presence of the pedestrian or probably the emergency vehicle? What may be the reason?
What is the reason?

MOHAMED SHAFWAN S .   14:31
Because.
Sorry, sir, because it might not be in the range of that census.

Shree Prasad M .   14:39
Maybe not range, but...

MOHAMED SHAFWAN S .   14:41
The rate is, yeah.

Shree Prasad M .   14:43
Maybe I think the better term would be to use field of view.

JAGADESHAN K .   14:44
List.

MOHAMED SHAFWAN S .   14:47
Okay, cool.

Shree Prasad M .   14:47
The range, the range can still, it can have within the detection range, but it may not be in the field of view because of this reason, you know, you know, it cannot detect. Say, for example, you know, the camera sensors field of view is as shown over here. I think you'll be studying all these aspects in your ADAS.

MOHAMED SHAFWAN S .   15:03
Mm mm.

Shree Prasad M .   15:06
But even it may have the detection range or probably it may have the field of view also, but its visibility or else its viewing angle would be blocked by probably a tree or probably an high rise building also. Okay, so that is the problem. So in this case, you know, probably, you know, the blockage of the view also isn't.
you know a problem over here. And also the other thing is the sensors, onboard sensor can only detect the presence of the pedestrians or probably the, you know, the oncoming emergency vehicle. But the problem here is it cannot detect or else directly get the intent of what these...
you know, actors in the road. When I say actors in the road, actors in the road, so that those are nothing but the pedestrians and then the vehicles. Okay, it cannot directly detect or else make out the intent of the pedestrians or the vehicles. This is the problem. Okay, so this is 1 scenario.
And then the next scenario is I think most of us would have faced this problem. So the blue colour vehicle, the vehicle that you are driving, and it's a two lane rd, you are just behind a truck. Okay, so this we call it as the preceding vehicle. The vehicle that is ahead of us is called as the preceding vehicle.
And there is another vehicle on the adjacent plane. So this is called as the oncoming vehicle and its direction of moment is specified. I have placed an arrow. Of course, you know, the direction of travel is indicated by the dotted lines here. Now,

SRIKANTH E .   16:40
The.

Shree Prasad M .   16:47
The ego vehicle.
You know.
Forget about, you know, the onboard sensors. Will there be any confusion in terms of, you know, when you want to overtake, you know, the preceding vehicle? So the intended path that you wish to take here is overtake this particular vehicle for which in any case we will have to go to the next lane, then again overtake the vehicle.
And then again, come back to the.
lane in which you are supposed to drive. So how dangerous it is or how safe is this? Comment in the chat box.
Actually, it is very dangerous because it's a very unsafe situation because you are behind a truck wherein your view is completely blocked by the height of the truck. So you don't know, basically you are in the blind spot. So it's very difficult to overtake this particular vehicle. Now let's...
You know.
You know, add onboard sensors for this ego vehicle. Maybe you can add a radar or probably a camera. Now with onboard sensors, can these sensors provide...
what you call as the warning to the ego vehicle about the oncoming vehicle. Comment in the chat box.
Is it really possible for radar to provide or camera to provide the presence of the oncoming vehicle?
If you observe carefully.

JAGADESHAN K .   18:22
Oncoming vehicle will not be there. It will not be there. Oncoming vehicle for PV, it will.

Shree Prasad M .   18:26
It does not.
It is blocked. The vision is actually blocked by, you know, the preceding vehicle, the field of view. That is the problem. That is the field of view is blocked. See, this is similar to, say, for example, in highways, specifically during in northern part of India, during specifically winter time.
Do you think, you know, cameras can detect, you know, the vehicles ahead of it in presence of extreme fog conditions? And we have seen a chain of accidents as well, right? I think it should be in Yamuna and Armada Highway or something where it was, you know, most of the cars were rear-ending each other, you know, because you know, there is so much fog that, you know, you don't know, you know, whether there is a car or not. Okay, so whether...
radar or camera works or not, you know, it's a difficult question to answer. But so in these cases, what we observe is the onboard sensors have its own limitations. Yes, it works good, but the point is whether is there any add-on technology which can...
actually not only detect the presence of the objects here, but also understand the intent of that particular object. Okay, so this is what we call as vehicle to vehicle communication. Say for example, this ego vehicle is communicating with the emergency vehicle and the ego vehicle here is communicating with the pedestrian.
Or is it can be any vulnerable rd users which can be bicyclist as well. Okay, cyclists as well. Now what it communicates.
Okay, what is that it would be communicating? One, it would be communicating the vehicle kinematics. Say, for example, for the ego vehicle, it will say that, okay, what is its current velocity? What is its GPS coordinates? Say, for example, what is its latitude or what is its longitude? And whether it has turned on the...
you know, the turn indicators, say for example, that is, I will call it as the yaw moment. Okay. So say for example, if it indicates or else when before we take a turn, we turn on the indicator. So that is also actually transmitted and we can also transmit what is the length of the vehicle, so on and so forth. And probably we can also say that, okay, we
If there is a special vehicle, we can say that there is an emergency vehicle. So, in this way, see, we are not only transmitting the latitude and longitude, but we are also transmitting the intent that is the OK, and this is actually given to the pedestrian, this is given to the emergency vehicle through which we can understand.
what you know the eagle vehicle intends to do and then the pedestrian is also made aware of the ego vehicle's intention to take right and then the pedestrian can be careful when he or she wants to cross over to the other side of the road. So this is 1.
typical advantage of V2V communication. Please note that now, so far I've been saying that, say for example, radar also works on, say for example, radio waves.
It also works on the principle of radio waves or using radio waves. Any comments in the chat box? What is the frequency at which the radar operates, which are already there on many vehicles, which you can see in India as well? 77 hertz or 77 gigahertz.
It does 77 gigahertz. Earlier it was 24 gigahertz. Okay.
Okay, and it was earlier it was, you know, 24 gigahertz. Now we are saying V2V communication. V2V communication is also using radio waves. Radio waves are transmitted for, you know, it is vehicle to vehicle communication. These are also for.

DHANUSH S .   22:13
Seventy-five.

Shree Prasad M .   22:33
These are also radio waves. Now, why is that we are not saying that V2V, when V2V, which is using radio waves, are not blocked by, you know, probably the high-rise buildings or probably the other entities. What may be the reason?
What could be the reason?
Material, OK, fine.
No signal interruption, OK.
No signal interruption wavelength. OK, even you have a V2V communication these days, we know which operate at high wavelength also. OK, but nonetheless, to answer for your question, or else to answer for this particular question, see, in case of radars, these are what are called as active sensors. OK.
These are what are called as active sensors, which operates on the principle of transmission, reflection, and then, you know, what you call as then detecting it. OK, so it is it goes in a very focused direction, and then I cannot say that, you know, it's because of, you know, the range is restricted. That is not the reason, but...
It is because basically the way the signal actually works, or else you know, it completely depends upon the way the sensor works. But in case of V2V communication, now primarily which operates in 5.9 gigahertz, okay, so this is what is called as ITS band. I'll come to this later. Okay, so 5.9 gigahertz, so...
in this particular scenario. And also remember that lower the frequency, lower the frequency, larger is the transmission distance. Now, right now, probably you may be watching your this lectures on mobile phones or probably on your laptops using mobile hotspot.
So your mobile phones may not be in the direct line of sight of the cell phone towers because the building walls will obstruct, you know, the radio waves. Even then you are receiving the signal. This is what actually happens in case of vehicle to vehicle communication. Still, we'll be able to receive some basic information. This is enough.
for us to take, you know, the proper decision, both in this particular scenario also, that is in the highway overtaking scenario also. Okay. So this is the need for, you know, what we call as where vehicles to vehicles communicate, we call it as vehicle to vehicle communication or else if vehicles communicating with
pedestrian, it is called as vehicle to pedestrian communication. Okay, now this is how a typical, you know, what you call as a, which would be there on a car would appear. So there are different cameras, LIDARs, and this is a suite of sensors which
Are used for realising now what you call as a any of the ad functionalities, and then what would be the responsibility of any ad functionality? You know, what are the stages? One is the perception that is, we perceive the environment. So, so far, you know, probably you are aware of, you know, the way the environment is perceived, so either it can be.
Radar, either it can be Lidar or else it can be camera. So, this is what I call as RLC. OK, just like the electrical network depends upon the RLC components, that is resistor, inductor, and capacitor. So far, for the perception, the vehicle heavily depends upon...
radar, lidar, or camera. And for any application, there is no probably a standalone solution or a silver bullet saying that, okay, with radar, we can implement all these, we can implement an adaptive cruise controller. No, we'll have to go for a combination of these sensors. Now, then there would be planning accordingly.
Uh, you know, and then there are nothing but what is called as actuation now.
This onboard, these are what are called as onboard sensors, OBS. Now, on to this, we are also adding another type of sensor, which is called as V2X, that is vehicle to everything communication, such that it will enable efficient detection of the vehicles surrounding us. Probably it is a more.
Low cost solution, because any automotive grade camera, see mobile phones, you know, we can purchase mobile phones, you know, more or less V2X communication is similar to your mobile phones, right? Basically, you need an radio communication equipment. Now, you can, you can, you can think, or else you know that.
For radars, again, then typical Automotive grade radar would be costing around 1 lakh or something. Although the costs are coming down, LIDAR would essentially cost somewhere between 3 to 4 lakhs. I'm just being very conserved and giving out these numbers. Camera also would be around roughly around the same range.
But V2X, I think it is much more cheaper. Only thing is, how well the OEMs adopt is the question mark. OK, so I've already mentioned, you know, the limitation of the onboard sensors. Now, what I intend to discuss is, OK, this is a typical scenario that you can observe.
wherein the different actors in the road are communicating with each other. They can see vehicle to vehicle communication, vehicle to pedestrian communication. Communication as in it's not like, you know, a pedestrian on road, he is talking with someone on inside the inside, you know, who is driving the car.
That's not the case, but here, you know, the information, the location of the actors are shared among themselves and also their intention. Okay. And also we can communicate with the infrastructure. Say, for example, this is what is called as, you know, the traffic signal. I'll come to this particular application where...
how actually we can improve the, you know, what you call as, how we can improve the traffic monitoring or operations, you know, using communication between vehicle to infrastructure. Okay. And also the other aspect is, so this is why, you know, we call as
vehicle to everything communication. This is V to V, vehicle to pedestrian, vehicle to interest, you know, what we call as infrastructure communication. So this entire thing is abstracted and call as X, which is vehicle to everything. Now with, so this was, you know, probably a few years back where there was no concept of the vehicle communicating with the network.
But with OEMs, you know, fast adopting, you know, for predictive maintenance application where the onboard sensors, when I say onboard sensors, you know, which is not radar, lidar, and camera, but the ones which actually monitor the health of the vehicle, the data is transmitted to the cloud. So that is where the vehicle to network will.
come into play. So this actually uses, you know, cellular communication. So we'll come to this. Now, there are a few questions in a chat box. You know, what's the difference between radar and lidar? And then, you know, what is an active sensor? More or less, both of your questions are one and the same.
In fact, this should be considered in your ADAS course. Radar, you know, it detects the objects based upon transmission of radio signals, mainly operating at 77 gigahertz. But whereas LiDAR operates on the same principle, which is time of flight sensors.
They operate using light waves. OK, so more or less earlier, radar and lidar, radar was mainly based upon FMCW frequency modulated continuous wave radars. Lidar was mainly based upon pulsed, but with advancements in technology.
Now, we have got FMCW Lidars as well. Now, the question is, what's the advantage of FMCW radars? FMCW radars can measure both distance and velocity. That's the relative distance and velocity. Now, Muthu Krishan's questions, what is the difference between active sensors and passive sensors? Say, radars and Lidars operate on transmitting the signal.
receiving the signal and then trying to extract the information. So LIDAR and radar come under that category. But cameras, they're dependent on the ambient light conditions. The sunlight, you know, illuminates the objects and that is actually captured by the camera. So camera is basically a light sensor which...
collects the light. So it's not an active sensor, it's a passive sensor. Hope this is clear to you. In fact, another category of sensors which fall to this category, that is active sensor, is ultrasonic sensors. OK, ultrasonic sensors are also active sensors. OK.
But I will, I there is no scope for me to go into the details of these two topics because they these are considered under your Hadask courses. OK, now.
So let's go into the application which is called as Platooning. Now, how many of you are aware of Platooning? Comment in the chat box.
OK, not many of you are aware of platooning. OK, before that, what actually is cruise control?
At least you know you should have heard about the cruise control.
What does cruise control?
OK, so in this case, cruise control the vehicle that is that you are driving. OK, what will happen is it will maintain a steady state or some steady speed. OK, so this we call it as ego vehicle. Maybe you can engage when this particular feature is engaged. You need not have to.
step on the accelerator pedals, the vehicle will, you know, drive automatically and the set speed may be 60 kilometres per hour or 60 or 70 kilometres per hour, whatever it is. Now, the question is, what if there is another vehicle in the same lane, which we call it as?
We're sitting weak, then what will happen to the feature cruise control feature comment in the chat box?
What will happen to this feature?
It will not notice because the cruise control feature is not equipped to notice this particular preceding vehicle. So that is when we add another prefix over here. So that is adaptive cruise control. Now the functionality of the adaptive cruise control is primarily
First, it acts as a cruise controller in absence of preceding vehicle, in presence of preceding vehicle in the same lane, that is also another condition, then it will maintain a time gap or a safe distance. It maintains a time gap or it is nothing but a safe distance, probably.
Which is called as dfdf. OK.
Now, you can refer to the ISO standards as to how adaptive cruise controller works. Okay, so this is what it is. Now, this is the case where the, you know, there are, it's a question about only two vehicles, you know, the ego vehicle and then the preceding vehicle. Now, in case of platooning, as you can see, it's nothing but a platoon of vehicles or else.
It's multiple vehicles going one behind the other. This is just like a train where there is the engine. The engine in this case is equated to our platoon leader and then behind the platoon leader are the bogies or there are multiple vehicles. Now, there might be questions as to...
how the platoon is formed. Okay, so that is one thing, how the platoon is formed, and then how we know the platoon, what will happen if one of the members in the platoon intends to exit. Okay, let's not consider that, but let's try to examine this particular application.
OK, see, with help of CV2X or you know V2X technology realised using cellular communication, there are many upcoming or there are many applications that can be realised. One is nothing but the platooning application. Now, what has what's the need for having this?
platoning. Is there any advantage in this?
Probably this can be in only in highway scenarios. Comment in the chat box. What would be the advantage of this? What would be the advantage of platformsupportwilpbits? Forget about all the other aspects.
Maintain same speed, that is fine.
See, one, it actually saves, you know, what is called as it improves the fuel economy. OK, so that is because...
only the vehicle or the platoon leader has to face the air drag or else, you know, the vehicles that are behind this, you know, actually it will improve the efficiency because of all your aerodynamics or whatever it is, okay? So this probably if there are specifically this is useful for, you know, the
Cargo vehicles.
where there are multiple vehicles which actually go one behind the other. Okay, so this is where actually it is useful. Now, this is what we call a platoon. Now, in case of platoon, there are N members that you can see here, capital N members, and the first vehicle is always the platoon leader. Now, what is the objective of this particular platoon?
The objective of the platoon is to maintain this particular set distance, which is called as D R.
desired distance. This is the gap between the two vehicles. Now, how to maintain this particular, you know, relative distance DR is what we will try to analyse in this particular case. OK, I don't think so. I have extra slides. OK, let me just try to manage here. OK.
So, this is the relative distance. OK, so this is the relative distance that has to be maintained, but actually what we want is the desired distance, which is called as D suffix D. This is called as desired.
Distance now based on this.
Now, this has to be adjusted. Now, we are indexing each and every vehicle by I. I is equal to 1 stands for platoon leader. I is equal to stands for the vehicle which is 1 vehicle behind the platoon leader. I is equal to 3 S, probably the second vehicle in this particular platoon and then so on and so forth.
Okay, now, so we will decide the controller of each of these particular platoons. So it says X double dot DS comma I. Now what actually is this? Okay, now say for example, we know that velocity is always indicated as DX divided by D.
OK, so this is velocity, and we know the acceleration is denoted as the rate of change of velocity with respect to time, so we will write this as V dot, sorry, not V dot, sorry for this, so this we can write the derivative, so this is X dot, which indicates the velocity.
Or else we can indicate the double derivative, so that is double derivative of X with respect to time, so this is nothing but X double dot, so the dot indicates you know the derivative, and so here the controller here is indicating the desired acceleration.
As you all are aware, we actually basically try to control the acceleration of the vehicle. And based on this, you know, that is why we call the pedal as an accelerator pedal. We have not, it is not called as a velocity pedal. Okay. Now in this, you can observe there are different parameters.
So, let me write this equation or the, you know, the controller equation for, say, for example, the platoon member 3, that is, I is equal to three. OK, now there are constants here which are called as A1, A2, A3, A4, A5. Let's not worry about this. They are just, you know, the weighting factors or weightage factors.
So, let's write A1, the desired distance minus the desired, or you know, the measured relative velocity. See, desired distance is what we want, and then the relative distance is what is actually measured. Now, this particular desired distance is decided by the...
Platoon leader now, who will communicate this desired distance to other members in the platoon?
comment in the chat box. We will communicate.
Yeah, product leader will communicate, but through means what? So basically, again, this desired communication or desired distance would be communicated through vehicle to vehicle communication. So that is through V to V communication. Okay, or V to X communication or whatever it is. Now, we will measure.
or each platoon member will measure the relative distance, D. Now, what sensor can be used in order to measure this particular relative distance, D? Now, even the justification that the desired distance would be communicated by platoon leaders to other members in the platoon is not, you know,
probably an 100% reason for us to adopt vehicle to vehicle communication. We'll see later where V2V communication is still important in this entire equation. So where, so how we can measure this particular DR, that is the relative distance.
Ultrasonic sensor, yes, good, but I don't think so. It does, it would work under high-speed conditions.
OK, it is basically radar. OK, radar will actually, you know, measure this particular, so still you require the onboard sensors for an application. It's not that you know you will completely ignore the sensors. OK, next we will consider the other factor that is alpha two, then X dot. So, in this case, we have considered platoon number.
X dot 3 -, X dot 3 -, 1, so that is X2. Now what is this term X3 dot minus X2 dot? So this is X3 dot and then this is nothing but X2 dot. What are these terms indicating? Comment in the chat box.
Yes, it is velocity, so X3 is the velocity of platoon member three, X2 is nothing but the velocity of platoon member two. Now, if you take the difference between these two velocity, what is...
What is it called as?
If you take the difference between these two velocities, what is this called as?
It is called as relative velocity. Perfect. It is called as relative velocity. Correct. Now, how you can measure this particular relative velocity?
How we can measure this relative velocity?
What sensor can be used? Or else, see, please note again, this is measured at the vehicle 3 or plateau #3. What it says is that, okay, at i is equal to 3, how far you know the vehicle or with respect to my speed or with respect to me, what is the speed at which?
You know the vehicle ahead of me is travelling. Again, we'll use radar in order to get this particular.
Term, so that does radar on this particular platoon member three. Now I'll ask some basic questions, say, for example.
The velocity of the platoon member 3 is 50 metres per second. The velocity of platoon member 2 is 50 metre seconds. What is their relative velocity? Do you feel you know there has to be some adjustment required at platoon member I is equal to three?
Comment in the chat box.
The relative velocity is 0, so you do not require anything to be adjusted. So, this is a very simple answer. OK, now, on the other hand, say, for example, if the relative velocity is equal to 40, or else sorry, if the velocity of H 2 is equal to 40, then what will be the relative velocity?
It is 10 or else it will be minus 10. So negative indicates here the distance is actually reducing. Okay, the distance is actually reducing. So this, you know, this relative velocity which is measured on platoon member 3 is obtained with respect to, so with respect
To the platoon member ahead is obtained using radar. OK, next is we will again calculate, or else in the next term that is X.3 minus X.1. Now, what actually is this?
Okay.
It's the same. Here, this term here is velocity of the platoon member 3, and then x.1 is nothing but the velocity of the platoon leader. So we are trying to calculate the relative velocity between the platoon member 3 and then the platoon leader.
So can we calculate? So try to visualise this. I'll let me use a different colour.
So can we calculate using radar onboard sensor? Comment in the chat box. Can we calculate the relative velocity between the onboard sensor? It's not possible because the visibility of the radar is blocked by the vehicle ahead of it. So what is that sensor that
We can use for communicating the actual speech, so in this case, X3.
The velocity of the ego vehicle, and then the velocity of the platoon leader, not the ego vehicle, sorry, the platoon member. So, what sensors will provide, you know, what you call as the speed values? Say, for example, here the velocity of the platoon member three would be given by wheel speed sensor, because if you know...
the revolutions of the wheel, you'll be knowing what is the circumference and then you can calculate the vehicle speed. Okay, so this is the vehicle speed, so which is required. So you can see the number of sensors involved here. Then again, X1, this would be communicated via V2V communication, that is.
The sensors or the wheel speed sensors on the platoon leader will measure, and then finally it will communicate to the platoon leader, or sorry, the platoon member I is equal to three, or in general to all the members.
Okay, and then the next term is here, we have alpha 4. So this is X double dot I2, that is 3 minus 1, that is 2. So what is X double dot equal to? Any comments in the chat box? Okay, Dhanush, I'll repeat this once again.
See how, say for example, at a particular on a vehicle, how do you know its actual speed?
Somling, just give me a minute. I'll actually have already said why platooning is required. I'll explain this once again. See, X3 indicates it's a velocity. Velocity is obtained, X3 dot, it is obtained by using the wheel speed sensors. Okay, now this has to be subtracted or in reference with, you know, what we call as
the platoon leader's velocity. Now, how do you get, you know, the platoon leader's velocity? See, as it is, you know, your vehicle is I is equal to three. You would be knowing your own velocity, but how you would be obtaining, you know, the velocity of the platoon leader? That is through V2V communication.
Does it make sense, DINESH?
Okay, now I have written another term where alpha 4 and then X2 double dot, which is corresponding to the acceleration of the just vehicle ahead of us. Okay, now how we can calculate the acceleration of the vehicle just ahead of us? Any comments in the chat box?
Can are there any sensors which directly measures the acceleration value? Always direct measurements are good.
Yes, will radar directly measure the acceleration?
Now, accelerometer would see accelerometer, yes, it gives, but you know it would be that is one thing. OK, yes, that is a good thing. So, there would be accelerometers on platoon member two, and that acceleration value will be transmitted by a vehicle-to-vehicle communication. OK, see, please note here that.
See, you can use, how many of you have heard of IMU sensors?
How many of you have heard of IMU sensor? See, always please note that we want direct values, direct measured values. Of course, you can using velocity, you know, you can actually calculate what is the acceleration. Now, what is an IMU sensor? See, IMU sensor, it is made-up of three.
One is accelerometer, then the other one is gyroscope. OK, it can also have what is called as magnetometer. OK, now in a vehicle there are we associate 3 axis.
OK, one is X axis, Y axis, and then Z axis. Probably X can indicate the direction of movement, Y can indicate the lateral, and Z is of course up and down for, unless your vehicle is flying or similar to a drone, no point in having the Z axis. OK.
So, we can measure acceleration along x-axis, y-axis, and then z-axis. So, there are three different accelerations. Through using this, we can also provide what is the acceleration of the vehicle, or else we can also give what is the angles, theta X, theta Y, and theta Z. So, this is given by the...
Gyroscope MXMY magnetic fields are given by magnetometers. OK, so if the IMU sensors are just measuring 6 readings corresponding to acceleration and angle, so this is called as six degrees of freedom if it is measuring.
Magnetometers as well, so this is called as 9 degrees of freedom. Now, do you think of so what we can do is the vehicle 2 will measure the acceleration, and then finally that will be communicated to the vehicle 3, which requires as part of it a controller. Of course, it can use the radar measurements at.
Platoon Member 3, where you subtract, you know, the present velocity with the previous velocity divided by time, and then calculate the acceleration, but that will be a very noisy estimate. OK, so this is one thing, and also, likewise, the last term here, it indicates the acceleration from the platoon.
leader. So, you can see that these two terms, the accelerations, can be communicated by V2V communication or vehicle to vehicle communication to this particular platoon member 3. Now, before I visit this equation once again, that is the controller equation, can anyone
Think about any other application where these IMUs are useful.
Inertial measurements units.
And applications, of course, airbag, I'm not sure. Drones, okay. See, I'm, let's not go into drones and all. Let's not fly from this particular class. Yes, electronic stability programme. Yes, that is correct. Recently, there is one more important features that are.
You know, that has marketed by many vehicles, which is called as Hill Assist Systems.
In order to find out whether a vehicle is climbing uphill or else it is going downhill, so we require, or else you know it is used by this particular hill assist system also. OK, now coming back to this platooning application, so in case of platooning application, we see...
We are trying to control the acceleration.
of the platoon member 3. Finally, this acceleration. This is what I said. We are perceiving, perceiving in the sense we are measuring the distance. We are getting to know about the velocity and the acceleration. After perceiving, we'll get to know what should be the desired acceleration that is
planning and based on this we will control the actuations. That is, we can actuate either throttle control or else it is nothing but brake by wire. Now, under what circumstances this particular desired acceleration will be non-zero. If the desired distance is not matching with.
the desired or the measured relative velocity, then accordingly the acceleration will be activated. Now again, if the relative velocity is not zero, say for example, if platoon member 2 may be accelerating or the platoon member 3 is getting decelerated,
Under those conditions, also, we will have some desired acceleration activated, and also, if the platoon member is not following, or else it is not having the same velocity as that of the velocity of the platoon leader, then also there will be some acceleration. OK, so these are all the impact.
which we are treating it independently. Similarly, if the vehicle ahead of us is also accelerating, then also it has an impact on the desired acceleration. So is with the platoon leader. So in this way, we'll ensure that all the vehicles, just like a convoy of vehicles,
of minister vehicles where all the members of the platoon will just follow with the specified distance. Is this clear?
So this is one of the latest applications that does platooning. In fact, if you search in YouTube, you can see plenty of videos that demonstrate this particular technology. If you have any doubts, please post it on the chat box.
Sending it out.
All good? See, don't hesitate if you want anything to be explained. What is alpha? These are all constants. We are not getting into the details of as to how we get these particular scaling values. Okay, that is out of scope of this particular syllabus. That has more to do with the control system design.
Okay, so any other doubts?
Is this new concept or this concept has been around a while for multiple, probably around for some time, but it's not, it's only thing is it does not gain some importance in our country. Okay. If DR constant remains for all day, yes, DR is constant. It has to be the same for all the vehicles, inter-gap. Okay, it has to be the same.
Okay.
So any other queries?
all this feature are in traffic. See, this is not how this is would be useful. As I said, you know, if you have a platoon of vehicles, it will actually drastically improve the fuel economy. And also in it will be more of a disciplined traffic.
It's not autopilot. It also improves or else provides comfort for the vehicles. You know, the drivers, you know, they will also feel comfort while driving. And also main thing is, you know, the improvement in the fuel economy.
Uh, then, Nanda Kumar, I didn't understand your question. It will be always a negative for relative velocity. Uh, no, I didn't get your question. Probably you can unmute and ask your question.
Others, you can take a break. I'll resume the class from 4.45. That is the actual technical content. In the meantime, you can raise your queries and get it clarified.
Will you please explain it? See, dd is a desired distance. Say, for example, I'll say that, okay, I'll set the desired distance to maybe 1 metres or two metres or whatever it is. Now, how fast, you know, the vehicles can communicate, that will decide how short, you know, the desired distance can be.
If the communication latency, or if the packets, or if the information that is shared by the platoon leader, if it is reaching immediately, then even I can make this particular desired distance to point metres, also.5 metres.
Okay, in fact, this is a very good question. How can we find the desired distance? See, this is 1 subject where, you know, the communication and then the vehicle kinematics has some role to play here. Okay, say for example, in case of communication, there are a couple of quality of service.
which we call it as quality of service parameters. Now, what actually is this quality of service? If you visit a hotel, how do you actually visit a hotel? Is it only based upon how taste, what is the taste of the food? No, we consider different parameters. What is the taste of the food? Yes, that is the primary factor.
then how far the hotel is from your house, then what is the parking facility, how clean the hotel is, and then how courtiers are the staff, so and how quantity of course, you know, I, you know, it's a and how we know value for money, everything. Okay, so these are all the parameters in the same way.
When it comes to communication, there are multiple parameters. One, next, what is called as the range. What is the range of communication? And then what is the data rate? Okay, how fast you know the data can be transmitted. And of course, you know, the latency also. That is, when I, that is the time elapsed between the transmission of reception of messages.
If I transmit the, just like saying, you know, speed post, if you send the post today, how many days it will take to reach the other destination? That is called as latency. In the same way, if the latency is very less, we can have the desired distance very small.
Okay, so this is one thing. So are there any other parameters?
So, sorry, are there any other queries?

JAGADESHAN K .   1:01:10
So, in the formula, you have explained that if you see after this alpha three, so it will be between alpha three and alpha four, it will be plus only, right? The symbol will be.

Shree Prasad M .   1:01:16
Mhm.
It will be pleasant. It depends whether the velocity is in. See if the distance, if the relative distance is increasing, then the relative velocity is positive. If the distance is decreasing, then the relative velocity is negative. Simple.

JAGADESHAN K .   1:01:23
The alpha.
So this A4 multiplied by X2 X double dot 2, so that will be plus or minus will be depending upon the velocity only.

Shree Prasad M .   1:01:52
Exactly, see, say for a yes, see, of course, you know, see, again, the point here is X3 may be, or else you are accelerating, and then, but the member ahead of you may not be accelerating, so it depends, it depends upon multiple parameters.

JAGADESHAN K .   1:01:52
Right.
number is not accelerating.
Yeah.

Shree Prasad M .   1:02:13
So any other queries or questions?

MOHAMED SHAFWAN S .   1:02:18
This is.
Yeah, I love you.

Shree Prasad M .   1:02:20
Please, yes.
The front or back side of the vehicle, not in that situation. See, that's what everyone has to be in agreement. Once when, you know, once when you engage this particular feature, it does completely, you know, the controller has to take the responsibility of maintaining this particular desired distance.
Okay, I'll try to get that video and then show it to you.
Muthukrishan, yes, it is just like a sequential, you know, it's a convoy of vehicles, just going one behind the other, that's it.
See, this is just like an adaptive cruise controller. Just when the platoon is formed, everyone will come into an agreement and will engage this particular feature. It's not like I'll join the platoon and then I'll say that I'm disobeying the rules. No, that definitely that member has to be kicked out of this platoon. Otherwise, it will lead to a chain of accidents.
Yeah, group writing is always helpful. I see, and also it also has to do with, you know, the reducing the accidents. OK, so just a second.
It also has to do with, you know, reducing the number of actions, just like, you know, specifically among the truck drivers. I think that was a key focus even this time in one of the conferences. I wouldn't say conferences, but rather an industrial meet arranged by CCMBAINAI.
Confederation of Indian Industries. In trucks, you know, the accident is mainly contributed due to driver fatigue. So if you have a group of vehicles, you know, trucks, you know, who are going one behind the other, then it actually it creates more, you know, disciplined driving and also it reduces the...
A stress on the driver, thereby reducing the accidents, and then also bringing up the fuel economy, OK?
Any other queries? All good? Okay, just one small query. Is the pace of teaching fine? Should I increase the pace or should I decrease? Please let me know in the chat box. Okay. And also, there is nothing as such called a silly doubt. You know, it is your interest. You can ask any doubts.
And then it is in your interest, you should get it clarified. Okay. And for those of you are facing, you know, difficulty in terms of understanding any of the concepts, I am not sure, you know, what is your schedule after 540 onwards. I can, I am ready to extend for 10 minutes and then explain.
the concepts as well. I can go over the concepts again and again until you understand after the class hours also. And I'll be giving my e-mail ID and phone number as well. You can reach out to me for any official e-mail communication in terms of any problems that you face, of course, which is in my hand.
Otherwise, in case if you want any of the doubts to be clarified, you can just call and get it clarified. My responses for WhatsApp messages will be very slow or it is very rare because you know usually I don't have patience for typing and all. Okay, so just a minute, I'll be on mute for one minute and then I'll resume the class.
Okay, well, resume the class. So this is one of the applications which I usually give this as an example, because it actually motivates the need for VIVEK communication, apart from a few applications that I'll be discussing now. And also I have shared one more link on the...
chat which is a FIRA consortium which is called as find ranging. This is actually specifically useful for a car connectivity consortium or something. Okay, Sachin, I'll definitely I'll discuss these. I'll share these videos and.
I, you know, I'll come to this particular topic in a little while later, OK?
Now, let me explain what are all the other different applications which can be realised using this particular V-to-V communication or V-to-X communication. Just a brief of these examples. OK, let me just OK, so this is one of the.
A couple of applications which have.
Taken right from, you know, what you call as a.
Home.
The DSRC document. OK, don't worry what actually is DSRC. DSRC stands for.
Dedicated.
Short-range Communication.
This is the full form of DSRC. And this is one of the technologies which realise V2V communication. And this is based upon Wi-Fi.
This is based upon Wi-Fi. OK, now in this I have explained, you know, different scenarios which are realised using NDSRC Communication One, which is based upon forward collision warning one. Another application is blind spot warning, then the other one.
The accident which was seen similar to in Yamuna Expressway, so that is electronic emergency brake light. OK, now what I think you know here, the direction of the vehicles are shown as per the driving of in the foreign countries, so you can observe. So, this is the approaching car that is.
the vehicle that you are driving and probably this is another vehicle which we call as the stopped car or the car which is having some problem and then it is suddenly stopped in between on the road. We have seen such kind of accidents specifically in India where the truck would is...
parked and then one of the vehicles specifically during night driving, one of the cars will actually go and just slam into the or rear into the parked vehicles. Okay, so in those cases, you know, this particular vehicle, these circles indicates here, you know, the transmission. So these two vehicles are communicating and then this can actually indicate.
You know that there is a vehicle, you know, there is a vehicle.
which is having some issue and it is parked right in the middle of the road. So in that way it can avoid accidents. So this is called as forward collision warning. And then next is, you know, the blind spot warning. Here you can observe that specifically the exclamatory mark indicates the vehicle is.
driven here. The vehicle is being driven and it wants to overtake by changing the lane. So I'm just showing the directions in terms of the arrows. But with V2V communication, we can get to know that there is a vehicle in the blind spot. So this is just like these days you can see the smart mirrors.
on the vehicles or in many high-end cars. So if you just go near the blind spot of the vehicle when you're driving a two-wheeler, you can just see an indication on the, or a small light will glow or it will start glowing on the mirrors, which indicates to the driver that there is an pedestrian, sorry, there is a vehicle in the blind spot.
So same way you can get these warnings so we can see where there is actually no warning or else it is safe to overtake. Okay. Now another interesting application here is electronic emergency brake light.
In this particular case, we can observe that.
This vehicle here has applied a brake, has applied a brake, and because of which the truck also will apply a brake, and then if you are not or if the vehicle is not maintaining sufficient gap or sufficient distance, chances of rear ending or you know colliding with the vehicle is very high with the truck. You would have seen these particular cases, right?
Okay, in those particular cases, or in this case, what will happen is this particular vehicle which has applied the brake, that is its intention, that is, the braking is also actually transmitted to this truck. Okay, and also it is transmitted to this vehicle. So in this way, with this warning,
They can take necessary action, so if there is a power failure in the stop car, if we do is possible through any, see, probably the all these, you know, features will work with, you know, what you call as a, you know, on battery. Of course, it all depends upon the cell level, say, for example.
If there is a broken vehicle or else you know which has met with an accident and probably will have to ensure that you know probably some sort of redundancy where you know still the V2X sort through there will be transmission of messages with some secondary source of power. Always there will be some room for failure.
But how much less we can reduce is something which will have to be decided. OK, does it make sense?
Okay, now next is V2I application. So far what we have considered is only V2V applications, that is vehicle to vehicle. Now we'll consider vehicle to infrastructure. Now don't confuse vehicle to infrastructure like vehicle to tower communication.
OK, so this is vehicle to infra.
structure communication, which is nothing but traffic light. I'll call this as TL. Now, in traffic lights, the traffic lights exchange information which is slightly different, which is called as SPAT. What is this SPAT? So this stands for...
Signal.
Space.
And timing, can anyone?
Just provide meaning of signal phase and timing.
What a signal phase and timing?
What does signal phase and what does timing? So if you are just waiting at the traffic junction, how we can relate to the signal phase?
T.
Traffic signal. What are the different colours in the traffic signal? So that is red, amber or yellow.
And that is green, that is colour. OK, signal phase indicates the colour, then timing indicates how much time.
you know, that signal will be in. Say for example, red will be for 50 seconds or green will be in for 60 seconds or whatever it is. So the phase here indicates the colour. So this is called as signal phase and timing information. Often we have faced this particular scenario.
As soon as we enter a traffic junction, or when we are approaching the traffic signal, we see it as green. We'll just hit our full throttle and then we'll try to go past, you know, the traffic signal, avoiding to, you know, just to avoid, you know, you know, waiting in the signal one more time.
Just when you are about to traffic, you know, cross that particular traffic junction, it will change from green to red. Only if you jump the signal, then you'll be issued a traffic challenge, because at least in Bangalore they have this smart cameras, you'll get in a traffic challenge, or else you know, in case you know if you stop the vehicle.
Then the vehicles behind you will apply sudden brakes or is there they can someone can you know collide your vehicle as well. So now what we can do is.
And also at times what happens is the traffic signals would be blocked by some trees or tree branches or something. So it is very difficult to know what is the state of the signal. And also if you are behind a bus or a truck, you can never make out what is the state of the traffic signal. Of course, you can go behind the bus only to be
Photographed and then issued in traffic challenge. OK, so this kind of information can be exchanged by the traffic light with the vehicles, and because of which it is easier to actually either you can decide to accelerate or you can decide to decelerate, which actually saves your fuel economy, you know.
Reduces your stress, and then everything. OK, and also it can it can also indicate whether there is a.
You know, some work going on, and also in a car speed warning. OK, I think these days you can see in Google, right? Specifically, if you select a route from source to the destination, it will show the route in different colours. If it is completely blue, then there is no traffic. If it is slightly yellow, slight traffic. If it is red, then it means to say that it is.
Traffic jam, and also it will indicate a man symbol, you know, holding a plough or a person symbol holding a plough, which means you say that there is some work going on over there. OK, so these are a couple of information that can be exchanged by way to way communication, that is, vehicle to infrastructure communication.
No, what actually, well, the messages that are exchanged, what that message is called as, okay? So let me just go there and then probably, okay. No, I don't think so. This is.
Now what is that message called as? Okay, that message is called as BSM, okay, which is called as basic safety message. So when we are using DSRC technology, so this is called as basic safety message.
This is called as basic safety message in case of DSRC standard, which is an United States standard, or else it is also called as cooperative awareness message, which is in case of Europe, which is cooperative intelligent transportation system.
Okay, probably I will appreciate, you know, for everyone to search for these particular terms. Okay, now what actually is enclosed in this particular message? That is BSM or probably it is in this particular KAMAT message. What does it contain? It contains the location.
That is, what does the latitude and the longitude of the vehicle? What does the speed and acceleration of the vehicle acceleration? And what is the heading that is the direction in which the vehicle is moving? What is the turn signal status, whether the vehicle is moving towards right or left? And then what is the?
height and width of this particular vehicle. So this is what this is the information that actually some information which is actually shared. Okay, now based on this, you know, you can build or else the other one can build a trajectory or it can just predict, you know, what would be the direction of your where actually your.
your vehicle is going at. Okay. There are a few other, we'll get into this BSM and then KAMAT later, how the messages is constructed and all. Now, before this, I want to discuss, I want to show one slide where which nobody will make any sense out of it. You know, when I'll start, you know, coining few terms, you know.

JAYAPRATHAP U .   1:21:33
1.

Shree Prasad M .   1:21:35
which is bandwidth or centre frequency, so which may be a little dangerous for class as well. But of course, you know, I'll have to start with this particular slide. Now, if you see this particular slide here, it says DSRC channels in gigahertz. Now, don't worry, although I might scare you in this particular slide, but
I'll be giving all the necessary prerequisites as promised. So what do you mean by channels? Any comments in the chat box?
Yes, Songling, any queries?
Have raised your hand, any queries?
Okay, fine. No response. So what do you mean by channels? Any comments in the chat box?
Okay, channel frequency, fine. Have you heard of any other technology where it uses channels also?
Or it uses the concept of channels.
Frequency band under FM radios, perfect. So FM radios, we say that.
Okay, say for example, you say that channel 98.3 megahertz. I'll be using this technology in order to explain or else probably we'll use 91.1 megahertz, so on and so forth. Okay, now what are these particular frequencies?
Comment in the chat box: this is just like some something similar to your normal frequency, also like 50 Hertz or whatever it is. OK, now what actually is this frequency?
What actually are these? How do you call this ab?
What actually constitutes a channel? Is it like, you know, although we address as 98.3, 91.1 megahertz, although I don't have any inclination or any bias for any of these channels. So these are what are called as centre frequencies. That is, this is the frequencies on which the information is transmitted.
So does channel only are they characterised by the
You know, a centre frequency, or else there is something more to it.
Is there something more to it? Comment in the chat box.
Quality of sound, yes. There is something which is also called as bandwidth.
You'll be hearing these terms quite often, even in Radars also, because your FMCW radars, the, you know, the resolution depends greatly on the bandwidth, larger the bandwidth, it can distinguish between the two fine or two closely spaced objects, so that is bandwidth.
For those of you who are wondering what actually is this particular bandwidth, we will consider later. You need not have to worry. OK, so here we can see that.
Of course, all these frequencies are given in terms of gigahertz, giga stands for 10 to the power 9, mega stands for megahertz stands for 10 to the power 6. So, what it says here is that channel number 172. Now, don't ask or else probably don't be wondering why here all of a sudden.
channel number 172, 174, it is just the name, that's it. Say for example, all the radio stations, rather than calling it as 91.1, they call it as Radio City or Radio Mercy. So it's more easier to remember a name rather than remembering the actual, you know, the frequency value.
Okay, same way here also. When I say channel number 172, it is 172. We are not bothered about the frequency band or frequency. So it says that so this is from 5.855 gigahertz still 5.865 gigahertz. Likewise, you have the start and then end of these particular frequencies. So when I consider...
Channel number 172, the start frequency is 5.855 gigahertz, the end frequency is 5.865. OK, now what does the of course you know exactly the centre value here? OK, would be the centre frequency. We will come to these concepts later, but I just want to highlight.
The motive for discussing bandwidth and then centre frequency. OK, now in this particular case, we can observe that the vehicle to vehicle message in the form of BSM, that is basic safety, and next the SPAT, or you know, which is also called a signal phase and timing information, it is transmitted on.
Channel number 172, that is this channel which you observe here. OK, this is reserved for transmitting this information, and remember the same channel is used by multiple vehicles in the vicinity.
is used by, I'll be considering this particular topic later, that is different multiple access. So it is used by multiple vehicles, single channel. It is just like saying you have a single channel, but it is used by multiple FM radio stations. That is, it can be FM radio city.
It can be, you know, what you call as a radio, or else probably I think there are hundreds of probably there are multiple stations. OK, does it make sense? So here, when we say a channel, so this will have to be clear. When we say a channel, 2 aspects are important. One is nothing but the centre frequency.
and next is nothing but the bandwidth. So these two aspects are really important. Okay, so these two aspects are important. We'll come back to the other channels where it is used and so on and so forth. Okay, now this is how what actually is DSRC channels. Now.
Question here now.
Are these licenced bands or are these unlicensed bands? Comments in the chat box.
Are these licenced bands or these are these are unlicensed bands? When I say unlicensed bands, say for example, Airtel, before it deploys a network, it has to purchase the spectrum from the government and then it has to use.
I won't need that, so...
Do you think if you make these frequency bands licenced, will there be widespread adoption of this particular technology?
I just like saying.
If you want to use Bluetooth, you just pay for the hardware. You are not paying for the spectrum.
So do you think it makes sense to make these bands licenced? That is, not only that you are purchasing the Bluetooth device, but also you are purchasing the, you know, what you call as the frequency band also. Okay. In fact, they are all in what is called as unlicensed frequency band. So you operate or you pay only for the hardware.
Not for the channel. OK, so this is unlicensed. Now, there is a special name given to this unlicensed frequency band. What is that?
There is a special name given to this unlicensed frequency band. What is it?
It is called as ISM frequency band. It is called as industrial, scientific and medical.
And these are the frequency bands in which the Bluetooth, Wi-Fi operate because.
The Bluetooth or even when you purchase Wi-Fi modem for your device to communicate your mobile phone to the Wi-Fi, it communicates via two frequency bands. Okay, one frequency band is 5.2.4 gigahertz.
The other band is around 5.9 gigahertz. You would have seen the IDs of the Wi-Fi. It will be clear to you. One that will say, or else probably, you know, the Wi-Fi, or the person who instals the technician Wi-Fi, he will say that, sir, your Wi-Fi has got 5G. We'll be confused that he is saying about, you know, 5th generation of mobile communication. No.
It does.
It has got two frequency bands of operation. One is 2.4 gigahertz, the other one is 5.9 gigahertz. I'll ask additional question later, but these are what are called as ISM frequency bands. So in order to install your own Wi-Fi network in your home, you need not have to purchase these frequency bands. Now one more question, which will be very much relevant in the later part of the course.
In which of these frequency band you get a wide frequency range or a large, sorry, a large range of communication or large communication range?
2.4 or 5.9 gigahertz.
It is 2.4 gigahertz in which we would be getting a large communication range.
But in which frequency band will be getting very good data rate?
in 5.9 gigahertz. So communication range, lower frequencies, data rate, higher frequencies. That's it. Okay. Now, so this was a brief intro to what you call as V2V communication or V2X. Dhanush, this is the main aim of this course where we'll be seeing why.
High bandwidth, sorry, high frequencies provide faster communications and why low frequencies will have long range communication. We will be considering this. There are two technologies in order to realise V to V communication. I will come to this pixel later because I don't want to scare with this, but I just want to highlight.
One important slide over here.
Okay, so this is one of these slides which is taken from Qualcomm, where it shows, you know, the see how we can actually realise a VIVEK communication. Now, if you all remember, not now, probably I think it's around when mobile phones started gaining popularity. I think it was in around probably...
around 2002 or 2006. Back then, there were phones which were called as GSM. Was there any other competing technologies with GSM? Can anyone probably remember the competing technology for GSM CDMX?
Perfect. So it was CDMA phones. So GSM was given by other non-Reliance companies and CDMA was by Reliance companies. See both.
had the same intention. Okay, both had the same intention, that is providing mobile communication, but the means of it was different. See, in order to go from place A to place B, it does, you have to travel, but the means that you select to travel is can be, can where you can either go by metro or else either you can go by your own vehicle, it doesn't matter, but.
The objective is going from point A to point B. So same way here also. The objective for mobile communication, the objective is mobile communication. Either it is GSM or CDMA, it doesn't matter. Same way for V2X communication, either you can go for Wi-Fi based. Please note Wi-Fi based.
DSRC or cellular based also, it doesn't matter. OK, now in this case we have shown here, or else you know this, I'll be explaining this slide later, but I have to.
motivate this topic as to why we have to consider all these parameters. We can see here that the communication range will actually decide the speed at which the vehicle travels and also the stopping distance. You can see here.
This is the graph. I'll be revisiting this particular graph later. Okay, so this is a really important one.
that two under specifically under IC road conditions and then normal road conditions. Okay, we'll consider this concept later because again here we can see there are two parameters here. One which says, you know, line of sight communication, LOS, other one says non-line of sight communication.
You can see if there is direct visibility with two communicating entities, then approximately V2X or V2X based on cellular technology can communicate almost for two vehicles which are separated by half a kilometre and for DSRC is much less, it is 260 metres. Likewise under non-line of site communication, just like
the way you are at house and then you cannot directly see the cell phone tower. See if you are not seeing the cell phone tower, then it is non-line of sight communication. If you can see your cell phone tower, it is line of sight communication. So that is how it is and then you can see the communication ranges and then the impact that it has on, you know, the velocity of the vehicle and then the stopping distance that we will consider.
later. Now, what I'm trying to motivate here is that whatever topics that we discuss, say for example, now I've thrown a lot of communication terms over here. One, what is the centre frequency? Next, what is the bandwidth? Now, the bandwidth that we select, is it sufficient to transmit the messages? Because
The bandwidth will decide the latency, and latency will actually in turn decide what distance that we can set in platoning. So these are all interrelated. It's not like you can say that I'll be a good ad as engineer without knowing, you know, the basics of communication. It is not possible.
Again, you cannot say that, and I'll be a good data engineer without knowing the basics of mechanics; that is also not possible. There has to be a some fine balance there, so this graph we will visit later. OK, now the other technologies which I intend to cover here is...
One is, because we are almost approaching the end of today's class, one what is called as intravehicular Communication.
I know I'll be flooded with answers for this question. What are the different intra-vehicular communication protocols?
Okay, different intra-vehicular communication protocols. I think, you know, one standard one is CHANDAN, controller area network, then local internet network, local interconnect network. Okay, then any other technologies that you can think of. The other one is FlexRay.
I'm not sure how often or how popular it is flex-ray these days, and then some might be someone has, yes, it is, some might be it does automotive Ethernet. OK, it does automotive Ethernet. Again, in case of automotive Ethernet, there are different.
technologies, one which we call as T1S architecture. OK, and then followed by some IP as well. This is one thing. OK, so primarily these are again in CHANDAN is also getting a world. J1939, this is for commercial vehicles. For commercial vehicles, we have J1939.
And we have a CHANDAN open.
And we have what is also called as ISO bus as well. So this is for industrial and agricultural and all sorts of vehicles. Okay. But basically they operate mainly on they use CHANDAN as an underlying hardware or underlying protocol. Okay. Now, what is that we can expect for intra-vehicular communication?
In this particular course, in case of CHANDAN, we will be motivating why ECUs have to be networked. We will create a CHANDAN network that is, we will understand the dcb file.
CAN dcb file, and we will also study about the different frame types in CAN, and also we will study about, you know, what you call as, you know, there are, you know, specifically in case of CAN, an important concept is what is called as arbitration and message filtering.
So, this is what you can expect for the intravehicular communication. In case of Lynn, what I would be discussing is, in case of Lynn again, how to create Lynn descriptor file that is creating a blue map for the network, what are the issues participating issues in the network, and then how they...
what messages are they communicating and then how signals are, how many signals are packed in form a frame and then communicated and then different types of frames in LIN also. So this is also we will be considering. Okay. Next, in case of Automotive Ethernet, again, it all depends upon how time permits.
My intent here is to cover T1S protocol as well. OK. And then actually some basics of some IP also. And showing demo using Raspberry Pi boards or whatever it is. OK, so this will be done. So this is conditions applying.
But these two are must, but in any case, the primary objective here is to show the difference between, you know, what you call as signal-oriented architecture.
and service-oriented architecture.
So how many of you have heard of these terms, signal oriented architecture and service oriented architecture, and how you can map with the different communication protocols? Please comment in the chat box.
Okay, good.
I'm not aware of all the above. That is also good. You be slightly aware after completing this particular course.
So can anyone comment to which protocols we can map signal-oriented architecture and to which protocols we can map the service-oriented architecture?
OK, I'll ask you one question: in case of CHANDAN.
Say, for example, can how you know we know that it will, you know, we have a network of issues, OK?
We have a network office. So if you want to add or delete, you know, one ECU, how easy it is on a CHANDAN network.
How easy it is to add or delete a node in case of CHANDAN.
Just say easy or difficult, that's it. It is actually rather difficult, you know, adding an ECU, it's not so easy in case of CHANDAN network. I don't know, I think I'm getting a contrary answer. It is actually rather difficult, okay, because you'll have to change the entire dcb file and the communication matrix, everything.
Okay, because it's a fixed one. You know, all the nodes will know who is communicating amongst themselves and then, you know, it becomes really difficult. Okay, but in case of service-oriented architecture, which is with the some IP, there is no such tension. Okay, it's all service.
No, it's just like you open the hotel and then you'll say, I'll offer these services. You know, one who is interested, he'll come and, you know, say that, you know, I just want so-and-so dish and then he'll purchase and that's it. You know, this is called a service-oriented architecture. Now, the signal-oriented architecture is mainly based upon CHANDAN blend.
And service oriented architecture, mainly it is by the Ethernet. OK, so this is what you know is about, you know, the signal oriented architectures and then service oriented architecture. Now, what has motivated to consider service oriented architecture? One primary motive.
One primary motive, any comments in the chat box?
Yes, it is flexible. Yes, that is true. But we'll have to be more specific when we say flexible.
Now say for example, how easy it is to upgrade a car which is based upon CHANDAN network.
How easy it is to upgrade a vehicle which is based upon the car, which is based upon CHANDAN network.
It is actually very difficult. You know, what is the reason for that?
What is the reason for, you know, very difficult in terms of upgrading the software for a vehicle which is based upon KAMAT work?
It is because of the ECUs are made-up of microcontrollers. Updating microcontroller, I'm not saying it is not possible, but rather it is difficult. But with some IP, we are going for a hybrid architecture. What is that hybrid architecture? Where it is having both.
the microprocessor and then microcontrollers. This is because of the need for the Adash functionalities. I don't know again how many of you have heard of. I'll be discussing all these concepts when I start this particular topic. We have what is called as the domain.
Architecture.
and what is called as zonal architecture.
When I say architecture, it's all about, you know, how the issues are networked. Priorly, you know, we were saying, or prior, we were saying, you know, issues are mainly made-up of microcontrollers, but we have these days, you know, issues which are also made-up of microprocessors and then microcontrollers now.
What is the current trend in the industry? Is it domain architecture or is it zonal architecture?
It is zonal. It is actually zonal architecture. Reason is that specifically because of, you know, the need for, you know, what you call as networking computationally intensive applications. Now, what are those computationally intensive applications which was not there in the traditional setup?
They are, you know, the ADAS algorithms, like you have to run Yolo algorithm or probably you'll have to process a lot of LIDAR data or probably radar data, camera data, where the bandwidth runs in terms of gigabits per second, which may not be supported by your traditional CHANDAN or LIN also. Okay, so I'll go through what actually is this domain architecture and then zonal architecture also.
And also, I'll try to see, explain, you know, what are the different kinds of HPCs, high performance computers. Say, for example, HPCs, you know, say you have the HPCs can be there for different purposes. It can be there for ADAS, it can be there for infotainment. So which company has HPCs for a tailored for which?
for what application. So these all will be, I'll be just trying to give an overview of all these aspects in detail. Okay. And finally, there is another technology which is silently making sound in the Automotive segment. Can anyone comment in the chat box? I've already shared that link in the chat box.
Chat box.
Please let me know.
Yes, software-defined vehicles. Basically, this is what the topic is, the software-defined vehicles.
How many of us get annoyed specifically during summers? The vehicle would be parked where there is no shade for the vehicle. And as soon as you enter or else get into the vehicle and switch on the AC, even though it feels like as though we are in pressure cooker. Okay, so what if there is a technology where
Even before getting to the vehicle, probably you know it just switches on the AC and then keeps the temperature optimal. Or else as soon as you approach the car, the vehicle will actually unlock itself. Okay, yes, ready to learn. Can anyone comment in the chat box, you know, what is that technology which is...
silently, you know, not many are noticing, but you know, it is also gaining equal popularity. Any comments in the chat box?
So it is called as one of the consortiums which are focusing on is what is called as car connectivity.
I'm just sleeping.
Car connectivity.
Consortium. OK, so it is also abbreviated as Triple C, and this is actually, you know, main the major motive here is, you know, say, for example, to use phone as a key. OK, phone as a key, and probably there are different technologies that are used here.
one which is UWB, ultra wide band radios. Then again, probably it will also use, you know, what is called as Bluetooth. Okay, yes, NFC card as well, each having its own advantages and disadvantages. Okay, of course, UWB radars, you also know that it is also used for
The, you know, child monitoring in most of the European vehicles, but this is more or less, you know, the topics. Now, how do we, how do I cover this particular topic is quite challenging. That's what there has to be a first from your side as well. A couple of things we can take it offline as well, or else probably I can extend the class.
10 minutes from next week onwards. So again, in this case, I want to cover a couple of topics in UWB also, so that, you know, when we say automate, see, no longer we cannot restrict ourselves to one particular technology saying that, okay, this works actually. So we'll have to, you know.
focus on, you know, we'll have to evolve as the trend is also evolving. Okay, so focus would be to just give, so what we can expect in this particular course, at least in the V2V plan. Okay, so in this particular course, first what we would be doing is specifically in the aspect of V2X.
In the next class, I'll be discussing about, you know, the channel or, you know, the what makes a channel, OK, and also how we actually distribute the channel, and then a couple of, you know, when we say, you know, one, there was one question in chat box, like why if we increase the frequency, the distance reduces, we'll try to understand.
couple of concepts such as you know what are called as channels and then its channel conditions. We are not getting into the details of these concepts because our primary aim is not to get technically or analytically understand this particular subject.
but it is just sort of a general awareness and then to apply the concepts what you have learnt in the examination.
Of course, there will be some math involved in solving and all, but it's not like, you know, it's totally, you know, it's not like you will not be in a position to solve anything or not. So again, we'll be considering modulation scheme and then everything. And also this is a couple, and also, you know, a couple of terms which will help you also in understanding.
other subjects as well, like based on EMI, EMC or whatever it is. Okay, so this is what I intend to cover in V2V communication also. So more or less, you know, significant attention would be given to V2V, both in file layer and then Mac layer. And then again, we'll consider the intra-vehicular communication also.
And also we will consider what is called as the UWB technology as well, because it's getting, you know, more popularity and then you have, in fact, I'll be showing an experimental setup also if it is ready by that time. And we'll try to demonstrate some of the V2V applications also. Okay. Now, how many evaluation components are there for this particular course? I'll be sharing anyways the slides.
As you all know, I don't know whether this is your first semester or I'm not sure. So totally there are three evaluation components for any of the courses. One which is called as EC1 component, which is mainly based upon the assignments. And the next, the other evaluation component is EC2.
Which is mainly and mid sem examination. OK, and finally, EC3, which is the final examination or final sem examination. Now, how the marks are distributed in this EC1 component will have 30 marks, EC2 will have...
30 marks, EC3 has 40 marks.
Okay, now what components are covered in EC2? That is evaluation assignment, EC1, evaluation component one. So in case of EC1, basically this will have the lab component where you'll be creating some dcb files and then you'll be in fact using an open ECU.
open ECU based on NXP microcontroller. You'll be using that and then you'll be performing some experiments. That I think would be roughly for 20 marks or so. I'm not sure, but five marks or 10 marks I would be giving for five marks for your situation learning assignment.
Okay, I'll be giving the guidelines on what is expected on the situational learning assignment. And then the other five marks, probably I'll be keeping it for, you know, what do you call us? I think, you know, probably I'll split like this, split this way. So 2.5 marks, I'll keep it for quiz.
whether it is subject phase or whatever it is, we will see. And then one more, either it is dcb file creation or it is dfdf file creation. I think I'll be giving it this as one more 2.5 marks as assignment. Okay. And EC2 would be, I think as of now, it is closed book.
where you are not allowed to carry any materials for the examination. Of course, pen and paper for writing. And EC3 would be an open book examination where you'll be shared the curated site. This is the current instructions that were given, but if there is any changes that will be informed in due course of time. Okay. Now, the next question that most of you would be asking is the question,
paper pattern? Is it a theoretical paper or is it only problematic paper or how is it? See, most EC2 questions are EC2. Most of my papers are mainly, you know, practicals, like, you know, to be mainly problematic. It would test, you know, your application of the concepts.
Application of concepts. I'll be directly giving, you know, the concepts or the problems related to, you know, what you call as, you know, can or probably Lynn or probably some IP, you know, so this is what actually you can expect. OK, don't expect, you know, theoretical questions wherein you can buy heart and then probably will be just...
you know, where you can expect, okay, I've written two to three pages, so I should be getting 5 marks. That will not work in this particular course. Okay, now.
So this is about, you know, EC2 and EC3. And next is the about the textbook and then the evaluation. See, textbooks, I'll be giving notes. I think you can refer to those particular notes. I think that should be more than enough because see, this is a very, you know,
A diverse topic and then it just keep on updating also. So all the required notes and materials will be given to you. You will have to refer to those particular materials. That should be enough. OK, so this is regarding textbook and the materials. Now coming to the evaluation part. OK.
evaluation part. So you would be asking how the evaluation would be done, whether it is tough or whether it is difficult or whether it is lenient. All I can say is just before EC2 and EC3, I will be revising all the topics for the examination. If you are clear with the examination, then
You can score very good marks in both EC2 and EC3. I'll be revising all the topics which are there for EC2 and EC3. And other thing is I would have been, or else I'll instruct my TAs that, see, when we answer, I'll be giving marks for the procedure and steps.
Procedure and steps.
If you answer a single line question saying that you have got so and so answer, even if it is correct, you'll be getting 0 marks. RS importance will be given for procedure and step, even if the numerical values are incorrect. If your procedure and step is correct, relevant, if your assumptions are correct, you'll be getting zero.
Marks.
At least 80 to 90% of the marks.
Okay, now I'll just enter my e-mail ID and mobile number. So in case of any questions, you can reach out to me.
So this is my e-mail ID. Don't use at the rate WILP because in Teams it reflects at the rate WILP, but you have to reach out to me on Pilani ID. So my mobile number is 0584609.
Okay, just reach out to me on this particular mobile number. Please call and then get it clarified. So if you say that, you know, if you send WhatsApp messages, definitely I'm very lazy to type on WhatsApp because there will be so many messages, it becomes very difficult to keep track. Yatar, I'll be.
sharing, you know, the important reference documents or whatever it is. See, this is a constantly evolving field. I don't think so sticking to one book is of, will make some sense. I'll be sharing the articles, everything which you can refer to, which actually discusses some practical use cases. Okay. All the required materials will be shared with you all.
Now, if you are having any doubts, please ask or post it on the chat box. If not, we'll conclude today's class.
And also, don't, again, I'm reiterating, don't hesitate in asking dot. Don't think that, you know, if I ask, this would be a silly dot, or else don't think that I won't be responding. I'll respond to each and every dot. And then if you are having any queries, or if you have not understood, or else if you feel hesitated that.
You know, I cannot ask in the class. Please call and then get it clarified. Okay. If I don't pick phone, definitely I'll call back. If you don't get response, you can call me back again. I don't have any reservations in terms of being called multiple times. Okay. Again, I'm saying no prerequisites.
I'll be explaining each and every prerequisites as and when required. So you can come to the course and every class open mind and invest your time in learning. OK, so we'll meet next week and we'll discuss, you know, V-to-V communication. OK, the parameters that are required to understand.
the V-to-V communication.
Okay, thank you all.

Shree Prasad M . stopped transcription