Automotive Communication Systems (S1-26_AELZG513)-20260801_154306-Meeting Recording

August 1, 2026, 10:13AM

2h 0m 41s

Shree Prasad M . started transcription

Shree Prasad M .   0:03
Act so couple of the shortcomings of.
The onboard sensors such as the radar, camera, and LiDAR, and which motivated us to enable communication between the 2 vehicles or communication between the vehicle and then infrastructure as welland with the OEMs planning for.
Predictive maintenance. It is essential for vehicles to communicate with the network as well. So we have seen the communication, different types of communication, the vehicle to another vehicle or else it can be vehicle to pedestrians or vulnerable road users. It can be vehicle to.
Infrastructure such as the traffic lights or it can be vehicle to network as well and that's why this is written as V2 X. That is X indicates either it is another vehicle, pedestrian infrastructure or network, hence it is called as vehicle to everything communication.
We have also seen the different kinds of application under each of these scenarios. Either it is collision avoidance or it is safety systems. In case of vehicle to infrastructure communication, it is traffic signal and priority or else we share the traffic light information.
Then again, vehicle to pedestrian it is safety alerts to pedestrians and then bicyclists. In case of vehicle to network, we can actually transfer our vehicle data to the cloud as well. And what in case of B to V vehicle to communication, vehicle to vehicle, what does the information that is being shared with the other vehicles?
So these are what are called as vehicle kinematics data. Either it can be location, which is in terms of latitude or longitude, it is in terms of speed or acceleration, heading, so on and so forth. And it is called by different names under different countries which have their own standards.
Say, for example, in US, it is dedicated short range communication which is based upon WiFi, and that is what we would be discussing. The name name that is given to the messages is nothing but basic.
Safety message which is also called as BSM and in case of Cooperative Intelligent Transport System that is in Europe, it is cooperative awareness message. So likewise you can see the multiple safety applications that are listed here.
Okay, so these are all what we had discussed for different for vehicle infrastructure and also one of the upcoming application that was discussed was to know the platooning. Now I will show this particular diagram and then later on you know we will start.
Discussing the concept of bandwidth and then channel or what actually is a channel. So this is roughly the DSRC channels and when I say DSRC channel, it is US standard or else it was employed in the US. You can see the different frequencies here.
The boundaries and then these are the start frequency and end frequency for the channel name 172 and as I have mentioned here, the basic safety message and then the vehicle to infrastructure communication. Is my voice breaking to everyone? How is it? Is it the same for Dhanush and everyone?
Just a second. Just a second.
In any case, I let me just say it once again.
Please let me know in case there are any disturbances, okay?
No.
So now that is identified for transmission of V to V messages or V to I messages. Basically, in general, this operates in 5.9 gigahertz frequency band when it is now breaking.
Not sure what is the issue because I am close to the network. Okay, so this is 5.9 gigahertz frequency band at which it is operating. Now before we get into the details of you know the channel and then you can also see the specifications.
Given by the different standards used for realising this V2 X communication, either it is 802 dot 11 which is based upon the WiFi technology or else it is based upon V2 X technologies based upon cellular V2 X either it can be based on.
The 4G communication or else it is 5G communication. You can see there are different technical specifications that are given now. These technical specifications are closely tied up with one of the slides which I would like to update here. So it is the stopping. You can see that.
There is something called as line of sight communication, non line of sight communication, so which has impact on the communication range or the maximum distance of communication, which in turn has an impact on the stopping distance. OK, so everything is interrelated. It is not something you say that, OK, I will just neglect.
The technical specifications of V2 X communications and then probably I'll just try to design a safety system that will not actually work. Okay, so we should be knowing both the working of the mechanics of the vehicle and as well as the V2 X communication part. Only then we can build an entire system because.
There is no way to avoid or its just to focus on one technology. Okay, now as you have seen here in this case we if we consider this diagram, so there is an important thing here one it is the center frequency that is FC and also.
What is called as the channel bandwidth or bandwidth? Now what actually is this? And then we will try to understand because I do understand many of you are from not from this electronics domain. So it is important for me to explain the concept of what comprises the channel.
Just right here itself. Now what comprises a channel? The channel is comprised of you know one the center frequency which is indicated as FC. So this we call it as center frequency and next we also.
Want what is called as the bandwidth band. It is also indicating a group and then width is indicating how much wide it is. So this is what actually constitutes a channel. You see today is in today's class, what is the importance of you know the channel with?
Help of a simple example which is nothing but FM radio transmission. Then later on you can relate it with your even your intravicular communication protocols such as scan length and other technologies. Now, what actually is bandwidth? First, lets try to understand.
Now this is an example or a depiction of a speech signal which is recorded for some duration. This is a speech signal which is recorded for some duration. Now we want the signal to be transmitted over the air and for some reasons we also want to do some sort of.
Frequency analysis. Frequency analysis. It means to say that we just take a frequency scissor and then try to find out what are its constituent frequencies or else what frequencies it is made up of. This is similar to saying the relation between the molecules and compounds.
So the chemical formula of water is H2 O, you can breakdown into constituent elements, that is 2 atoms of hydrogen and then one atom of oxygen. Similarly, the signal time domain is broken down or else it is dissected into the constant frequency components and then it is found that.
The significant frequency components are in the range 0.3 kilohertz to 3.4 kilohertz. That is if you break the signal or else if you try to find out what frequencies the speech signal is made up of, we will find significant energy within the range 300 Hertz to.
3400 Hertz. Okay, so this is the frequency range. This is called as the bandwidth or else so this is the range of frequencies. Now it is always difficult to write, okay, the speech signal spans from 300 to 33400 Hertz, so that is why we define.
What was called as the bandwidth. So where the difference between the high frequency component is taken with the low frequency component, so that would result in 3100 Hertz. So that is 3400 is results in 3100 Hertz.
So this is called as the bandwidth. Bandwidth is most important thing because even in EMI, EMC, if the bandwidth is very high, chances of interferences are very large. Okay, so I think most of you who would be working on this would be aware of this now. So far we have discussed only.
The concept of bandwidth now, another example that probably I would like to consider here is for audio signals. What would be the bandwidth or what is the range of signals for audio signal? When I say audio signal, it is also in the background. You can also see multiple.
Music instruments. Any comments in the chat box?
Any 20 Hertz to 20 kilo, do you all agree with?
At this.
Okay, so the audio signals here actually it ranges from 20 Hz to 20 kilohertz, that is the range of frequencies. So say for example, let us consider an FM radio station. So say for example, FM radio station one and then FM radio station 2, both the stations are.
Playing an audio signal and both the signals are in the range 20 Hertz to 20,000 Hertz. I will just ignore the Hertz term over here. So both the signals are transmitting or both the signals are playing the audio signals within the same range. Now if these 2 signals are transmitted as it is.
Do you think we can hear 2 distinct audio signals from 2 from these 2 radio stations, that is FM one and then FM 2? Please comment in the chat box.
Can we hear or else the FM radio stations are transmitting the audio signals as it is, as it is, and can we hear? It is not possible. Even if we hear, probably we will be hearing an overlapping sound, just like you know at sometimes on mobile phones you will be hearing.
2 voices, one the intended voice to the person whom you have called and also in the background you would be hearing someone else noise also. So this is actually what is called as cross talk. Okay, so this is what you know signifies you know FM one or else you know 2 radio stations to how to overcome this particular scenario or else how to overcome this situation.
Comment in the chat box.
It is tuning. Nandakumar, you're close to this answer. So what we can do is we can transmit.
At or else will slightly shift and then transmitted at a different frequencies. In fact, when you select the central frequency, this is what you are doing. You are actually selecting at that frequency and from that you are extracting the audio signal. OK, so if I were to say for example, you know the FC one here they should be transmitting the signal at 91.1 megahertz.
And then this would be transmitting at 98.3 megahertz. So what you would do is you will.
So what you will do is you will just tune to the required frequency and then extract the required audio signal. OK, now before we go ahead, can anyone comment in the chat box what is the bad bet of the audio signal that is transmitted across FM radio stations? Any comments in the chat box?
What is the bandwidth of, say, for example?
It is not just the radio station will be transmitting at some specified frequency. Now let us assume that.
The radio station 91.1 megahertz.
It is transmitting the audio signal. So what is the minimum bandwidth that this station requires?
Okay, someone of you have said now 200 kilohertz, yes, the bandwidth here centered around the carrier frequency. Here the bandwidth is equal to 200 kilohertz. So any comments in the chat box as to what will be the start frequency?
And what will be the end frequency for this?
What will be the start frequency and what will be the end frequency for this?
See we can write 200 kilohertz as this is nothing but 0.2 megahertz. Okay, so the signal here will be transmitted if a radio station is centered at 91.1 megahertz then 0.1 megahertz towards.
Left side and then 0.1 megahertz towards the right side. So this would be equal to 91.0 megahertz and this is equal to 91.2 megahertz. So that is the total frequency range. So I can indicate here this is the band of frequencies or this is the bandwidth occupied by this channel.
So this is it means you say that the carrier frequency is 91.1 megahertz and then the bandwidth is 200 kilohertz or else it is nothing but 0.2 megahertz. Now what is that you can comment here? Now the channel bandwidth assigned here is 200 kilohertz.
But what is the bandwidth of the signal? Any comments in the chat box?
What is the bandwidth of the signal assigned here?
Dinesh, this is the question that I'm also trying to address now. What is the bandwidth of the audio signals that we are trying to transmit?
So it is probably even if you take the difference the range of frequencies 20 kilohertz or else probably it is I would say it is 20,000 Hertz -20 so this is equal to 19,00980.
Sorry for this. So this is 20,000.
-20, so this is 19,980 or else approximately 20 kilohertz. Now the actual signal bandwidth is 20 kilohertz, but the bandwidth that is used for transmission is almost 10 times of it, that is 200 kilohertz. So probably I would be.
Indicating the audio signal is centered around 91.1 megahertz and then it is just occupying the 20 kilohertz bandwidth within the 200 kilohertz bandwidth. This is like a person travelling on an private jet okay or else you know a single person travelling on an.
Big aircraft, OK.
Probably Santosh, I will try to answer your question. Okay now so why is it required? Why the channel bandwidth that is assigned it is significantly greater than you know what you call as the signal bandwidth whereas the reason is that it is because of you know the hardware reasons.
Because of the hardware reasons.
This is because of hardware reasons. You need to have probably kind of we don't have any ideal hardware. Basically, how do you separate in the? I'll just explain this one. See, basically the signal bandwidth here is you know what you call as 20 kilohertz.
But on the other hand, we have the channel bandwidth which is almost 10 times, which is nothing but 200 kilohertz. There are. This is because of not the of course environmental noise. Also, this is but because of the hardware design. See, basically the way we extract here is.
When you tune to this particular 91.1 megahertz, when you set the knob at this particular frequency, basically you are using a bandpass filter in order to extract the signal. Okay, basically you are using filters to extract the signal which I will show it over here now.
Which is shown in the blue colour here. Now you cannot actually you cannot actually design an ideal filter which will perfectly filter out this particular signal. Again, I am showing you know here it cannot do that way. We cannot do that way. There are no reason. So that is why we actually what we do is we actually.
Transmit or else we go for more bandwidth in this particular scenario. Dinesh, is this clear? This is because of the hardware reasons or this is because of you know, we cannot design ideal filters. This is one of the simple reasons that we can give. Of course, later on we will be seeing we go for more bandwidth so that we can just transmit more amount of data.
In fact, this is also visible for everyone if that if you can observe in WiFi modems at our house, there are 2 bands at which the WiFi operate 2.4 gigahertz and then one is at 5:00 gigahertz. Now the WiFi operating in which frequency band will provide higher data rate.
Or else you can see the YouTube videos without any buffering circle.
Without any buffering circle comment in the chat box 5 gigahertz. Reason is that higher the frequency more is the bandwidth that we get and hence we can actually watch YouTube videos without any problem or else without any buffering. But what is the advantage with 2.4 gigahertz? Any comments in the chat box?
What is the advantage with 2.4 gigahertz band? There the distance is large, the communication distance is large, although the data rate is very less. Okay now what it means to say here is that with the typical FM radio example that I have given, if you want to operate an...
FM radio station, okay, then you have to purchase a channel. Now what that channel is made up of, that channel is consisting of you will be given a center frequency FC and then you are given the bandwidth. So this is just like you know your plot.
The plot has a number. OK, the plot has a number. Plot number is your center frequency and then the area of the site is nothing but the bandwidth. The area of the site is nothing but the bandwidth. Now you can ask the question probably to answer Dinesh question.
So why is that 20 kilohertz and is the actual signal bandwidth, but still we are going for bandwidth which is 200 kilohertz. Although for 4 persons in a family, you may not require probably 2 BHK or probably not more than not even more than one BHK, but still we go for duplex, triplex and also that is the bandwidth. So.
It is for more comfort, it is for more easierness easiness for extracting the information. OK, now this is an simple analogy that I can give. So which means to say that in order for one way communication I need one carrier free or I need one channel. So for one way communication.
For one way communication.
I require one channel.
OK, now see we have.
Muthu Krishna, the distance is high, but you know the bandwidth is very less. We will consider that later. OK, the distance will be actually a large, but at the expense of lower bandwidth. OK, we will see how center frequency and then bandwidth are related in the later questions. Again, these questions are important because the center frequency.
As center frequency increases, the bandwidth also increases. If the bandwidth increases, it will also decrease the latency, which means to say that we can communicate or we can transmit the safety messages in a quick in a quick instances, which will actually bring down the stopping distance of the vehicle. So everything are related over here.
In fact, if you try to relate this with the FMCW radars now, what was the reason for transitioning from 24 gigahertz to 77 gigahertz range? You know, in case of FMCW radars, simple reason is that.
It is because radars are used for some short distances applications as welland for short distances application we require the range resolution in terms of one metres and for this we require higher bandwidth in terms of one gigahertz which is provided only at.
77 gigahertz span. So that's the reason over here. Okay, now coming back to our subject. So we have made an statement that one way communication it is one channel. Now in FM radio stations, do we observe single way communication or a 2 way communication?
Comment in the chat box.
Is it a single one way communication or else is it a 2 way communication?
Why it is a single way communication? If I want to make it as a 2 way communication, what does that I have to do?
Okay, apart from this.
VR receiver, one more channel needed.
Increase the frequency, okay?
Do you all agree with Abhinand's answer?
As it is not the question about the hardware, but...
It is the question about first.
See, it is not the question about whether we have see you may have the car, you may have the hardware, but the point is whether is there a road on which the car can travel. Okay, so yes, of course with one channel also it is possible if you use it on the time sharing basis, but let us not get into the details of that.
For time being, OK, so we can possibly have 2 way communication only if we have 2 channels. So this one channel is like a single road, OK, or one way road. So wherein you can go only on one direction, OK, you can go only on one direction.
Now if you want to have simultaneous 2 way communication, that is the traffic is flowing smoothly in the both the directions, then we require 2 channels. So that is why you can observe in this particular diagram.
The a mobile is shown. The mobile can also indicate a vehicle and then it is communicating with the network. Here, maybe you can think of an example where the vehicle is trying to Santosh. I will just address that. Also, I will try to address the question on walkie talkie as well.
So the mobile is communicating with the network, probably it is trying to push this center data into the cloud through the network and at the same time probably a software is getting updated on the vehicle. So its a 2 way communication. I am just giving an example so that you can see you can feel connected with this subject.
Now, when you are pushing data from the mobile phone to the tower, so this is called as uplink. Okay, again, uplink in the context of the mobile phone because we are generally located at a lower at a lower height as compared to the network or the cell phone tower. So we are just.
Pushing the data towards upside or towards up now again we would be receiving the data so which is called as downlink. You would have heard of uplink speed and then downlink speed as well. I don't know even now we know does that really matter. I am not sure downlink speed and then uplink speed and all OK.
Now for this uplink speed, so we will be having carrier frequency FC one and then downlink speed will be having carrier frequency F2 along with the bandwidth as well. Okay, now to answer couple of questions which is actually mentioned in the chat box here. Now if there is one way communication.
If it is one way communication, OK. If it is one way communication, it is called as simplex channel.
It is called as simplex channel. Okay, that is only one channel, okay.
Dinesh will be answering these questions probably in next class. Okay, so simplex it is called as simplex communication. We have only one channel, okay, and now but the point is we want 2 way communication also.
We want 2 way communication also now.
We can achieve 2 way communication such an okay, I will repeat. So if there is only one way communication that is just like your FM radio to your radio, then it is simplex channel. Okay, it is just one way communication. That's it. You are just receiving the audio signals.
In turn, you are not communicating with the radio jockey who is actually deciding with song to be played, correct? But on the other hand, in case of 2 way communication, you would you love to you also want to have 2 way communication. OK, in case of 2 way communication, there are what is called as.
Half duplex communication.
And full duplex communication as well. Sachin, hope this question is hope I have answered your question. Okay, now what does this half duplex and then full duplex communication now before I give the example of walkie talkie, walkie talkie which is a typical.
Half duplex based communication. Now for walkie talkies, do you require any infrastructure just like your mobile phones require, that is the cell phone towers?
Do walkie talkies require infrastructure just like you know cell phone towers, just like your mobile phone? It is not required. Okay, it is a self organised network. This is an really important example here. So this is an self organised network.
This is also similar to your V2 X communication. Even in case of V2 X communication, we don't require any infrastructure. The reason there are reasons are to one if the network infrastructure fails, okay, if the network infrastructure if the cell phone card fails then.
2 vehicles cannot communicate. And then the other reason is that 2 vehicles or multiple vehicles may be in the area where there is no coverage. So in those cases, we want you know, 2 vehicles to communicate directly. And these days, we have seen ingenious way of, you know, mobile phones communicating even in absence of.
Cell phone towers also using Bluetooth also people can exchange messages. I don't know what that app is called as, but I just forgot. OK, anyways, now what actually is this half duplex communication? Say for example, in case of walkie talkies, OK, 2 phones.
Or the phones or the handsets that is held by the defense people, okay. It can be either police or probably a law enforcement agencies or defense. They communicate directly. Okay, now have you seen any traffic enforcement or law enforcement?
People talking simultaneously on this walkie talkie, the way we walk, the way we talk over our cell phones.
No, either one is speaking, the other side is listening, and then later on the other side will respond. So here it is a 2 way communication, but the point is it is half duplex when one is transmitting. Say for example, if this is transmitting, there is no transmission from the other side.
So once when this is done, probably I will just call this as time instant one so and then again after next time instant T2 this will transmit. So this is half duplex communication. This is just like saying a single road being used for traffic on both the sides, but only one side at a time.
So this is half duplex communication, but in case of full duplex communication, we can allow traffic simultaneously both ways. When you are speaking on mobile phone, you can speak either ways. Okay, so this is it. After this, what we can comment on the nature of V2 V communication so far, whatever we have discussed.
Is V2 V communication an half duplex communication or a full duplex communication?
Really full duplex communication going by the frequency band that we have specified here.
Say for example, we have only one channel.
Which is channel number 172, which can be used by all the vehicles in order to transmit the message. So can 2 vehicles take access of this channel number 172?
I have got only one channel, channel number 172. This is just like saying I have got only one FM radio channel. Now with this FM one FM radio channel, how many stations can operate simultaneously?
How many stations can operate simultaneously, either one or probably multiple stations or else with this one channel 172, how many vehicles or how many cars can transmit the message simultaneously? One or multiple at any instant of time. Please note at 1:00 instant of time.
Multiple cars or multiple FM radio stations can exit with only one channel, but do you think can they exist at only at? What about at a single instant of time?
Now please note, I think this is where you will have to get your basics at 1:00 at any instant of time. OK, there is only one channel that is available for transmission if one vehicle is already using it. If another vehicle also tries to transmit, then there will be an.
Overlap just like overlap of the voice signals. Okay, so that is why you have your protocol such as CSMA protocol. Heard of the CSMA protocol in any of the intravicular communication?
Heard of this protocol anywhere?
CSMA protocol even is the used in can protocols also even in can you have a single web and there are multiple issues which are communicating. So there it goes for CSMA. I will be explaining these things in detail. We need not have to worry. Now we have a single channel.
Which is used by multiple vehicles. So can 2 vehicles talk simultaneously?
With one channel, can 2 vehicles talk simultaneously? It is not possible, hence V2 V is also half duplex communication.
Is it clear to everyone?
Any doubts or any queries please let me know in the chat box. OK for now. But we will have to understand more topics because before I get into the details of other questions that is being asked in the chat box, I should also explain what is nothing but cellular networks that is very important.
And then we'll move on to...
What is called as because this is critical for vehicle to network communication, at least how the frequency bands are assigned and so on and so forth. That's it. That is what I will be considering here. Okay, now if you see here in this particular case, you can see.
This is okay here also you can see the vehicles or the vehicle and then the mobile phone is communicating on this is uplink. I will call it as welland then this is nothing but downlink. Okay, now if 2 see when we here comes the concept of.
Uplink and then downlink if the communication or else if the mobile and then if this particular vehicle is community communicating via this particular network. Okay, so this is how a typical 4G or 5G network works.
Even though both the vehicles, that is the car and the way the mobile phones, if they may be in the vicinity, but the traffic is routed only through the network.
Dinesh, this is actually a good question. If 2 vehicles try channel, which one gets the priority? How it is priority? So these aspects we will consider when we take the layer called as MAC, Medium Access Control. OK, all these things will be discussed.
Okay, now in if 2 vehicles interact directly without the involvement of the infrastructure or the network as shown over here. So this is actually called as side link communication. This is a new kind of link that was actually added in 4G standards onward that is 4th generation.
Mobile communication onwards. OK, please note this is I have coined 3 terms over here. One is uplink.
Next is downlink. These are all related to infrastructure based communication that is via the network.
We are the network and next is nothing but the side link where there is no direct. So this is direct communication between the 2 vehicles. OK. And also when I have illustrated the entire ecosystem here where the 2 vehicles communicate via side link.
And then you have mobile phones and then base stations working I know communicating in case of cellular V2 X. Please note that I am just explicitly specifying cellular V2 XEV. There are 2 different technologies.
For implementing for exchange of safety messages, one is cellular V2 X communication and then the other one is based upon IEEE 822 dot 11 P or this is also called as BD. So these are 2 different technologies specific to cellular V2 X.
Okay, logically the links where the entities directly communicate with the network, it is called as UU interface. This is just like the names that we give for different roads, right? You know, this is called as I don't know in Delhi, you will have the Janpath road and then different other roads are given.
Different names right in the same way the highway of information where the mobile phone interacts with the base station it is called as UU interface and then this direct interface or this side channel this is called as PC 5 interface. OK, so these are the standard names that are given.
Now again in this case in order to achieve simplex or duplex communication, okay, simplex or duplex communication. So this is done in I think I would better rather discuss another slide and then later on I can actually explain these things, okay.
So let me just switch over to that slide and then I will explain. In meantime, if you are having any doubts, please let me know in the chat box.
All these channels are.
All these channel and then everything has to be cleared to everyone. OK, so I'll open one another slide and then probably we will go ahead, OK.
So don't think that a particular doubt would be silly. It doesn't matter. You should get all your doubts clarified. That is the primary importance over here. OK, so let me just open this.
Any doubts? Okay, so I don't see any doubts here. So let me just open the presentation.
Okay, couple of things I have already explained over here. What does the you need not you? If you want, you can go through this particular slide, but for us, the prime importance is of the channel concept which I will be discussing now. Okay, now say for example.
You can observe this particular frequency or even say, for example, you have purchased some bandwidth. Okay, so when I say bandwidth, it is just like, you know, you have purchased some band of frequencies. I will just explain this. Let me just go here because I can explain.
More concepts. So with this, so let us assume that just like you can be an operator or whom you purchase the frequency band from 2400 megahertz to 242404.2 megahertz.
Okay, so this is the frequency band that you have purchased. Now, what is the comment in the chat box? What is the total bandwidth that you have purchased?
Comment in the chat box. What is the?
Total bandwidth that you have purchased.
So you just take the difference so that would be equal to 4.2 megahertz. So this is the bandwidth that you have purchased and we will be using this particular bandwidth in order to support or in order to provide communication for a group of users. Okay, say for example, you know you can, you may want to establish your own.
Cell phone network or whatever it is or some sort of communication system which will help people in communicating. OK, now lets say assume that just like your FM radio channel which is used for transmitting audio signals, what I will say is that OK, in this particular case, I have a very good hardware and then I will decide.
The bandwidth of each channel that would be for either uplink or downlink transmission that would be 100 kilohertz or else I will say that so this is equal to 0.1 megahertz. So this is equal to 0.1 megahertz. Now don't worry about this particular U1 U2 till U.
21 or probably D1 till D 21. Now please let me know in the chat box how many individual channels are available.
How many individual channels are available? Please comment in the chat box.
How many individual channels are available? It is the total system bandwidth that is 4 point megahertz divided by 0.1 megahertz. So this would be equal to. So totally there are 42 individual channels. How do we use this particular 42 individual channels that is left to us?
And these individual channels is named as simplex channel. This is named as simplex channels. OK, now you can see here the division or else the start and end frequencies of each of these individual channels which are named from U1 to U 21 and D1 to D21, which will be clear as to why it is called as.
U1 to U 21 D1 to D 21. Now say for example one individual channel here it is named as or else its starting frequency is 2400 till 2400.1 megahertz. OK, so this is the start and end. Likewise I have indicated the start and end frequencies for all the.
Channels. OK, now say for example, now if I consider channel number U2, you know what is its bandwidth, what is its start and end frequency? Now what will be its center frequency? Any comments in the chat box?
What will be its center frequency?
Is it? It is exactly the midpoint of this. It is exactly the midway between the.
Say for example, I can just write this as the center frequency is equal to the extreme frequencies that is 2400.2 and then 2400.1 / 2 exactly the midpoint OK, so that would be equal to 2400.
0.15 megahertz. So you can see that is the exactly the midpoint. Does it make sense? OK, now same way you can calculate the frequencies for all or else you know the center frequencies for all the particular channels and then we can also.
The bandwidth remains the essentially constant, that is 100 kilohertz or 0.1 megahertz. Now what I will do is I will use 1/2 of the channel for uplink transmission and then the next half of the channel for downlink transmission. So as you can see here, the first 21 channels is used for up.
Blink transmission, so that is why it is named as you and then the next half of the channel is used for downlink transmission. Okay, so now if a user or else, say for example, you are a cell phone operator and then a cell phone or else and a customer request for a channel or else for a communication.
Then we will be assigning, say for example, here the channels I will just indicate like this: channel U 17 for uplink transmission and channel or else D 17 for downlink transmission that is uplink. The user will be transmitting his voice.
Downlink he will be receiving the voice from the network, that is from the person on the other side of the network. Now please note that yes, actually in fact I was about to raise this question. All these channel are assigned in pairs. OK, say for example, along with U 17 I will be assigning.
The channel number D 17 I cannot assign D 16 or D 18 or even D1 all. So what is the reason for this? Comment in the chat box.
What is the reason for this?
See, if I assign the channel in pairs, what will be the frequency distance between the uplink and then downlink channels? Comment in the chat box.
What will be the distance between the U 17 and then D 17? What is the frequency separation? Can anyone calculate? You can see that it will be equal to 2.1 megahertz.
Correct. Am I, am I making any mistakes over here? Yes, it is 2.1 megahertz. So irrespective of the channel that you assign for a particular user, OK, you assign for a particular user, it is the same. It is 2.1 megahertz is the because see.
It is not only about maintaining, maintaining the frequency distance between the adjacent user, but it is also about see if you are talking and then the other person is also talking simultaneously, you cannot make out anything. Say for example, at the mobile phone, we will have to separate what you are transmitting and then what you are receiving only then.
Will be able to hear properly. So that is why this 2.1 megatz suppression is really important. Now with this pair of channels, whether you can provide simplex communication or duplex communication and also in duplex whether it is simplex or full duplex communication, please comment in the chat box.
Duplex or simple? Simplex or full duplex?
Please note here since we can actually provide full duplex communication.
We can actually provide full duplex communication. Okay, now with so how many pair of channels are available with 42, you can see here totally there are 21 duplex channels. Please note that duplex channels. So with this you can provide.
Full duplex communication. When I say full duplex, it is made up of 2 simplex channels. Okay, now with this 21 duplex channels, how many simultaneous phone conversations are supported or how many simultaneous radio transmissions are possible?
So totally 21 can communicate simultaneously.
Okay, so this is actually called as capacity.
For traditional voice network, of course, for data networks it will be totally different. Okay, so this is the capacity now. But do you think in urban cities or in urban scenarios you cannot have or else being a cell phone operator, you cannot say that I will support only 21 simultaneous phone calls.
Right, because there are thousands or lakhs of people who are trying to connect simultaneously. So how to solve this particular scenario?
Correct, see I cannot one. One of the solutions that you would suggest is okay, let us go for purchase of additional frequency spectrum. But if you are going for additional purchase of frequency spectrum, then it will incur cost. You will have to pay money to the government and then which is actually kind of.
A burden for the OEM. Okay, so what we do is we actually go for what is called as dividend rule policy. Okay, now what actually is that particular dividend rule policy? Let me just explain this one and also please note this scheme is called as frequency division duplexing.
This is called as frequency division duplexing because in this particular scenario for uplink and downlink downlink we are using 2 frequencies that is FU one and then FD one. So that is why it is called as frequency division duplexing. In case of duplexing that is 2 way communication.
How this is actually done, whether it is using frequency or whether it is using time. So this is the this is what actually it means. OK, now next.
Say for example, now what we will do is in this particular case. So essentially what we have said here is we have one particular, don't look at this particular slide for time being. OK, so what it essentially says is that.
As a cell phone operator, you.
You consider an city which I am representing as a rectangle shape. Of course, it may not be the case, it may not be always rectangle. It is not rectangle in first place any cities at the boundaries. So now I am saying you know, OK, I can support only capacities only.
21 I can only support 21 simultaneous users, but I want to expand it. So here the concept that we employed was we will use a single network or a single base station which is high powered base station which is covering the entire city.
And because of which we can provide only 21 or we can accommodate only 21 simultaneous phone conversation. But with divide and rule policy, say for example, you can see here with divide and rule policy, what I do is rather than using single.
Cell phone tower, we will use multiple cell phone towers, of course, use of multiple multiple cell phone towers. So it will, you may ask, is it not increasing the cost? Yes, it increases the cost, but its is it is the cost is not as high as purchasing the spectrum. Okay, so in this particular case, what I will do is.
Say, for example, here you have 3 distinct regions which you can observe over here, okay, which is called as a BNC. Now, each of these particular hexagon shape regions are representing the coverage area. These are representing the...
Coverage area and each of these particular hexagon shells will have one tower which is responsible for covering only the prescribed or its corresponding hexagon cell. Is it clear? Say for example, I can put a cell phone tower here now the cell phone towers responsibility is to provide.
Radio coverage only for this particular shaded region. OK, now so like this you have 3 distinct regions as I have shown over here. So here there are 3 distinct regions. So now what I will do is the 21 number of channels that is available will be distributed among.
These 3 cells and that is shown in this particular table. We can observe here that the channels, say for example, U1 to U7 uplink followed by its corresponding chair, corresponding pair of channel is assigned to cell number A, likewise for cell number B.
U8 to U 14 DA to D 14 and then for cell CU 15 to U 21 and then D 15 to D 21. Does it make sense? Okay, so I will write here. So say for example, this is occupying the channels or this is being assigned the channel one to.
7, this is 8 to 14, then this is 15 to say for example, 21. OK, now earlier we had taken or else we had considered one single cell phone tower which is providing coverage to this particular large region. Now we will use these particular.
Hexagon cells which are of smaller radius which you can see over here. Now you can see this distinct group of cells that is ACB. This is called as a cluster.
Yes, Sir, this is called as a cluster. Now each of these particular cells in the cluster has got how many channels. Any comments in the chat box? Each cell in a cluster has got how many channels. This is having 7 channels. Please note it is having 7 channels, OK.
Now I will repeat this particular cluster because the cells are providing essentially coverage or a small region. So I'll repeat this particular cluster once again, I'll repeat this particular cluster, I'll repeat the clusters until the region or else until we can provide coverage to the entire region. Of course, there will be some blind.
Spots over here. OK, let's not worry about this. So these are all the blind spots. Let's not worry about this. Now, do you think the capacity is now the same as the previous scenario? In the previous scenario, this entire region here, the entire region here was providing.
Or provided coverage only for 21 simultaneous users, but here this small region itself is providing coverage for 21 users. But we have used a cell phone tower with small power or low power. Now, how many times you know this cluster is repeated one?
23456789101112131415161718 and 19. So 19 times or there are total 19 clusters which are you know what you call as.
Which are providing coverage for this entire city. Now each of these cluster, how many channels are available for each cluster?
For each cluster, how many channels are available? 21. Now can you compute what is the capacity?
Can you compute what is the capacity? It is nothing but 19 * 21. I think some one of you have already calculated, so that would be equal to 399, correct? Is it clear to everyone now? See the increase in the capacity. Earlier it was.
21 and now it is equal to 399, so almost it has increased by a factor of 20. So this is the increase in the capacity. I don't know how many of you have used the traditional cell phones, those Nokia phones back in 2010 or 2012.
Whenever you move from one area to another area, besides besides or below the cell phone tower or below the tower strength indicator on the mobile phone, it will also indicate the network name from which it is receiving the signals, the area name.
Right, this is the essentially that is what it does here. We are dividing the entire region into small clusters. That channel of the same frequency can be accessed simultaneously in different clusters. Hari Babu, this is actually good question. What you are saying is, say for example.
From here to here, is it possible to access? We design the system in such a way that the signal will not essentially leak out from this particular corresponding cell.
Okay, I think I have answered your question, Kartik, how to map this multiple of clusters with respect to an area? See, this is all, you know, again, radio engineering or probably this is actually not as simple as it sounds, but this is the concept. In fact, you know, deployment of the networks actually it has.
lot more difficult than what we think. Okay, Abhinand, what happens if more people than the capacity, you know, where it is, tries to make a call, then you will get the user is or else you know the network is busy. I think who would have experienced this specifically during.
New Year days OK around 12 midnight everyone will be trying to or everyone will try to make call for their near and dear ones. At that time the cell phone network will be totally jammed. I don't know these days because with WhatsApp and all its lot more smoother but back probably at least you know 5 to 6 years back.
Getting calls connected on New Year Eve was really difficult. I hope you can actually relate it. OK in border areas or said how 21 is getting multiplied by 19 since the same say for example E every the cluster is repeated 19 times.
Or else probably if I were to draw this one, or else if I were to explain this even more in detail, see each of these cells are getting 7 duplex channels, correct?
So that means you say that one cluster here is getting 21 channels. Now if I write 7 channels for each and every cell here and then if you multiply it, OK, so you'll be getting 21 * 19 itself, OK, in broad border areas or city states where.
Usually comes of signalling what happened state usual issues comes of signalling coming in. See, this is because of the poor network coverage. See, usually the network operators try they give coverage areas where actually there is a revenue, right?
If there are few, very few people, why they would bother? You know, border areas are probably nearing the borders. Not many people would be there. The population density is very less. So that is why they won't bother much about you know the coverage area. So the coverage, see again, you know my you might ask the question here, is it like exactly the coverage is hexagon shape?
It is actually no.
Actual practices, it is the here we are assuming the coverage areas are all uniform, but in actual practice the coverage area from one cell to another cell will be totally different. But this is the concept. What does this cluster have? Say for example, this cluster has.
No, say for example, one cell phone tower and then all the mobiles within the cell would be connected to that tower. Does it make sense, Dhanush?
Is it all your doubts are cleared? OK, so I will take a break of 5 minutes. The time is now 4:48. Lets resume the class at 4:53. In meantime, if you are having any doubts, please comment in the chat box.
Okay, there is one more question, but the frequency is limited then it is possible. Yes, we are actually we are using the frequency. Say for example, I will give you the, I think the best example for this is say in Bangalore we are using the radio station 91, 1.1 megahertz.
Okay, say for example, so this is in Bangalore. Now, do you think that Chennai is not using the same frequency?
Or don't you think a radio station in Chennai would not be using the same frequency in Chennai also they also use the same frequency 91.1 megahertz. So of course you know yes I think you know it is confirmed Karthik confirms thank you. So now how come you know we are using the same frequency both in Bangalore and.
In a Chennai, the reason is that because the signals that are transmitted in Bangalore, it is not going beyond Bangalore region. Ideally, same thing with Chennai, what actually the signals transmitted in Chennai, it is not travelling beyond Chennai. So that is why here what the say for example, the region I am highlighting here.
Is not transmitting ideally into or spilling over into the adjacent cells or the same cells in the different cluster.
OK, but frequencies, do we have sulfur? Yes, we have sulfur sulfur for each, not for each cluster for each cell. If I were to show the diagram here where this particular diagram went.
This is how it is a typical old network, your 2G networks, you can see the towers for each and every cell.
Covers for each and every cell.
Is it clear?
Okay.
So if there are any queries, please let me know in the chat box.
OK, I'll be on mute for 2 or 3 minutes and then I'll resume the class.
Okay, there is one more question. If it is for each cell, then every cell see each cell has a capacity of only 7 channels. Each cell here, if you see this particular table, each cell has a capacity of only 7 channels because we are dividing it among.
3 cells, OK, we are dividing up it among the 3 cells, OK, so that is why so as a cluster, it has got 21 channels, that's it.
So I think I have answered your question Muthu Krishna. Next is can we make you 2 as downlink? No, this is one more question which I will have to address. I will come for that in a moment. If I take an a call in area one, if it refer it cluster, I got walk while I am going.
Then see, okay, Dinesh, I understand your question. What happens? You know, a person moves from one cell to the another cell when during an ongoing call or probably doing an ongoing transmission. That process is actually called as handover.
We will not go into the details of this, OK?
Any other doubts?
I think I have clarified your doubts. If still, if you are unclear, please let me know in the chat box.
Okay, Dinesh, see what you're saying is during an ongoing transmission.
If the mobile phone moves from one cell to the another cell, is your question correct?
Have I understood your question correct? So if during an ongoing call if the cell phone moves from C to B then there is a process called as handover and this happens in milliseconds and then the user is actually unable to notice this one will not get into the technical details of this is not required, this is not within the scope of the syllabus and then we don't.
Don't require also, okay.
Is this clear, Indira Dinesh?
Any other questions?
Okay, so there is a question. But at the time of initial explanation, each tower was mapped with 21 capacity. After that, we came to cluster topic. How this is splitted to 7? See, initially what we proposed was okay, we will just use a single cell phone tower.
If we use a single central tower which is providing coverage to the entire area, then we cannot increase the capacity beyond the channels that I have purchased, that is 21 channels. But with 21 channels with the amount that I have invested, I cannot make any profit. so.
For this reason, we will use multiple cell phone towers whose coverage area is restricted. As you can see here, and in this particular case, I have just said that the cluster size is 3, that is the channels are divided distinctly and uniquely only among.
3 cells and the cluster is repeated. OK, so this is the concept that we are proposing and this is how it has been working for almost a century since the first generation of mobile communication. I think probably it was, I think more than 70 years or so, OK.
So this is how it is working. OK, now what will happen if we split among 7 different cells? So as you can see here, rather than cluster size, here the cluster size.
Was equal to 3 here the cluster size.
Is equal to 7.
Okay, the cluster size is equal to 7 and you can see the club there are totally 7 cells over here. There are 7 cells now in this particular case you can see how many channels, how many duplex channels does each cell will be getting? Any comments in the chat box for 7?
Seriously, 7, please note that this is one cluster. One cluster has 21 channels.
21 divided by cluster size that is equal to 3. OK, now we will repeat the same thing for this particular area, but here the cluster size is equal to 7.
So now you can observe that in this case you can how many times you know the cluster is repeated. Any comments in the chat box?
How many times the?
Cluster is repeated 123456789 and 10 approximately 10 times. So there are 10 clusters into and each cluster has 21 channels. So what is the capacity here?
The capacity is totally 10 clusters into 21, so that is equal to 210. Now, what is the observation here?
For cluster size 3, what was the capacity?
The capacity was equal to...
399 but in case of capacity or cluster size 7 the capacity is equal to 210. So what is the impact of selecting a higher cluster size comment in the chat box?
What is the impact of selecting a higher cluster size? It is it improving the capacity or else is it decreasing the capacity?
And the capacity is being reduced. So ultimately what you will do you will. So what is the guidelines for deciding the cluster size then?
What is the guidelines for selecting the cluster size? So should I go for a smaller cluster size or should I go for more cluster size? Okay, lesser clusters, so smaller cluster. Okay, now I will just explain everything with diagrams.
I'll skip actually few details this time because I'm more interested in covering some extra topics like some IP learn and can in more detail, so I'll not be covering few topics which is essentially not required. Also, now looking at this particular diagram, which we call it as tessellation where.
The hexagon cells are arranged one by one. What is that you can? What is the cluster size over here?
What is the cluster size? The cluster size is 3. Of course, I had actually had to hide after asking this question because it was displayed at at the top of this particular diagram. So this is called as tessellation where it shows the hexagonal arrangement of the cells. See, you can how many distinct alphabets you can observe over here.
In this, it is totally 12, and 3. So this says only 3 or the cluster size is 3. Dhanush, is this clear? OK, now we can observe here that so this is one cluster.
Okay, I will just draw it in border and then you can see the all the clusters are arranged one by one. Now let us consider the cell under consideration. Say for example, when I say cell under consideration, say for example, your cell phone or you are located at the boundary of this particular cell.
Okay, now because of this desolation, you are also surrounded by cells operating in the same frequency. As you can see over here, I am just drawing this. You can see you can also observe here you can observe cells operating in the.
Same frequency.
Differentiate difference between cell and a cluster. Cluster is a group of cells. Okay, cluster is a group of distinct cells that is A Bc. Now if you collect the channels corresponding to the cells A Bc, then you will be getting the channels assigned for the entire system. So that is nothing but cluster.
Does it make sense? I will show you this pictorially. Also, if you consider this tessellation diagram here, A is nothing but a cluster here, okay, but A, Bc, know the group of distinct cells and this 1a, this is called as a cluster. Okay now.
Lets assume that you are standing at the border of this particular cell and then you are surrounded by the cells in the different clusters operating at a different frequency. Now, do you see any problem here?
ideally, do you think that the cell A here, okay, is it like, don't you think the radiation or the radio signals will actually extend or else travel beyond A also? Any comments in the chat box?
Don't you think the see it is just like saying, say for example, whatever signals that I am transmitting here it will just be restricted to the boundary. Just like you know, I have just placed one radio fence which is not allowing you know, the signals to go beyond this boundary. that.
Cannot happen. In fact, the signals that you know it will actually spill over into the adjacent cells also. Now will it not create problem for the adjacent cells? Any comments in the chat box?
Why?
Will it not create problem for the adjacent cells?
If it is operating at different frequency, what is the advantage?
You can apply filters, bandpass filters, and then you can reject the unwanted signals. Just like the way you set your frequency on the radio, I will select only the different with the required frequency and all the other frequencies are actually automatically filtered out. but.
Here.
The signal from this particular tower tower, I'll use it. I'll use a different colour here.
Okay, I will use a different colour. Do you think the signal from the cell here will spill over to the cell which is marked in the yellow colour area? That is, will there be, will you also be receiving assuming that you are standing at the edge of the cell, do you think?
Will be receiving any signal from the other cell in a different cluster which is operating in a same frequency? The answer is maybe or the answer is definitely yes.
The answer is definitely yes, and because of which if someone in this particular cell OK is using the same channel for conversation, you will hear that voice also that is actually cross called as cross talk. OK, that is called as cross talk. Is it clear? I will repeat this particular.
Question here and then it will be clear. Let us say assume that you are standing at the edge of the cell here which is marked in green or sorry which is marked in yellow. Then again I am just say for example there is someone else marked by green in this particular cell.
Where the that person I am just indicating it by black. If these 2 persons shown by red and green or black, if they're if they're active, if they're in the active conversation at the same time, and if they're using the same frequency, then definitely there will be interference amongst each other.
Okay, how to avoid this crosswise mostly Karthik, this is what this will happen when the distance between these 2 cells which operate at different or same frequencies, which we call it as Co channel cells, this is called as Co channel cells.
When they're actually near.
OK, when they are actually near now, how to avoid this particular, you know what you call as, and do you think you know it is receiving interference from only one core channel? See, for me, the useful signal is the signal that I get from my own tower. Any signals that I am getting from a different tower.
To which I am not connected to, it is nothing but interference. Do you all agree?
I will repeat this for this person in green for this person or else you know I will use the same colour for this person in yellow the signal that he receives from a which is in yellow shade it is an useful signal. Now any signals that he or she is receiving from this.
Green coloured shade that you can actually observe over here. Okay, which you can observe here. This is actually interference. These are unwanted signals. Unwanted signals are always interference signals. So now comment in the chat box the person indicated by brown colour brown into.
From how many cells he or she is receiving the interference?
He or she is receiving the interference.
Is it just one? Is it 2 or is it 3 or is it 4? It is from totally 6 Co channel cells and you can see the 6 Co channel cells. These are called as the first tier Co channel cells.
Which I am indicating it by green coloured line over here. So these are interference. So the way I can write this here is the useful signal power USB USB is I am calling it this as useful signal power then followed by the interference power from.
One Co channel cell that is 123 OK then again followed by 4 and this is 5 and then 6 so plus I2 + I3 all the way till I6. OK now for.
The crosstalk levels to be very low, the useful signal power should be see in this case numerator is representing the useful signal power, the denominator is representing the unwanted power. So for an quality conversation, what is what is required?
Do you expect the numerator to be greater than denominator or the vice versa? Do you think the useful signal power should be greater than the interference power, or else do you think the interference power should be higher than the useful signal power?
In presence of interference, if you have or else if you are in a room where multiple people are speaking at the same time, for you to be heard, will you speak in a very lower voice or will you start speaking loud?
Interference is higher, right? So you will have to speak louder, but that means to say that the numerator should be greater than denominator. It is not like saying you know you will increase the transmission power, it will have the same effect eventually. So you will actually make the numerator greater than denominator. So how to make this one?
Okay, the only possible way to make this one is in this particular case. Please note that the signal power, whether it decreases with distance or whether it increases with the distance travelled.
The signal power, whether it increases with the distance travelled or whether it decreases with the distance travelled, it decreases right? So the only possible way is increase the distance between the Co channel cells such that a required signal to interference ratio is.
Maintained, that is when I say signal to interference ratio, it is useful signal power to interference power, that is the ratio. OK, so likewise you can see here. So this is for the cluster size 2. So for most of you who were asking, OK, the cluster size is 3, great it.
Provides large capacity, but the point is it may accommodate more number of users, but most of the times the users will be experiencing a bad service quality because they'll be experiencing cross talk. So what I can do is I can go for a large cluster size over here.
OK, so you can see here if I increase the cluster size, what has happened to the distance between the Co channel cells? Any comments in the chat box as it increased or has it decreased?
With the increase in the cluster size.
With the increase in the cluster size, the distance between the core channels has increased, and because of which we can achieve a quality communication. When I say quality communication, it is what do you call as a communication without cross talk. So always the trade off here is between the.
Capacity of the network against interference can always counter. They always go in orthogonal directions. Okay, so again, I am not getting into the details of this one. All I want to give is one simple formula just for the sake of calculation.
Okay, because this is really important. So how do we calculate the signal to interference ratio? Okay, the signal to interference ratio I will be giving the summary slide which you can refer to. It is given by square root of 3 into n.
Where n is called as the cluster size divided by the number of cells that are causing the interference, that is the 6, and then n is nothing but the path loss exponent. So this is actually important.
So this is called as path loss exponent. Now what actually is this path loss exponent? Any comments in the chat box?
it is taking into account one the cluster size which we have already seen larger the cluster size, you can see larger the cluster size, higher is the signal to interference ratio. Now, what actually is this particular? Yen, can anyone comment in the chat box? What is this path loss exponent?
Not distance, yes, a good attempt anyways.
Power. OK, now in which scenario the signal quality will be good, whether it is in urban areas or whether it is in rural areas?
Where the signal I see, okay, lets assume this way where we get a very good signal quality, whether it is in of course in urban areas because the population density is more okay, even in urban, even in urban areas, where do you get?
Good signal quality, whether it is in a clear space or where there are more blockages.
Its in clear space, correct? So I think this is this is good answer. This is in clear space. Why? Because in clear space, you know, the rate of power decreases very less as compared to the power decrease in case of rural areas, and that is nothing but the.
Factor small n OK, this n here is nothing but the cluster size and then small n is nothing but the path laws exponent. It says that what is the type of how we know the power is actually degrading in a specific environment, ideally if it is a clear space.
We will be taking north is equal to 2, the path plus is equal to 2 in clear line of sight communication. In case of urban scenarios, it is greater than 2, it will be 3 or around 4. Okay, now we will solve a particular problem then actually it will be.
Much more clear to you and this is what I intend to discuss in this particular topic, nothing much more beyond this one. All you want to know. I will also tell you know what are the important points for this particular examination. OK, see essentially we are computing signal to interference ratio.
Now, what is the signal to interference ratio? This is nothing but the useful signal power divided by the interference power which is received from 6 Co channel cells. This is what you should remember. Now, based on the signal to interference ratio, we will actually get to know what would be the cluster size and then.
We decide what is the cluster size now, say for example, let us assume the cluster size north is equal to 7, OK, and then the path loss exponent is equal to 2. Then what will be the signal to interference ratio? Here the signal to interference ratio would be.
Square root of 3 * 7 whole square. The path loss exponent is 2 / 6. Obviously, you can observe that the square root and then square terms will cancel out. So this is nothing but 3 7s are 2121 / 6.
Any comments in the chat box as to what does signal to interference ratio linear value?
What will be this equal to?
Any comments?
21 by 6i think it will be roughly 3 point something.
So it is 3.5, yes, actually it is 3.5. Now what it says is that the signal power, the useful signal power that you are getting is.
3.5.
Or else this is equal to probably I can write this as 3.5 times the interference power. So this is the significance of signal to interference ratio. But the value of signal to interference ratio is decided by.
Not just the next question is, okay, who decides what should be the value of signal to interference ratio? The value of the signal to interference ratio would be decided by some or it is decided by physical layer technology. So what sort of physical layer technology say, for example, you would have.
Heard of some parameters here, say for example, what is called as modulation or probably what is nothing but the coding schemes as well.
Decoding schemes as well. Does it make sense? So this is about know what I intended to discuss about know what you call as know the signal to interference ratio. So this is how it is decided for examination what you have to focus.
Here is 1 the concept of.
You know, concept of...
Cell and cluster.
How to compute the capacity? OK, the relevant data will be given and then also how to calculate the signal to interference ratio. You'll be getting only problems based on this particular concept. Is this clear to everyone?
Any doubts or anything?
Because next I will move on to another topic where which connects. Why is that we had used or else we had considered when we did the frequency division here if you.
Observe here. Why is that lower frequencies was assigned uplink for uplink transmission and then higher frequencies were assigned for downlink transmission?
Any comments?
Is the question clear? Why lower frequencies is used for uplink transmission?
And why?
Higher frequencies are used for downlink transmission. Okay, this is what we will try to answer in the next topic, which is let me create.
Okay, so this is what we are talking answer new topic which we call it as the signal propagation.
OK, signal propagation. Now why we why this particular topic is important? So this topic is mainly about deciding the communication range of the signal.
Communication range of signal.
Okay, so this is the communication range of the signal.
Okay, now we will get into the details of this one. Now, what we expect in this say, for example, if we have 2 vehicles communicating with each other, that is vehicle to vehicle.
OK, so these are 2 vehicles which are communicating with each other, and what is the max distance of communication? OK, so which we call it as R, and it is also important for us to find out what does the received signal.
Power at this particular range because it will decide on multiple factors again, what is the range and then how reliable the link is. Okay, so these aspects will become important when we consider the Mac layer again, we are not getting into the details of.
The analytical derivation, everything we just will get awareness of this particular concept and then how to apply this particular concept. This is what we will be trying to understand. Does it make sense?
Is it clear? Okay. And also using this particular topic will also help you in understanding, okay, I will repeat this one. So using this particular in the signal propagation, we will try to find out how we can actually decide what is the.
Maximum range of communication between the 2 vehicles and also it is important for us to find out what is the received signal power What is the received signal power because based on the received signal power it will decide.
How reliable the link is OK. How reliable the link is OK. Say for example, if this vehicle on the left, if it is transmitting the signal with transmission power PT, then what is the power that it can receive? All this have an impact on what is the stopping distance and then what is the latency.
Each and everything, OK.
OK, now hope this answers your question. OK Dhanush, now and also this will help you in understanding the principle of to certain extends radar also. OK, now to generalise, let us say that here there is.
Transmitter and here there is a receiver. So lets say assume that so this is transmitting with power PT. Now lets say assume that these 2 entities vehicles are separated by a distance R then.
How you can based on what factors the received signal power is decided? Any comments in the chat box now or else to what will happen to the signal power if the distance increases, whether it increases or decreases? Any comments in the chat box?
If the range increases, or say for example, if I increase the range between 2 vehicles, whether the received signal power will increase or decrease.
Net decreases. So basically it is inversely proportion that is one by R. We will see what actually the factor is.
Now what will happen if the transmitter power increases? If I increase the transmission power, whether the received signal power increases or decreases?
It increases so directly proportional to the transmitted power. That is one thing. Then are there any other parameters which we have forgotten to mention over here? Does the received signal power just depends upon the range and or the distance of separation and then transmitted power?
Pathlos will come, will come in the moment, Hari Babu.
Are there any other parameters?
What about the frequency or wavelength?
What about frequency or wavelength? If the frequency increases, whether for the same set of values, whether the received signal power increases or decreases.
See, you have observed this, say for example, in their WiFi link.
For 2.4 gigahertz and then 5 gigahertz the range is larger for 2.4 gigahertz as compared to 5 gigahertz. So I can we can confidently say that yes, as the frequency increases the received signal power also decreases. So if frequency is increasing, what does the effect on the?
Wavelength, whether it is increasing or decreasing.
If frequency increases, wavelength also increases.
Let us write the relation C is equal to FC lambda. OK, now you can C is equal to F lambda or equal to FC lambda or else you can see that lambda is equal to C upon FC. As frequency increases, the wavelength decreases. So.
As wavelength decreases, the received signal power also decreases. Okay, so the way I can write the received signal power for distance separating is equal to. So this is nothing but in the numerator, we will write the terms so which actually contribute.
So one is the transmitted power, next is nothing but the wavelength square lambda square, and this is nothing but R square.
OK, now comment in the chat box. Are we missing any term in the denominator?
Is there anything that we miss in this denominator?
Anything are we missing? Okay.
Here we are missing what is called as 4 Pi whole square.
OK, so this is what it is bandwidth. No, I think these are all good guesses that you have all made. But in this case, it is 4 Pi whole square and this is lambda square. OK, so this is but still we are missing something OK which is in the numerator. So this is the equation. So this is for signal transmitting in.
One way direction OK and of course later on you can similarly relate to the radar where the transmission is 2 way because of which the radar has an propagation loss of one by r to the power 4 because one by r square one direction.
That is from the ego vehicle to the preceding vehicle. It hits the preceding vehicle then again one by R square from the reflected direction that is one by R square into one by R square. It is one by R power 4. So that is why detecting radar signals are quite challenging. Okay now.
Let's bring in the anthana here.
Please note whatever explained, this will also help you in understanding the field of use of radars as well. OK, now in this case, let's assume an antenna where the input power is equal to one watt and the radiated.
Power is also equal to one watt. So this is the ideal scenario. It's an hypothetical scenario. See, in this particular case, the antenna is radiating in all direction.
It is an isotropic radiator. It radiates equally in all direction, just like a football you can take in any angle or whatever it is, you know, it says radiating one watt equally in all directions, that is in all 360 degrees. So that is imagine an football enclosing the antenna.
And it is one watt in all directions. Now, do you see any disadvantage with this where the power is equally radiated in all direction? Imagine you know you are mounting such an antenna on a car like this. Of course, this is not as per the scale wherein it is used for adaptive cruise controller.
Do you see the wastage of power in using such type of an antenna?
I can also write like this, so this is the full circle.
Okay, so here the transmitted power is one watt, that is the input to the antenna is one watt. The radiated power is one watt. You can see here. So this is one watt. So this is one watt. So this is nothing but one watt. Now what we can do is the.
Power which is radiated in the unwanted direction, which I am showing it by the dotted line. To the right side is the wanted power, to the left side is unwanted power, which is actually being radiated towards onto the engine side of the car or towards the banner. So what if I redirect this radiated energy in the required direction?
Okay, in the required direction. So what it is just like flipping or folding here folding a chapathi. So in that case, what will happen is draw this diagram here.
So here one watt of power being radiated equally in all direction. Now what will I do is I will radiate the unwasted power in the required direction that is in the forward direction. Now in both the cases the transmitted.
Power is equal to one watt or is the input part to the antenna is one watt. Here one watt of power is radiated equally in all direction. But what would be the radiated power if you are focusing the power in the forward direction or else you know the direction in which you require? Please comment in the chat box.
What will be the radiated par?
It would be 2 watts because you are just increasing the area. All the one watts of radiated power will be focused on the forward direction. Now note the input to the antenna is nothing but one watt, but the radiated.
Power is equal to 2 watts. So what is the gain?
What does the gain? Yeah.
To the antenna. Please note the antenna is not amplifying. You may ask the question as the antenna is amplifying? No, the antenna is not amplifying, but you are mechanically designing the antenna in such a way that the power in the unwanted direction is focused in the required direction. Tinesh I'll explain this.
Okay, so in this part and also it increases the range as well. Okay, so here you can observe that the first scenario, the antenna is placed exactly at the origin, the input power is also or the input power to the antenna is one watt, the radiated power is also one watt, but that.
Power is radiated in all direction. Basically, this is an hypothetical antenna which does not exist. Okay, hypothetical antenna. But what we do is okay, this is an hypothetical antenna, but in order to ensure the power is not wasted, we alter the mechanical structure of the antenna such that.
The power in the unwanted direction, which you can see here, OK, is refocused in the required direction, that is in the forward direction. When you do that, the 11 + 1, OK, it is not 1 * 1 1 + 1, then what happens is the radiated power would be equal to 2 watts.
So with the same input power of one watt, we are getting a radiated power of 2 watts just by altering the mechanical design of the antenna. So here what is the gain that we have achieved that is equal to 2 clear Dinesh?
Is it clear? OK, now this is what is called as the gain of the antenna. Now can we further improve this gain? Any comments in the chat box?
Can we further improve this particular game?
OK, we can further improve this particular game. What I will do is this one quarter I am just.
Here again, you can see this 1/4 of the image you can make it make it as a cone. I think that is perfect. So what I'll do is here.
This is.
Sphere, then again this is half sphere where the gain is equal to 2, then we will make it a cone.
We will make it a cone nowhere the radiated power would be again 22 + 2 so that is 4 watts. Again this is equal to 2 watts. Again this is equal to one watt for the same transmitted power. Please note I have not changed the transmitted power. So here in this case.
Still, the transmitted power is equal to one watt.
OK, does it make sense? Is it clear?
OK, now has it got something to do with the field of view of the antenna?
What does the What does field of view refer to? The field of view here refers to. Say, for example, for one watt of, see if I place an antenna here, in case of an isotropical antenna I will be receiving the signal, but in case of an antenna which is having...
A coverage of red, I'll not be receiving any signal. Similarly, if I place another antenna here, so I'll be receiving the signal from an antenna which is having red coloured field of view, but not in the green coloured field of view. OK, so this is what actually because you're not in the coverage area, it is not only about the distance.
But this is also about the radiation pattern. Now, can you map these radiation pattern to different applications of radar?
Say for example, I will this is just to encourage you to think that this concept is not just required in this subject, but in other subjects as well. Say for example, if I write adaptive cruise controller, blind spot detection, or blind spot warning. So if you.
To which application or to which colour you can map ACC to, whether it is for red colour or whether it is for green colour? Definitely not blue one.
Please comment in the chat box.
In case of adaptive cruise controller, the say for example, the adaptive cruise controller, it has to monitor only the lane in which it is being driven or to the adjacent lanes also.
It has to monitor only the lane in which it is being driven, so it has to have a narrow field of view. So that is why so this would be having a green coloured pattern in case of blind spot warning and also see for adaptive cruise controllers you would require a large range, okay.
But in case of blind spot warning or blind spot detection, the range is very less, but you have to map or else you will have to observe a large area. So that is where you would be mapping it to green or red pattern. Okay, so this is where you know the field of view will also come into play, okay.
So in the next class, we will just continue our discussion and we will complete this particular topic. So any doubts and understanding, please let me know. I will enable the mics as well. See, Please do not hesitate. If you have any doubts or any concept which is not clarified, I am ready to explain it once again.
Have enabled the mics. Please comment or please let me know if you have any doubts or any queries.

SACHIN MAINI .   1:58:27
Sir, last part, can you please repeat for adaptive cruise control and the other portion?

Shree Prasad M .   1:58:32
in case of adaptive cruise controller, say lets assume this is the vehicle that you are driving or this is the lane in which the vehicle is being driven. So this is the vehicle. So if I represent the radars field of view, which is exactly the cone shape, it has to monitor only the lane in which the vehicle is being driven.
OK, so in that way you need not have to monitor whether there is a vehicle in the adjacent lanes or not. That job is done by the camera. OK, so it has to be very focused. It has to just focus on whether if there is a vehicle ahead of me or not. OK, so that is why for a narrow field of view.
And long range.
We go for narrow field of view where the gain is very high. In fact, you can look at the specification sheet also, OK, but if you go for blind spot detection, in case of blind spot detection, if I were to represent here, OK, so here this is how the field of view should be.
It has to cover a very large area, but the distance is very small, so this would be 2 watts. Does it make sense?

SACHIN MAINI .   1:59:45
Yes, got it.

Shree Prasad M .   1:59:47
Okay, unfortunately, there are no these topics. I am just clubbing from multiple books. I will share the notes in by tomorrow. Just go through it is more than enough. Okay, wherever required, I will be sharing the articles, research articles, just go through that one that is more than enough.
So any other doubts? Are we good to conclude today's class?
Okay then, thanks for attending today's class. I will be uploading the lectures by tomorrow. Sorry, I could not upload it last week, so I will upload. You can just go through that one. Thank you all.

ANSHU KUMAR .   2:00:28
Thank you, sir.

Shree Prasad M .   2:00:30
Yeah.

DINESH KUMAR G P .   2:00:32
Thank you.

Shree Prasad M .   2:00:33
Yes.
Any doubts?

Shree Prasad M . stopped transcription