Automotive Communication Systems (S1-26_AELZG513)-20260808_154224-Meeting Recording

August 8, 2026, 10:12AM

1h 59m 5s

Shree Prasad M .   0:03
Go ahead with today's lecture, so based on previous class requests, I'll enable the mics for everyone, so you can allowing the mics for all the attendees, so please speak or please keep your mic on.
mute if you or speak, you know, please unmute your mic only if you have any questions to be asked. Okay, so we'll start and let me start sharing the screen and also I'll just have a quick recap of.
what we had covered in the previous class as well. Just a quick recap.
OK, so in the previous class, we had this, we had considered.
the channel, the concept of a channel.
And what a channel is made-up of? Essentially, channel is having a centre frequency and then followed by the bandwidth. Bandwidth is the frequency at which the signal which intends to be transmitted is centred around. And bandwidth specifies what is the range of
bandwidth specifies, you know, just like, you know, the region around the centre frequency which is reserved or which kind of packs the signal which is to be transmitted. Okay, so that it can be easily extracted at the receiver. For example, we know in case of the audio signals.
The range of audio signal is from 20 hertz to 20 kilohertz.
ANAND.
The bandwidth in this case, we obtain by subtracting the higher bandwidth value, or else in an upper limit manner, lower limit frequencies. So this is approximately 20 kHz of frequencies or range of frequencies that we transmit. But for FM radio stations,
The bandwidth assigned for transmission is 200 kilowatts, which is almost 10 times, you know, the bandwidth of the audio signal. So, this enables, you know, the clear separation of audio signals that are transmitted by the different stations, and also it can be easily extracted from the...
We had also seen the concept of the cellular networking. In case of cellular networking, we had seen that with our, instead of using a single high power transmitter, instead of using a single high power transmitter, which would cover the entire region with limited number of channels.
We will go for divide and rule policy where the region would be divided into smaller areas and each area would be served by a low power transmitter only with the radio footprint for that particular region. And with this, we could increase the capacity.
of accommodating more number of simultaneous voice conversation. So this is what we had discussed in the previous class. And we had also seen the smaller the cluster size, larger is the capacity, but the issue is the signal to interference ratio also.
Has to be considered into account, because there will be a multiple number of cells which should be using the same set of channels and which can causeway amongst themselves, which we call it as co-channel interference. Increase the distance between the co-channel cells that is only possible by increasing.
The cluster size, so by increasing the cluster size, the distance between the co-channel increases, and then we can control the interference received from the other co-channel cells.
Is my voice breaking once again? I'm not sure. I'm very close to the...
Uhh...
Then cheque your Wi-Fi connexion.
No.
Is voice breaking for everyone?
Bala Ganesh Chandan. Okay, anyways, I think it should be okay now.
I think maybe you'll have to cheque internet at your ends. Okay, so after this, started with the concept of, you know, the large scale fading.
And, in case of, you know, the large scale fading, what is the objective here? In case of large scale fading, say, for example, in case of communication, one of the important parameters is to determine the communication range. If you consider the...
Communication range.
In case of, we all know that there are two different technologies supporting the V2X communication. Either it can be DSRC based, which is IEEE, E22.11P, or else it can be based on cellular V2X as well. So here, the communication range would be approximately...
In 200 metres to 1000 metres range.
Now, who decides, you know, what should be the communication range? And then when we select a particular communication range, what will be the parameters? Like what will be the transmitted power? What will be the gain of the antenna? And then what will be the coverage of the or field of view of the antennas as well? This is...
You have to see, you have to see this parameters in the context of the way you understand the parameters for any perception sensors that is used for either it can be a lidar, radar, camera, or radar. OK, so.
I'll have to see in this particular direction. So, when we consider the communication range, so it is defined in terms of, you know, the minimum power that can be received. So, you know, that is called a sensitivity also, but essentially...

GOWTHAM C .   6:29
You.
Excuse me, sir. Sorry to interrupt, sir. Can we also add ultrasonic sensor also that comes or ultrasonic sensor?

Shree Prasad M .   6:37
Pardon?
Okay, I'll add that one also.

GOWTHAM C .   6:43
Yeah, yeah.

Shree Prasad M .   6:46
Okay, thank you.

GOWTHAM C .   6:47
Yeah, thank you.

Shree Prasad M .   6:51
So, these are the different Radha sensors. Of course, you know the ultrasonic sensors are not actually used for Radha applications because it is only for warning systems. Anyways, never, never mind. OK, so in case of, you know, so what, what in this case, you know, the communication range.
When you calculate, it also depends upon the received signal for specific distances. It does significantly impact, you know, the communication range, this received signal power. So, in case of large scale fading, so if we have two different antennas which is representing...
Two vehicles, so let's assume first vehicle is transmitting with these parameters: that is, PT is the transmitting power, GT is the gain of the transmitting antenna. We have roughly seen what gain refers to in the previous class, and of course you know the wavelength of transmission is lambda, so it...
For us to find out what is the say received signal power such that there will be or else you can decode the message that was transmitted. OK, so we had seen that the received signal.
Depends upon the following parameters directly, so the received signal power increases with increase in the transmitter power, and then the gain of the transmitting and then receiving antennas, and as we increase the wavelength, please note increase in the wavelength means you know the decrease in the...
Carrier frequency, the received signal power also increases, and then followed by 4 pi whole square, and then D square. So, here you will have to clearly observe that the received signal power is indirectly proportional to the distance square, so that is why, in case of...
The radar range equations, I do not know whether you have studied already or else you have been already have in course of radars, so the received signal power will be a factor of 1 by D to the power four, because in forward direction the received signal deteriorate by a factor 1 by D square.
Then, again, in the return direction, after hitting the object, again, it is further attenuated by a factor which is 1 / b square, so this equation.
Also, call back.
A propagation, so does free space propagation, because here we are assuming that, OK, this is also called as free space propagation. Here, we are assuming that there is a clear line of sight between a transmitter and receiver.
OK, so I think I had written these equations in the previous class. OK, I think I have completed this equation, and more or less it is also equally important to understand the significance of the gain of the antennas in case of the gain of.
And ask if the transmit power is 1 watt or sorry if the input power.
Radiated power, this is really important. The radiated power is nothing but PT times GT. So, we are not in case if we say the radiated power is 2 watts, so which means to say that essentially the antenna is providing a gain of factor of 2. So, essentially you are not increasing the transmitted power that is PT or the input power.
It is with the design of the antennas that we are increasing the radiated power, so this is an important equations which you should be remembering, that is the received signal power at some specific distance. We will solve problems based on this, because this is very much essential.
In this, you'll be understanding a couple of parameters which will help us in understanding or reading the data sheets. OK, so I'll just open. OK, let me just give a problem and then we will go ahead with this one.
Okay.
So today's and probably next two to three contact sessions. So this will be mainly based upon the solving these kind of problems or mainly about the signal propagation. And then later on we'll consider the concept of networking and then again we'll consider a few V2X messages as well. So that is the end goal of this course.
No.
Here, so this is the problem that is provided to us: the transmitted power is considered to be as 10 dBm, and the gain of the transmitter and then receiving antennas are specified as 10 dB. Of course, it is specified as 10 dBi.
But let's stick to 10 bdb, then again the distance between maximum distance communication is.
Hundred, OK. Now, if a problem of this of this similar problem is given.
Here, I may not specify the frequency. So the frequency by default is nothing but 5.9 gigahertz because we are assuming the band of operation is ITS, that is Intelligent Transportation System band. So we'll have to make this particular assumption.
Yes, Dhanush, I'll be discussing CHANDAN dcb, ELBIN, everything, so those things will be discussed. OK, so next is followed by this, we'll just solve this particular problem. OK, now...
Any comments in the chat box as to what does this bdb refer to? See, essentially the mistakes done by students in solving the problem is the units conversion. Many will not convert the linear values to decibel values or decibel values to linear values. This is where most of you would be losing marks.
And also, as I've reiterated, in examination, marks will also be given for these steps. OK, so you need not have to worry even if the final answer is wrong. OK, if the procedure is correct, significant amount of marks will be given, or else at least you know 80% of the marks will be given.
Now comment in the chat box, what is DBM? In fact, we'll be seeing this quite often in the data sheets corresponding to V2X communication or else whenever there is transmission power. And also, I don't know how many of you have read the article.
Last week, V to V communication will be mandatory, I think, by 2028 in all vehicles.
This is what you know the government is proposing, so gradually B2B is also entering the Indian market. OK, so please refer to the article it was given in Times of India as well. OK, it would be a legal requirement or whatever it is. OK, now let's go ahead.
So, with this, what is actually the transmitted power in terms of 10 bdb? Let's see this one now. Say, for example, if I have, you know, the power which is given as, say, for example, 10 watts.
Now, how would, see, usually the power values, they are all very small values. They are in the order of, you know, 10 to the power minus 12 or 10 to the power minus 9. So it is always difficult to orally specify these numbers. I cannot say that, look, the power is 0.512 into 0.5 into 10 to the power minus 12 or 0.518 into 10 to the power minus 13.
So, pronouncing these numbers is actually difficult, so that is why all these numbers are expressed in terms of decibels. So, one way of expressing the power is in terms of decibels, which we also call it as dcb watts, so that is 10 log 2 base 10 of...
10 and this is equal to, when you write, so this would be equal to I think what is log to base 10 of 10, it is nothing but one itself. So this is nothing but 10 bdb. In fact, these values are also equal or equally important when you understand the concept of radars also.
OK, now next, what is a dBm? OK, dBm is when the same power, say, for example, so this is the power which is equal to 10 watts. I'll specify, or else I'll express the power in terms of dB milliwatts. Here, it is very simple. All we are doing is we are dividing.
The power by, or we are normalising the power by 1 milliwatt, which is nothing but 10 to the power -3, so this is...
One milliwatt. Now comment in the chat box what will be the answer for this one.
What does?
Ten log to base 10 of 10 / 10 to the power -3.
Can anyone help me with this 10 into log?
So, this is equal to 40 bdb.
I don't know.
This is equal to 40 bdb.
OK, because when you take 10 to the power, see, this can as well be written as 10 log to base 10.
So, when 10 power three is brought up, so that would be equal to 10 to the power four, so this is equal to 40 bdb. So, we are including this dcb term because this is normalised by 10 to the power minus three. Now, this is all good, but how do we...
convert the DBM values back to the linear values. So here for linear value, so this is equal to, let me, I'll have to show at least you know step for this one because this is where most of you would be making mistakes. Say for example, so this is power in bdb.
Is equal to 10 log to base 10 of you know the power in linear value, so log to base 10 of power is equal to power in terms of bdb divided by 10. Now, how to...
Get rid of block to based in term. Any comments in the chat box?
How do we get rid of lock to base 10?
So just take 10 power. When you take 10 power, so this is equal to...
That is taking anti-lock, so this is not dividing by a log to base 10. Again, we will have log to base 10 there. I have to get rid of this particular log to base 10, so this would be equal to 10 to the power power in terms of decibels divided by 10.
Similarly, we'll do it for the...
DBM also, so power is given in terms of DBM, so this is equal to 10 block to base 10 of the power divided by 10 to the power -3. Please note this will be a very useful understanding for your other concepts and other subjects as well. So, again, how to solve for this one?
So, again, you can write this as log to base 10 of power divided by 10 to the power -3, so this is equal to...
Power in bdb M.
This.
Divided by 10.
OK, now again, take 10 for both sides, that would be equal to taking anti-lock, so this is equal to 10 to the power.
Power in terms of bdb.
Divided by...
BALAN.
I'll just explain. OK, now again, we can get the power is equal to 10 to the power -310 to the power in terms of bdb.
Divided by 10. OK, so then what essentially?
What indicate here? See, one is how to convert linear values to decibel values, but usually the specifications either for radars or probably for even in V2X, it is given in terms of dB milliwatts. Now, if it is given in terms of dB milliwatts for transmitted power or...
The receiver power, how to convert back to linear values if you are trying to find out some parameters, say, for example, what is the communication range, or maybe for other computations or calculation purpose. So, here we have seen this equation, so essentially what we do is...
converting decibel values to linear values. So we have log to base 10 of P, that is power in decibels divided by 10, take 10 power on both sides, 10 power and then log to base 10 cancels out. So this is the power which is equal to 10 to the power P decibels divided by 10. Same thing we have done for.
bdb milliwatts as well, OK, but only thing is here the extra parameter is the factor which is divided by 10 to the power -3.
This should answer your question. OK, now we'll solve this particular problem here. The transmitted power is given in terms of this. I think it is specified as.
solution. So the transmitted power is specified as 10 decibels. So we have to convert this to linear values. So how do you convert to linear values? This is the procedure I have said. So directly you can write the transmitted power in terms of linear value. This is equal
To 10 to the power minus 3, 10 to the power 10 bdb divided by 10, so this is equal to 10 to the power -3 and 10 to the power one, or else this is equal to 10 to the power -2 watts, so this is the transmitted.
Power the value that we have got.
Now, here, please note the gain of the transmitter antenna is and gain of the receiver antennas are equal. We are considering to be equal to 10. Again, the gain also is specified or else given in terms of 10 decibels.
Okay.
So, how do you convert again decibels to linear value? Please note that decibels and bdb milliwatts are decibel milliwatts are totally different. So, here again, the equation for getting gain in terms of linear values is...
The first equation here, which you can observe, so that does gain in terms of...
decibels divided by 10, so this is equal to 10. So now we have obtained all the required values, so that is what you call, you know, the, you know, what you call as the power, the transmitted power and then gain. Now the other parameter which we are not
Or, which is yet to be calculated as the wavelength. How do you calculate the wavelength? And, as I have mentioned, if what frequency we will be assuming? We know that the velocity, the relation between the frequency and wavelength is equal to...
fdfde lambda, if not nothing is specified, we are assuming the centre frequency is equal to 5.9 gigahertz. OK, so with this, how we can calculate the value of lambda?
We know the value of lambda is equal to C divided by the carrier frequency ffe. Lambda would be equal to C is the speed at which the electromagnetic waves propagate. Any comments in the chat box as to what will be the speed of light?
The.
Any comments in the chat box?
What is the speed of link or what is the speed at which the electromagnetic waves propagate?
Perfect.
So, this is 3 into 10 to the power 8 metres per second, and then 5.9 gigahertz, so 10 to the power 9. So, if you...
Do the computations will obtain the value of lambda to be equal to 50.85 millimetre, that is 50.85 into 10 to the power -3. Please note this will be the standard value which we will be using.
Throughout this particular course, or throughout the course.
OK, so now we have all the values that we want. So, the way you solve this is first write the equation. So, this is also the procedure that is expected in the examination. We can follow any order, but these are the required steps, so that is...
PTGTGR Lambda square then four Pi whole square and then distance square. So this is equal to the transmitted power is 10 to the power -2 watts.
Gain of the antennas are equal, so I will just write it as 10 square, then lambda square is 50.85 into 10 to the power -3 whole square.
Divided by...
Four pi whole square into.
Red square, so any comments in the chat box? What would be the answer for this?
D is the distance separating the two vehicles.
I am assuming the distance separating the two vehicles is 100 metres, so we are trying to find out what does the received signal power at one of the vehicles.
OK, so I have done the calculations already, so here we obtain the value which is given as, so this is equal to, so let me just find out, so the received signal power.
Is equal to...
1.6374 into 10 to the power minus 9 watts.
Is it clear? Any doubts with this? Please let me know in the chat box.
See if you have two way if you have vehicles which are using.
P2P communication. OK, and if they are using the same radio equipment.
Okay, same radio equipment, which is following some standard. Obviously, the gain of the transmitter and then the gain of the receiver should be equal. That is the logic here.
Say, for example, the gain of your mobile and then the gain of the mobile used by your friend, or you know, when I say gain of the antenna in the mobile should be one and the same.
Okay.
We are not considering any, we are considering an ideal system. For time being, we are not considering any losses, any antenna losses as such. Of course, you know, if I include a parameter L here, so this would include the system losses as well. We are ignoring for that because again, we will be unnecessarily getting into discussions which...
will not add much value into this particular course because we have more topics to consider. Okay. Is this clear, GOWTHAM?
Okay, now how do we express this in terms of VATs? The way you can express this in terms of VATs is...
Signal in terms of...
Uh, see, say, for example, say this is...
The transmitter.
And this is the receiver, another vehicle. We are using the same radio hardware, so that is why both are equal. Dhanush is the same. Don't confuse it with radar where the transmitter and receiver are co-located. OK, so now I'll convert this to decibel.
So, this is 10 into block to base 10 off.
1.6374 into 10 to the power minus 9.
So, if you calculate the answer here, is equal to minus 87.86 bdb watts. We can also obtain in terms of dB milliwatts, so in terms of dB milliwatts, so this is equal to...
Ten lock to base 10 off.
1.6374 into 10 to the power minus 9 divided by 10 to the power minus 3. So, in terms of I have just normalised it by milliwatts, so this is equal to 57.86.
bdb, yeah.
Does it make sense? Is it clear to everyone?
Okay.
Now, so this is this is the way you can get the problem. Of course, you know, I can specify, you know, the max distance of communication here is.
You know, say, for example, so this is equal to 100 metres, and at this the minimum power level should be, say, for example, minus 60 bdb.
And then probably will be asked to find out what is the transmitted part I would provide, you know, the gain of the antennas, so on and so forth. Okay, so this is also one of the procedures, you know, where you can actually find out the other way around also. Now, we have to define, or else we have to get an alternate
equation here. Now, what is that alternate equation? The alternate equation is that it should be in terms of path loss. We will define actually what the path loss is. Before even we define path loss, we have to define, or as you know, the equation here is actually, you know, a tedious, you know, every time you know what, if I write this particular equation,
Every time I'll have to, you know, say, for example, plug in the values of GTGR, we know lambda square is constant, and then V4 pi is also constant, and then D square. So, essentially, in this equation, the only parameter which would change here is the distance. So, can I get an alternate equation which is simple enough?
to calculate the values.
Can I get an alternate equation which is simple enough to calculate? So, that is what we will consider now, an alternative equation. So, now what we will do is...
In this particular entire system models, when we deploy a vehicle to vehicle communication, we know the transmitted power, the gain of the transmitting and receiving antennas are all fixed. There is no change in this. Okay. And also even the wavelength is also fixed. Now what we will do is, as part of field trials,
Will consider a reference distance which is called as D naughty, so this is nothing but reference distance.
This is reference distance, and we know this particular distance a priori. OK, we know this particular distance a priori. What actually is this particular distance? And at this particular distance, we will consider, or else we'll calculate, what is the part that is, we will take an equipment.
And then we will measure what will be the power at this particular reference distance, so this is called as reference power, or else power at the reference distance.
So now, using these two known values, can we simplify the equation here?
So, this is the logic. OK, so this is what we are trying to achieve here. So, now let me just plug in the values here. So, a distance D is equal to D naught, so this I can call it as equation one or two or whatever it is, so then...
We will have transmitter power is the same, there is no change with this gain of the transmitting and receiving antennas are not changing, divided by 4 pi whole square and then D naught square. So this is called as equation one and this is called as equation 2.
Please note both equation one and two.
Are operating in the same system setup. Only thing is, I have assumed a known distance. I have taken the power equipment and then measured the power. So, if you take the ratio of these two equations, that is, the received signal power at some general unknown distance, OK, at some unknown general distance.
divided by, okay, we know what is the reference signal power which is being measured. We just, you know, divide equation one by two. If you divide equation one by two, I think I need not have to write all the terms in the interest of time.
So, I just write only the terms which are totally different, so this is the distance square, and this is nothing but a D naught square.
All the other terms will cancel out. Of course, in this particular scenario, yes, this is what we are remaining with. Now, all the other terms will cancel out. So we have the power at some distance, which we don't know. We are aware of the power at some reference distance. So this is.
One divided by D square, all the other terms will cancel out.
and then 1 divided by D naught square. Now, you know what does denominator of numerator and then denominator of denominator. So, we can write the received signal power at a specific distance is equal to the reference power or distance at power at reference distance which we know it is nothing but
D not divided by T whole square. Now, assuming the other all the other parameters are fixed, that is, PT, GT, and GR, which equation is better? Is this equation better, or you know the first equation is better? Comment in the chat.
Box.
This is better, right? So...
It's an easier one now.
But one thing that we have not specified is how D naught and then D should be related.
How D not and D should be related? That is, is D not greater than D, or is D not less than D?
What should be, how they should be related both?
I.
How they should be related? This is the question. OK, please note that always the value of...
The distance be I think more or less I've written both the same, both are same. In this case, the distance should be greater than the reference distance. So if I depict this pictorially.
So, if I depict this pictorially, please note the distances are in terms of the radial distance, so we know.
The reference distance, D naught, and then at reference distance.
Then I shall explain. OK, so at this particular reference distance D naughty, I know what is the signal power. So the signal power at all at a radial distance D naughty is all equal to P naughty. OK, now using this particular distance we are.
Calculating what would be the power at the reference distance, or at a particular radial distance, which is equal to D, as specified over here.
Okay.
So, this is PR of D, and here we are assuming the fixed parameters that is the transmitter power is fixed, the gain of the transmitter and receiver is fixed, and then the lambda is also fixed. OK, now, Dhanush, the objective of the reference power and then the reference distance is that.
We are trying to simplify our computation, nothing else. Since we know in the entire setup the values of transmitter power, gain, and lambda are fixed, we are trying to get an alternative simpler equation. That's it, nothing else.
Does it make sense?
OK, now say, for example, I'll say that, you know, for the same problem, I'll say that at reference distance D naught is equal to, say, for example, 10 metres. OK, I'll specify that, OK, the measured power value is equal to -10 bdb M.
OK, the reference value or reference power is equal to 10 dcb. Now, determine the received signal power at D is equal to 50 metres. Now, how will you solve this particular problem?
Have I mentioned or else have I specified the gain of the antennas? Have I specified the wavelength? Nothing else specified. All you have to do is, if I consider the solution here, so all I have to do is I have to 1st convert the decibels dBm to dcb or, you know, linear values. Of course, you can directly go for an alternate dBm equation also that we will see in a while.
So, this is equal to 10 to the power -3, and then 10 to the power -10.
Divided by 10. OK, so this is the formula for converting bdb milliwatts to the linear power, so this is equal to 10 to the power -4 watts, or else this is equal to 0.1 milliwatts. Is it clear?
OK, so now we have two known parameters that is D naughty 10 metres reference power 0.1 milliwatts, so we will compute what is the received signal power at distance which is equal to 50 metres.
So, this is 50 metres. Again, we will take help of the previous equation here. So, always write the equation which will fetch your marks, even if you may not show complete computations. So, this is reference power, reference distance divided by D whole square. So, this is equal to...
Point 1 milliwatts, 10 to the power -3 into D naught, D naught is nothing but 10.
OK, and divided by 50 whole square. Now comment in the chat box, what will be the answer for this?
So, this is nothing but 0.1 milliwatts.
Into 11 by 5 whole square, or is this is equal to?
1 divided by 25.
OK, so what will be the answer for this?
So, in calculators, enter the input as...
Point 1 E to the power E minus 3 divided by 25.
So, this is equal to...
Four microwatts.
This is equal to 4 micro watts. Is this answer correct? Any comments in the chat box?
It does 4 microbats. Now, get this in terms of...
Decibels, or bdb milliwatts, so this is equal to 10 into log.
To base 10, please be careful in calculators, don't select natural log. So this is 4 microwatts, 10 to the power minus 6, divided by 10 to the power minus 3. This is equal to...
Yes.
Okay.
And then to log off.
Four east to the power -6 / 1 E -3, so this is equal to...
Minus 23.98 bdb. Are you getting this answer?
Okay, now there is another concept here which I don't want to get into the details of it because I know it will actually take the discussion elsewhere.
But before that, you know, I'll also get, or else you know, have this equation in terms of the bdb milliwatts as well, so this is the reference power D naught divided by D.
OK, now what we will do here is that we will multiply by one by three or 10 to the power -3 or divide by 10 to the power -3 both sides, so this is 10 to the power -3, then divided by 10 to the power -3.
D naught divided by D whole square. OK, just some simple simplifications.
Shivam, VIVEK, you know that distance D naught will change depending upon the problem. I'll try to give a pointer for this, but I'll not get into the details of this. Just give me a moment. OK, after solving this, I'll tell on what value actually it depends upon. OK.
Now take 10 log on both sides. So this is 10 log to base 10 and the received signal power divided by 10 to the power minus 3. Again, you can use log of ab formula, so that is 10 log to base 10 of.
By 10 to the power -3 plus.
Ten log to base 10 of.
D naught by D.
Whole square, so this is what actually we get.
Now, what is this? This is power in terms of dB milliwatts, so this is PR of D in terms of bdb milliwatts.
Then, again, so this is reference power in terms of the bdb milliwatts - just a second.
Okay.
Let me cheque if the audio is...
Is my voice audible? OK, if...
So, we have the reference power. Now, it is also in terms of bdb milliwax. Then, again, it is 10 log to base 10 of, or else you can just take the exponent. This will be 20 log to base 10 of D naught divided by D.
Now, so previously here we had converted the decibel values to linear values and then solve, but here you need not even have to do that also. Just plug in minus 10 here, minus 10 dcb, and we know 20.
Log 2 base 10 of, you know, the reference distance is, you know, what you call as 10, and then we are computing at 50 metres, so if you solve for this...
We should get back, or else you should be getting the same value in terms of decibels, like this, minus twenty-three point.
98 bdb.
OK, it is. This is how you would be getting. OK, hope this is clear to everyone. So, these are all, you know, see, there is nothing much physical significance associated with it. This is just an alternative ways of representing the equation and also the standard ways of representing a number, specifically in terms of bdb milliwatts.
To answer to the question, so what will be the value of the reference distance? See, the reference distance, it varies depending upon the nature of the antenna or else, you know, the physical design of the antenna or dimensions of the antenna. Actually, in fact, an antenna has two regions.
One water, and for those of you who have understood till here, you can.
This is more than enough. Water B is being said is all extra information. If you want, you can take it. So for every antenna, there is.
A radial distance called as the far-field distance, so which I'll call this as far-field distance now.
Two entities can communicate only outside the far-field distance.
So, this is vehicle one and this is vehicle two, so the communication ranges are only outside the, you know, what you call as the far field distance, and this particular far field distance depends upon the aperture or else opening of the antennas. Now, what do you mean by aperture or opening of the antennas?
Say, for example, in case of your dish to your DTVs or dish television, you would have seen, you know, the parabolic antennas which are of parabolic shape, and then there will be a set like this, you know, where the signal is received.
On this one, so this is the receiver. Now, the opening that you can see here, so of the disc television, so this is called as aperture. In fact, in case of, you know, the deep space communication, we would have seen antennas which are having span in terms of hundreds of metres.
So what happens is more the aperture, larger the aperture of the antenna, more is the radio waves collecting capability. It can intercept more number of radio waves, hence the gain is also very high. Now you see that we are relating the gain of the antenna to the aperture or opening of the antenna.
The opening of the antenna is also similar to the bucket which is placed under the tap. Now, if the opening of the bucket, say, for example, if imagine you know keep placing a bucket when it is raining, larger the opening of the bucket, more water can be collected.
very fast. If the opening is small, then very few or very small area is intercepting the water droplets, hence not much water can be collected. Similarly, an antenna which is having a large aperture area can intercept a large number of radio waves and which and hence it
actually increases the gain of the antenna. So that is why for deep space communication or else for operations or for explorations, deep space exploration such as the lunar or probably Mars, we have antennas which are across metres apart or else which are you know metres wide.
OK, so I think this should answer your question. OK, but let's not get into the details of how we calculate, you know, the far field distances and all that will unnecessarily cause more confusions. OK, now this is one way of specifying, you know, the received signal power in terms of, you know, the reference power.
So, the received signal power is equal to the reference power plus 20 log to base 10 of D naught divided by D. So, this is one way of specifying. Similarly, there is one more equation here. So, these are all equations which is required which often you will be seeing. In fact, in radar also you will be seeing.
Which is called as path loss. In fact, this is important here. Also, we will be solving problems based on this. Now, what is path loss? Path loss is defined as a function of D, so it is defined as the ratio of the transmitted power divided by the received signal power.
Now, what is the analogy that I can give here? So, let me first write the equations here. So, here let me start with our basic equation: that is, the received signal power is equal to the transmitted power, the gain of the antennas, lambda square divided by...
Fort.
Whole square, and then D square. Now, we will take the ratio as specified here, which is the transmitted power divided by the received signal power, so which is equal to four pi whole square, and then D square divided by...
the gain of the transmitting and then receiving antennas followed by lambda squares. So this is what we call as path loss. But you should know the definition, the path loss is the transmitted power divided by the received signal power. Now, what is the unit of the path loss?
Dhanush, yes, you can all take these equations directly. Now, what is the unit of the path loss where we have taken the ratio of two powers?
What is the dimensional unit of path loss?
Seriously, it is the decibels. Say, for example, here, please note still this is a linear equation. As per this, it is unit less because you are taking the ratio of the same quantities. OK, you are taking the ratio of the two same quantities. Now, if this being the case, what is the significance of path loss?
Why we have to calculate this?
Now, if you see in case of typical amplifiers, we'll specify the gain of the amplifier. This is V input voltage, so this is V output voltage. The gain of the antenna is defined as, or the gain of the amplifier is defined as, the output voltage by input voltage.
What it says is that...
And the gain is 10, irrespective of what are may be the input voltage value. The output voltage value is amplified by a factor of 10, right? But in this case...
This is the input to the channel that is, you know, where actually the free space it is propagating. So, here this is the transmitted power, and then here this is the received signal power. Here we are computing path loss, which is given as the that is, we are just calculating PT divided by...
PR of D. So, any comments as to what will be the, you know, the significance of this? In the previous case, where we took, you know, the V out by V in, it was specifying by how much amount the output voltage is amplified. So, what will be the path loss here? The path loss here specifies that by how much amount?
Now, the transmitter power will be decreased, so the definition itself is saying path loss. OK, so you have to see under this particular lens or in this perspective. This is just like, say, saying, you know, what is the returns from an investment that is made for a particular?
shares of a company. We can say 10% or 20%. So it would specify that if you have invested 100 rupees, the total profit would be 110 rupees. If it is minus 10, you would have lost by minus 10. So this is a very generic figure. So you can plug in or you can input any amount and then you can get any.
You know, for any amount invested, you can find out what is the return or the loss. Same way for an path law says that, OK, what is the loss that would be suffered? So, if you know what is the transmitted power, of course, you can calculate what is the received power. The same way, if you know what is the received power, you can calculate what is the transmitted power.
OK, now based on this, we will incorporate this definition here, so this is...
You know what you call as the reference power?
And D naughty whole square. Now, in this equation, we will divide both sides by the transmitted power.
Divided by PT, so this is also P naughty divided by PT, and this is D naughty divided by D whole square. So now you can observe that, so this is the path loss at reference distance D naughty, so this is equal to path loss.
Sorry, the right hand side, left hand side is path loss at a distance D.
path loss at a distance D, and this is equal to path loss at a distance D naught, reference distance divided by D whole square. Does it make sense? You are expressing in different ways. Now, this also has some relation to one of the terms which we had mentioned in the earlier
class where N was called as path loss exponent. If you remember, I'm not sure, so this is called as path loss exponent.
So this is where you get this particular, you know, definition for the variable.
Based on this particular equation here, now I can convert this to decibels when you convert this to decibels, so this would be equal to 10 log to base 10. I will not show this one.
So, this is in terms of decibels.
So, path loss in terms of decibels.
OK, then again, reference distance.
In terms of decibels, plus...
20 log to base 10 of D naught divided by D. So this is an important equation. Now, how to generalise this for any environment? So whatever equation that is shown over here, this is valid for free space. That is, if there is no blockage between.
Between the transmitter and receiver, OK, saying it is.
Okay, I think I made a mistake. Yes, it's good that at least, you know, I verified, you know, someone is watching the mistake that I've made. So if you divide this, see, path loss is PT divided by PR, correct? So then can I write this as, so can I take the inverse of this equation? So this is equal to.
PT divided by PR of D, so this is PT divided by P naught. I am just taking the inverse, so this is D divided by D naught whole square. Does it make sense enough? Thanks for pointing this mistake.
So, this will be equal to now path loss at distance D is equal to path loss at reference distance D naught D divided by D naught whole square. OK, actually, in fact, you can see.
The path loss increases with increase in the distance. OK, so this is very much clear from this particular equation. Now, if you take 10 log on both sides, that is, we are just expressing in terms of decibels. So, this is path loss as a function of decibels, so this is equal to path loss at reference distance.
Function of decibels plus, so this is equal to...
I'll clarify this one. So then again, 20 log to base 10 of D divided by D naught. See, obviously, path loss at reference distance is equal to, but this is nothing but the input PT divided by that is received at reference distance. Obviously, there will be signal loss.
Even at reference distance, right? So it will not be the same as what was transmitted. Does it make sense, DINESH?
OK, so this is the equation now.
See.
This is the antenna.
And this is the reference distance, D naught, so this is the transmitted power. Of course, you know, the transmitted power is not same as the power that is received at reference distance. Both are not equal. Now, I am trying to calculate what is the loss suffered at the reference distance.
So, the loss offered at reference distance, it can be calculated at any distance, so this is nothing but what was transmitted and then and what was received.
Correct. DINESH, is this clear?
Should I explain once again?
OK, now this is the equation, so we can generalise this path loss equation, which I'll be doing it later. So, this path loss is equal to...
Path loss at reference distance D naught D divided by D naught to the power N.
Now, this N is called as path class exponent.
This specifies.
How the path loss decreases with the increase in the distance? Please note here, the only variable term here is nothing but D. D not is fixed, the path loss at reference distance is also fixed.
Now, for urban scenarios where there is no line of sight communication, is N is equal to two? That is the question.
Is N is equal to two? Please comment in the chat box.
For urban scenarios where there won't be a visual line of sight communication between transmitter and receiver, do you expect?
Anything.
Or will be N is equal to two? Always remember that for urban scenarios, N will be greater than or equal to two, so the rate at which the path loss, you know, increases is with distances more, so that is N is equal to two, so please note this particular fact. Now, another...
important questions that we should ask here is that say for example, let's assume there is a vehicle here which is trying to transmit its message or you know vehicle to our BSM messages with all the other vehicle in a highway scenario as shown over here.
So maybe there are multiple vehicles. I think I've written the vehicles or the arrangement of vehicles, which is more disciplined, which may not be or which is a rare case in some of the countries. Now.
Here, we want to see how reliable the communication is.
Reliable the communication is.
Now, please comment in the chat box: is the exponent?
Going to affect, or will the path loss exponent affect the reliability of the communication?
Will the path loss exponent affect the reliability of the communication? Now, how many of you have faced problems in speaking with our friends and at some specific zones, or else also when you are trying to pay a merchant via scanning a QR code?
you would experience some network issues. There, you know, if you're scanning, you know, probably, you know, it would say that, you know, the internet connectivity is poor inside the shops, so you'll come outside the shop. Then again, you will try to do something, importing the QR code or whatever it is. So there, the reliability of the communication, it is also affected by the propagation.
Channel.
Okay, it is also affected by the propagation channel where the value of this N would be significantly large. Okay, so if N is large, then the communication range also reduces. If the communication range reduces, then we cannot reliability the VSM.
Packets or BSM messages; we cannot basically transmit basic safety messages. I shouldn't write this message once again over here. OK, so we'll be coming back to this again later, but essentially this is what this is the significance of path loss exponent. Now, how to compute this particular path loss exponent?
So what we do is in urban scenarios.
Matter of plural or whatever it is, we will measure.
I know the received signal power at some distances.
will measure, practically measure, you'll take an equipment and then you'll try to measure, look at this distance, distance D naught, this is the reference distance or this is the reference power or and this is the reference at this distance, this is the power. And then later on, you'll try to use these particular for measured powers and then you'll try to find out.
the path loss exponent. This I'll be doing it one of the classes. Now, sir, can you explain D&D naught distance diagram? I'll do that one. Now, you all guys can take a break of 5 minutes. I'll resume the class at 4. I think it is time is now 4.50. I'll resume the class at 4.55.
In meantime, if you are having any doubts or else if you have to explain any concept, others can unmute and ask your questions or post in chat box as well. Now to the question, explain D and D naught distance diagram. Say for example here, let us consider a vehicle here. So this is the vehicle which is trying to communicate with the other vehicles.
And this is another vehicle here. Maybe the here the reference distance is very less.
OK, so this is the reference distance, D naught, and the radial distance at which the other vehicle is present is nothing but D.
DINESH, is this clear?
So, we will be knowing what is the reference power also at this distance, that is P naught, or else we will be knowing the path laws at this distance as well. So, all we are required to compute is either the received signal power at some arbitrary distance, or we have to compute.
The path loss at this particular distance.
Does it make sense?
So, any doubts, please let me know in the chat box, or else you can unmute and raise your queries as well.
Is it okay, DINESH? Should I scroll up or?
Yeah, I'll upload today, Muthu Krishna. I'll just upload the notes.
DINESH, is this okay? You can confirm.
So anyways, I'll be on mute. If there are any questions, post it on the chat box.
OK, we'll resume the class now if you have, so we'll just continue with this.
So I think we have considered about, you know, the path loss exponent. I think that should be clear. OK, maybe we'll consider one problem and then it will be clear to you.
This is based on the path loss exponent, OK?
So, let's say, assume that...
Beep.
Plus.
Sing, buds.
Yeah, I think let me cheque my audio.
Okay.
It's audible.
So, here the path loss is given as 20 decibels.
And the reference distance again will consider to be maybe this time will consider 5 metres because again it should not be under the impression that is a fixed like the earlier case. So this is 5 metres and we are required to compute what is the path loss.
At a distance D is equal to 150 metres.
And
What is the path loss? Now, we know the solution, or else we are aware of the equation.
Just right.
The path laws are distance D.
Is equal to.
Plus, at the reference distance, not because everything.
If.
What is?
Twenty off, because we are assuming the path loss exponent is equal to two.
Divided by D naughty, if you plug in these values, so this is 20 decibels plus 20 log to base 10 of the distance is 150 metres divided by 5. You can see this entire thing, this will be equal to...
Thirty 150 by three is 30, so can.
Someone can comment, or you know.
Is still my voice breaking or is it for everyone?
I think I need to cheque my audio device or next time I'll switch over to the headphones. That would be better. Okay. So anyways.
Still, is my voice breaking?
Okay, anyways, if it is manageable, next time I'll cheque because there is no connectivity issues. I'll probably there may be some audio issues. I'll cheque that one. Okay. Now we'll calculate this, open the calculators.
So, 20, sorry, so 20 into log of.
150 divided by 5, so that is.
20 decibels plus.
This is 29.54, so this is equal to 49.54 decibel, so this is the path loss. OK, now say, for example, you can repeat this one if I provide, you know, the transmitter.
power as, say for example, minus 10 bdb, we can repeat this particular problem, compute.
For the same distance, the received signal power.
can compute the message signal path. We can just try to do this particular simple computations. Okay. Now just to summarise as to what we have done so far, we have considered the line of sight model. So this is simple ways of specifying. So this is line of sight.
Model.
and in line of sight model.
We have said the received signal power is proportional to 1 / d square.
And we have also obtained the received signal power in terms of the reference distance. This is what is actually more important for us. That is D naught divided by D square. And also we have computed the path loss as well, which is the transmitted.
Power divided by the received signal power. OK, so we have computed these things.
No.
The concept that I discussed now, it is equally valid in other scenarios as well. If there are two vehicles, let's assume two vehicles, there are two antennas here.
Now, one is it is communicating via line of sight communication. Now, do you think this is the only route through which the signal can reach from one vehicle to the another vehicle? Please comment in the chat box.
Please comment in the chat box, because...
Okay, I'll, you know, inform that clicks. Now, is this the only way where the signals can reach from one vehicle to the another vehicle?
Please comment in the chat box.
Okay, see, please note that wired communication.
When we say wired communication, can you think of the context in Automotive vehicle wired communication where there can be wired communication between 2 entities?
any application in Automotive where two entities communicate via a wire. Simple, actually, you know, it's an popular one. It does ECU communication, CHANDAN network or LIN network. So in this, the signals are confined within the wire.
So we call basically this as an guided communication. OK, now.
Here in case of wireless communication, it's not a guided communication, it's an unguided communication. The signal is free to wander anywhere it wants to. That is why we specify field of use. Nobody will say what is the field of view of a CHANDAN network. It's a foolish question to ask because it's a guided communication. The signals are confined only on the wire.
For radars, we say that, OK, so this is the field of view, because that is the view in which we get.
or we can detect the objects. Similarly, the field of view for the V2X antennas says that this is the region in which we can transmit or receive the signals. So that is why this is not the only path along which we can receive the signals, but there are signals which we can hit this particular.
You know, side railings, and then it can reach this particular the another entity as well. Likewise, it can hit the ground and then it can reach the receiver, or else it can hit the other railing or probably any other object and then it can reach this particular receiver. So, here it is, transmitter.
and then receiver. Now, so this is called as multipath propagation.
This is called as multipath propagation. Just a second.
OK, now we will know what is is multi-path propagation advantages or disadvantages? Now, here we were receiving in line of sight one copy of the signal.
One copy of signal. Now, are we receiving one copy or multiple copies of the signal? Please comment in the chat box.
Of course, we are receiving multiple copies, so with multiple copies, well, the strength of the signal increase.
Will the strength of the signal decrease? Please comment in the chat box.
Decrease, okay, one answer is decrease. What about others?
But it's counter to what you are saying, right? We have multiple copies, but you are saying, you know, the signal increases and some are saying it is the same. What may be the reason?
Noise added. See, we are not considering the noise here.
There is no noise. I think you know all are saying, you know, no, see, that's what you know we are trying to analyse, you know, the path loss, whether you know the loss increases or whether the loss decreases. OK, so I'll try to this way to understand the the.
We'll try to understand the situation here. OK, now say, for example, let's assume that...
I'll say for sake of ease of analysis, we'll just consider two-ray propagation that does.
Will consider.
Two-ray propagation. Now, what actually is this particular two-ray propagation? Let us assume that...
This is a vehicle which is having a transmitter which is of height H. Then again, this is the vehicle which is having an height H. Now, what is the approximate height of a particular vehicle, a passenger vehicle? Any comments in the chat box?
What does the high?
Maybe our car or the height of your approximately 5 feet, then I would to be as two metres. Let's assume the communication is between two vehicles which are equal in height, that is, and then HR is equal to two metres. So now there are two.
Propagating directions, one which is along the line of sight path, which you can see here, this I will call it as line of sight path, and then the other path is shown in the red colour, which is nothing but the reflected path, as you can specify, or as you can observe.
And then the distance between the two cars, which are separated by distance D, is indicated by distance D.
Now comment in the chat box when we can, you know, what you call as a.
Um...
when the reflected signal will reach or when the ground reflected signal will reach you know the receiver. So that's probably an important question. This is transmitter and receiver. So when I can say that the red coloured signal which hits the ground will reach the receiver.
Of course, it could have actually gone in a different direction.
But uh, actually...
See, this is here we call as specular angle, specular angle of reflection.
OK, what it means to say is that if you locate the angle of incidence, which is measured with respect to as shown here, and then theta R, theta is the angle of incidence, theta R is the angle of reflection. So, in case of specular angle of reflection, what is the relation between theta I and then theta R?
Any comments in the chat box?
What is the relation between theta and theta R? Both are same, so this is one of the important conditions in case of specular angle of reflection, the angle of incidence and then the angle of reflection must be equal. OK, now...
An important point here is that we have two signals. OK, so this is, you know, what I explained, you know, just pay attention to this one, then it will be much more clear to everyone. OK, now in this particular case, say, for example, here...
We have totally two signals, one that a signal that is received across line of sight, then the other signal which is reflected by the ground reflected path. Okay, now the question that we are asking here is that, are these signals adding up each other or else they are?
subtracting with each other. Comment in the chat box.
How do we find out this one? So essentially some of you have said it can decrease and some of you have said increase or some of you have said you know it would remain the same. But how is it?
Adding, let's try to find out this one. Now, what is the distance?
Travelled by.
The signal along LOS path comment in the chat box.
What is the distance travelled by the signal on the line of sight path?
Yes, both the signals are travelling in the same instant of 10.
The signal is transmitted, one signal is transmitted along line of sight path, the other signal is travelling along. So we know that the distance separating the transmitter and receiver, which is also equal to line of sight communication. Most of you are agreeing. Now comment in the chat box, what does the distance
that is travelled by the ground reflected path that is the red coloured path as shown over here.
So that is the ground reflected path. Any comments in the chat box as to what is this that distance?
Ground reflected path - any comments in the chat box? How do we calculate this one?
See, the main objective here is we are trying to find out what is the difference in distance that each of these signal paths are travelling. That is, we are essentially trying to find out a difference between the ground reflected path and then the line of state.
path. So I think you know I shouldn't have written this delta here. So this is D, ground reflected path, and then the line of sight path. We have to calculate this one. And one of you have said D into two. Let's see how to calculate this one. OK, let me redraw this diagram here.
So, this is...
The transmitter HD, and this is the receiver HR, and the ground reflected path.
is indicated here. Okay, so this is nothing but DG. The way we can calculate this by using the concept of mirroring. That is, we just take the mirror of this particular HR over here, exactly the height, okay, exactly the same height, and then we will...
Extend this particular.
you know path or whatever it is. So this is based upon the geometry which you can try to find out. So of course you can observe here that I'll just indicate the line segment. So this is A, B and then O and then followed this is by B prime. So here you can specify that or else you know.
The length AOB is equal to AOB prime. I think one of you have already calculated this.
Now, how to calculate?
the length AOB, which is essentially the distance of the ground reflected path. Any comments in the chat box?
How to calculate using this particular geometry?
OK, all you have to do is construct an right-handed triangle again.
I'll just take the mirror image here. So this is nothing but HD, same distance. This is HR prime. OK, and you can see this is this distance we know already. So this is nothing but distance D. So the diagram may look cluttered, but essentially this is how.
the concept or else this is how we are actually calculating. Any comments in the chat box as to how do we calculate or how do we get this distance?
How do we calculate? So, you know, the hypotenuse here is nothing but the distance ground reflected path, correct? And then, so I basically I can write here, you can use Pythagoras theorem. I think this is a good answer. So, this is the ground reflected path is either you can write ht plus ht or else I can write this as
H T plus HR. I will write this as H T plus HR. Please note this can be the same whole square plus B square that is LOS square, so that is square root.
LOS or L. Please note here, I could have as well written this to HT's, 2HT also, both are same because she is equal to.
Does it make sense?
VIVEK.
Okay.
Now, with this, we can, of course, I will be solving this with the help of a problem. Now, we can calculate the delta D, or you know the distance difference, which is equal to the ground reflected path minus the LOS path. Now, if you know what is the distance in the difference, what is that information that we can extract?
Is it possible to get the phase difference between the signals, and when I say phase difference?
This is important.
Why phase difference is important? OK, now let me again explain with the help of an example here.
So this is the time access here. Let me draw two ways.
This is 1 sem.
And this is another signal. Now, what is the phase difference between these two signals? So, when we combine these two signals, will this result in amplification of the signal, or else will this result in destruction of the signal? Comment in the chat box.
Why it will result in destruction of the signal?
What do you mean by anti-phase here? What is the phase of the signal here? We say the phase difference is 180 degrees. Correct, we would have all studied this one. But why we say that the phase difference is equal to 1. Please note here that phase indicates the starting point of a waveform. So if I draw a waveform here, one full cycle corresponds to
360 degrees, so this is 0 and this is 360 degrees, and this point corresponds to 180 degrees.
So, again, as you can see here, the signal here is starting at 0 degrees. On the other hand, the signal here is starting at 180 degrees, as based on this particular diagram. So, if you take the phase of this phase difference, that is 180 degrees minus 0 degrees, so this is equal to...
180 degrees, so the signal will cancel out each other. So, essentially, what we are trying to find out here is that the distance will translate in terms of the phase difference. That is what we have to find out. Next class, I will be explaining with the help of some simulation diagrams. There, it will be much more clear.
Now, how to travel or how to translate this distance in terms of phase difference is what we will try to understand here, OK?
Now, again, here, let me draw this particular diagram and one full cycle of a sine signal or a cosine signal. So, we know this is 1 wavelength, or this is in terms of metres, or this is in lambda. DINESH, what should I repeat?
OK, I will repeat this one. See, essentially, here we are trying to identify the phase points of the signal. The phase points of the signal indicate what is the starting point of this particular waveform. So, the initial point is considered to be a 0 degrees, and then, as the wave progresses, we get 180 degrees, and then followed by...
Three 60 degrees, so one full cycle corresponds to a phase of 360 degrees, correct? Now, exactly the halfway point is 180 degrees, so here the first signal is starting from 0 degrees, and then the second signal is starting from 180 degrees, so at least two signals travelling at the same.
Phase values or safe same phase points? No, one is starting at 0 degrees, the other one is starting at, you know, 180 degrees. So, here the phase one, and this is nothing but phase two. So, if you to calculate the phase difference, that is...
you know, at what phase, you know, what is the phase difference, so which is nothing but phi 2 minus phi 1, we will be getting 180 degrees. So they are out of phase. So this is what we call as phase difference. Now, we will be translating this distance difference in terms of the phase difference.
And based on which, we will try to understand whether the resulting signal will add up each other or whether it will cancel out each other. Is this clear, DINESH?
OK, now.
If the signal has travelled a distance of lambda metres, OK, for this particular lambda, what is the phase? So, this is the lambda metres. After going through one full lambda metres, what is the phase change? Any comments in the chat box? You can see this is the starting point. The wave has travelled a distance of lambda metres, so the same point.
So what is the angle in terms of that test travelled? Any comments in the chat box?
It is 360 degrees. It is 360 degrees. So what essentially it says is that one full wavelength travelling by one full wavelength corresponds to 360 degrees. So if you have travelled by delta D, this may sound a little confusing.
It will be clear with the help of simulations and illustrations that would be done in the next class. So this corresponds to what is the phase difference that is delta Phi. So if I travel by a distance D or else if there is a distance difference of delta D, what is the phase value or what is the?
delta phase delta F delta phi. So, I can write this as delta phi is equal to delta D into 360 degrees or else you can also give in terms of 2 pi and then this is equal to lambda. So, this is how we calculate. We will solve a problem.
then this will be much more clear. As of now, things may look a little Greek and Latin, but we will try to solve a problem and then there it will be clear. And in next class, I will be showing the simulations as well. I think that is 2 weeks from now.
Okay.
So, next week, because of independence, Independence Day, it will be, you know, in you know, it will be independent, not attend classes. It will be already be independent, not attending classes. I think the class is scheduled on weekdays. Please make note of that schedule. OK, now we'll come back to the topic.
So, let us consider the scenario here.
There is transmitters.
Receiver.
And the heights are equal to two metres, then again, this is the ground reflected path, and this is the line of sight path, and then the distance between the two entities are 100 metres. Now, find out...
For this geometry, the two signals will add constructively or whether waveform simulations in the next class. OK, so here first we know the line of sight component is equal to 100 metres, which is equal to the distance.
B.
OK, now let us also try to calculate what is the ground distance of the ground reflected path, which is, please always make a habit of writing the formulas: H, HR square, D square, and then square root, so that is half.
OK, now plug in the values, so here the ground reflected path, so DG, this is equal to...
2 + 2.
Metres.
Then, 100 metres square, then followed by square root, so this is essentially equal to 2 + 2 is 4, 4 square is nothing but four square is 16, so this is 16 plus.
Hundred square, 100 square is 110,000, so square root, so comment in the chat box what will be the answer for this.
Comment in the chat box. Please note, you will have to write till the decimal last decimal point. Otherwise, if you approximate, you may think that there is no phase difference. Someone has unmuted. Please, can you mute?

THAMIMUNANSARI M .   1:42:13
Okay.
Okay.
KADAM.

Shree Prasad M .   1:42:27
Okay.
So this distance, how is this value? If you calculate, this is equal to 100.07997 metres. So we'll have to write to this or 5th or 6th decimal place for your answers to be accurate. If you round off, then the answers will be incorrect or whatever you get.
OK, now we will calculate what is the...
Distance difference, so this is equal to the ground reflected path minus the line of sight path, so this is equal to 100.07997 - 100, so this is equal to 0.07.
997 metres. Does it make sense? Is it clear?
As of now, just take this particular concept. Next week class, when we, when I consider simulations, there it will be much more clear, where we will illustrate, you know, the general arrival of the signals and then how they constructively add or how they destructively add. All these things will be explained in the next class.
OK, now for this phase, you know, distance difference, what will be the phase delta Phi? Please comment in the chat box. Delta Phi would be calculated as delta D.
Into 360 degrees, you can also consider it to be as two pi, and then followed by lambda. Now, what is lambda equal to? Any comments in the chat box?
What is lambda equal to? Did we use this value in today's class?
15.35 millimetres, so 55 or how much it is?
Uh...
I think you know you'll have to use, you know, the wavelength or the frequency of ITC frequency, ITS frequency band, 50.85. OK, so use, you know, the proper values.
And this is.
Fifty-point.
Eight 5 millimetres, or this is nothing but 10 to the power -3. So, finally, when you calculate what will be the answer, you can also tell in terms of the angles.
So, this is equal to...
5 degree. So this is the phase difference between the two received signals. Now looking at this, what is that you can comment? Is it constructively adding or is it?
I know what it is. Please comment in the chat box.
How do you say that it is adding? First of all, the angles should be in the range 0 to 2 pi or else it should be in the range 0 to 360 degrees. So we'll have to modulo this one. So here, let's assume 566 degrees minus 360. So this is equal to, or else we can say delta.
is equal to two knots, 206 degrees. Now with 260 degrees phase difference, is it constructive additions or else whether it is close to destruction or else whether it is anti-phase? Please comment in the chat box.
He.
If I consider the relative phase delta phi between 0 degrees to 180 degrees, the signals are getting slowly destructed. It is getting slowly destructed or it is close to anti-phase. Now, as we move towards 180 degrees towards 360 degrees.
Again, the signals are getting constructively added. So, in this case, what will be your comment for the examination? Is this adding constructively or else is it close to destruction? Comment in the chat box. These are the kind of questions that you can expect for the examination.
Is it clear? Should I explain any topic once again? Please let me know in the chat box. This is a compulsory question for your EC2, whether it does makeup or whether it is regular. Definitely, you'll have a question based on this particular topic. Akesh, four to five.
Clear, good to word.
So, in this case, we can say that this is close to out of phase.
Yes, I will explain this out of phase, hence the signal will destruct, or else you know it is destructive addition. OK, let me repeat this particular procedure, because this is this is important in many ways. One will have to understand here is that in the next class I will be explaining with the help of time of arrival of these signals also.
then it will be much more clear. But one thing that you should understand here is that important aspect here is how do you convert the distance difference to the phase difference delta phi. This is more or this is really important, okay, because based upon the phase difference, the signal will add.
Either constructively or it will, you know, or else it will, I cannot say add constructively and destructively, it will either construct addition or else it will be distracting each other. OK, now, so far computing distance difference is much more clear, or else it is clear.
So, where you are subtracting, you know, the ground reflected distance and then the line of sight distance, but the confusion for most of you would be how we are getting this particular formula: delta D is equal to or else delta phi is equal to this is nothing but delta.
D into 360 degrees divided by lambda, so this would be a confusion, OK? And then, see, essentially what we say is, I think we would have studied in your PUC also, right? So, if there is a wave, if you consider one full cycle, OK, the wave actually fluctuates.
right, it fluctuates from minimum value, goes to the maximum value, then again goes to 0, again minimum and then to 0. So this point does 0 degrees and this is nothing but 360 degrees.
Now, here, this is the distance.
So, this is nothing but lambda. OK, so this is, or else I would say that probably no easier way to say is this is 0 metres, this is nothing but lambda metres, correct? I think this will make probably many to understand this particular concept, so this is nothing but 0 metres.
And then, at lambda metres, you know the phase difference, or else the phase is 360 degrees. Now, if I compute, you know...
The phase corresponding to lambda, what I do is I take lambda metres minus 0 metres, the corresponding phase differences, so which is nothing but 360 degrees minus 0 degrees. So, what essentially it says is that lambda metres, or else if the wave has completed one full wavelength.
It means to say that it is corresponding to 360 degrees over here. Similarly, if there is a point A, if it has moved by a distance of lambda, then it says that it has moved, or else it has completed a face of 360 degrees. So, if lambda corresponds to 360 degrees.
Again, this will be simulated in the next class. Then the delta distance D difference is what? So that would translate to phase. Always a wave travelling some finite distance will lead to the phase difference. This will be much more clear when I show simulations in the next class.
So, you will calculate the delta phi is equal to 360 degrees into delta D divided by lambda, and if we get C for us, we all know that based upon our understanding in today's class.
If it is close to if the delta phase, if it is close to 0 degree.
Or it is constant degree it is addition. If it is close to 180 degrees, then the waves are. What the question is this clear?
Okay, now this was all good, but what if if
Two of unequal site are communicating, say, for example, here.
Here is.
A transmitter of height HT, then there is another receiver of height HR. Of course, the distance of separation here is the. Now, can anyone explain what would be the possible vehicles for transmitter and receiver?
OK, Abhishek, in case if the phase difference between the waves are zero and 360 degrees, again, please don't worry. Next week, or else in the next class, there will be a simulation on this one, so it will be much more clear. Then the phase will align, or else you know the signals will align each other properly, so that will lead to...
Addition.
That will lead to addition, in case if the two signals are having a phase difference of 180 degrees, then it is destruction. Please do not worry, we will be considering this in the next class. OK, now I will just, you know, point out to this scenario, and then we can conclude today's class.
So, let's assume previously where the height of transmitter and then height of receiver was equal, but here we are considering an unequal case. Any comments in the chat box? What would be the scenario? Can anyone, you know, map the vehicles here for HT and HF?
Say, for example, this can be a truck.
Which is having probably 4 metres height, OK, and then a car which is of two metre vehicle truck and a car. Now, what will be the line of sight distance, and then the ground reflected path? Comment in the chat box.
Are they equal or are they different or how is it?
How do you calculate?
It will differ, but how do we calculate this?
How do we calculate?
So I can, I'll just give the equations, try to find out. I can find out the line of sight distance as.
The height of the transmitter minus height of the receiver, whole square plus D square, so this is the square root, which is half.
And then, what will be the distance of the ground? So, this is our, you know, the ground reflected path. So, this is again HT plus HR whole square plus D square to the power half.
You have to use basically the same formulas. Now, can you repeat the same, you know, problem? Let's try to calculate, you know, the line of sight path. So, here, so this is equal to 4 - 2 meet whole square plus. Again, let's assume the distance is equal to...
100 metres, then 100 metres whole square, square root, then again the ground reflected path distance, so this is equal to 4 + 2.
Whole square plus 100.
All square and then square root. Can anyone help me with the values?
So, this will be 2 squares, so this is nothing but six square. Try to get the answers for this. We will continue in the next class. So, in the meantime, if you are having any doubts, please let me know in the chat box.
Please don't hesitate. Don't think that the doubts that you ask is silly. You can please feel comfortable to ask the questions in case if you are, you know, if you feel hesitant to ask in the class, you can call and get it clarified as well. So if HT is small and then HR is larger than.
Exactly, you know, but it doesn't matter since that difference and then it does squared, it doesn't matter, so this probably I would rather indicate this here by a tilde symbol which indicates the difference, OK?
So any other queries or comments?
Okay, then we'll meet in the next class. Next class, I'll be getting the simulations and all, so you need not have to worry. And followed by this, we'll get into the diffraction and there are a few topics which I'll try to present it in a different way so that, you know, we can...
Spend more time on CHANDAN networks, and then some IP as well, of course, on the V2X messages as well. OK, so please let me know if there are any doubts, or else we'll conclude today's class.
OK, and I'll, I'll, I think in notes, when I upload the notes, everything is already there. I'll try to upload, you know, the summary slides as well. That should not be a problem.
OK, then we'll meet next week. Thanks for attending today's class. Thank you.

Shree Prasad M . stopped transcription