Automotive Communication Systems (S1-26_AELZG513)-20260829_154115-Meeting Recording

August 29, 2026, 10:11AM

1h 59m 4s

Shree Prasad M .   0:03
So, we have the networking in today's class onwards, and probably again we'll come back to the topic of, or probably you know, the V2X communication after completing this one. Okay, so I'll enable mics for everyone, so in case if you are having any doubts, you can...
Let me know in the or else you can unmute and then you can raise your questions as well.
But make sure that you know you're on.
Mute if you don't have any questions, because it may cause disturbances as well. OK, now before I begin with the topic of modulation and the previous classes, and all you know I was mentioning about, you know, the ways which may add construct.
or destructively. Once you know I was referring to the frequency and then again I was measuring to the wavelength and time period. So I'll just show one simulation which is about the concept of a travelling way and you can explore if you are if you are interested in this particular topic.
Of course, you know, it's not required for their examination. This is only for your general understanding purpose as to how 3D waves propagate in the environment. So now what I've done is I've actually considered a travelling wave. At layman's language, what does this
A.
Travelling wave.
but a wave which actually travels along the space. Now, you may be wondering how that is possible. That's one of the reasons as to why you would be receiving the signals from FM radio stations to your radio receivers, or probably that is the reason as to why you would be receiving the signals from.
mobile base stations to your phones as well. So that's one of the basic reasons that you should keep in your mind. Now again here, as you can see, one I've shown, this is the equation you need not have to worry about the equation part. I know that the simulation is not as smooth, but I'll share this particular HTML file.
We can just go through this in the later classes, because I think there will be certainly some lag. So as we can observe in this case, you know, here Z is indicating the direction of propagation. In fact, even your radar waves, you know, propagate, radar signals propagate as shown in this particular.
diagram. So in this case, you know, you can observe one Z is along the direction of propagation, and it is shown in terms of the wavelength that is lambda, and at 5.9 gigahertz, which is the wavelength at which the ITS band operates, or where the B2X messages are exchanged, it is 50.85.
Millimeter, and then of course, you know, if it is 5.9 gigahertz, you can also calculate what will be what will be in the time period as well. If so, I've shown the signals for one lambda and then one time duration as well. Now, this the the most point of where...
where it is placed, that is nothing but the origin. It's a three-dimensional plot where I'm sliding the mouse, it is indicating the Z axis. I think you should be able to see this. And on the other side, which is where the axis is going into the board, it is actually indicating.
The time axis, I can observe that at this particular point, which essentially indicates the point where, see, this is the point where chase that, you know, Z is equal to 0, and then the time instant is equal to 0, or this is nothing but the origin. So, this actually refers to...
know what equal as, you know, the exact location of the transmitter and then how the signal is actually varying. We can see that the signal is not only varying with respect to, it is varying actually with respect to time also and then the signal is actually propagating as well. And if you fix a particular distance here,
Okay, you can see the signal is actually moving along the Z axis and also the signal is varying with respect to the time axis T is equal to 0. So this is actually the concept of, you know, the travelling wave. You know, you, when we say, you know, the signal phase, you know, add up, you know, constructively or destructively.
We are considering we are freezing the time instant for a particular time instant, because essentially if there is no relative velocity between or if the if the distance between the two vehicles are not increasing or decreasing, then essentially now you are freezing the time and then you are ensuring or else you are seeing whether the signals are adding.
constructively or restructively. This is not so important. This is just for your understanding purpose. At least you should, when you are getting into the details of Automotive radar, or probably, you know, the way to ex communication, always having a knowledge of what a travelling wave is, how to at least visualise the travelling wave is.
very much essential. With this topic, we will move on to the contents from where we had, we'll start from the contents from the previous class. So today we'll have to consider the modulation techniques and then what is its impact on the
You know, what is the impact on the distance and then the relative velocity between the vehicles, which will be seeing or which will be considering in the later classes. OK, let me switch out to the system and then start with the topic. Just give me a minute.
So I'll be sharing these particular slides or this particular animation. It can just go through this one. In case if you have not understood anything, it's fine also. It doesn't matter. Okay. So let me just stop sharing the screen. Just give me a minute.
And switch off.
Okay, all of you should see this screen here.
Today, we'll just get into the details of, you know, the modulation, and then we'll see how the digital signals are represented, so which will help you all in understanding the concept of baud rate and the other topics and all. OK, so...
Yes, I'll be sharing them. So I've just done the annotations. I'm just converting the handwritten nodes to the annotated ones.
So in the previous class, let me just open that slide. Okay, this is fine. So in the previous class, we had started with the concept of the modulation technique, and we had considered what modulation is all about, shifting the signal to an higher frequency.
region. So in fact, you know, to start with, we had said, you know, we'll be considering the carrier signal to be equal to AC cos of 2 pi fecfdbd. So this is basically the carrier signal or the frequency that will be setting on your radio receivers or the frequency that would be allocated by.
the base station to the mobile phones. And here, the carrier frequency here, usually, at least in case of ITS frequency band, so this is equal to 5.9 gigahertz. And likewise, we will have what is called as, you know, the baseband signal, or it is also called as the...
Message signal, so which will have the maximum frequency content, which is FM, and there is always this condition where the carrier frequency is very much greater than FM, or the maximum frequency content of the signal. I've given an example also for audio signals, the maximum frequency is...
20 kilohertz and in case of...
If you consider vehicle to, or else you know, if you consider FM radio stations, so that is in the order of 90 megahertz, so you can observe that the carrier frequency is very much greater than the message frequency, or the message is also called as, you know, the baseband signal. Okay, so I'm gradually introducing new terms so that it will be easier to understand.
So one of the first modulation schemes that got seen was the amplitude modulation scheme. So this is more than sufficient for us to understand some special type of signals, which we'll be considering later. Because as one of the key points to be highlighted here is that we would be not transmitting the raw.
data bits, that is, you know, the vehicle kinematics data as it is. So it will be appended with some extra signals before being transmission and then this can actually add to the latency of the reception as well. Now for the sake of simplicity, we had considered the message signal to be of.
this, you know, single tone. This is just for the sake of understanding. And then, so finally, when you multiply M of T into C of T, so this is the simplest modulation scheme that you can think of. And this is where there will be a product of 2 cosine terms. The product of 2 cosine terms has to be converted to sum of
two cosines and then you can observe it will result in two frequencies which is nothing but FC plus FM and the other one is nothing but FC minus FM. So this is the basic logic. Now the entire discussion from this slide onwards is based only on cos of A plus B formula.
All you have to do is.
All you have to do is, you know, what you call as convert the multiplication into, you know, what you call as sum of these two signals. Just give me a minute.
Sorry.
So, we had also seen the corresponding waveforms. So, here we, if we consider a message signal of frequency, FM is equal to 1 kHz and then FC is equal to 10 kHz, we can see the message signal and then the carrier signal, and then upon...
Multiplying, you can see the envelope that is, if you connect all the peaks of these waveforms, so that is called as the envelope. You can see the envelope of the carrier signal varies in accordance with the message signal, so that is, you have changed the characteristics of the carrier signal. Now, what is the best way to understand?
The best way to understand is looking at the frequency domain. So as mentioned in the previous class, we know that the frequency of the message signal is 1 kHz. That is what we have assumed. Note the scale of the frequency access is given in terms of kilohertz. Likewise, the carrier signal is a frequency.
10 kilohertz, so I'm just indicating a peak or I'm just indicating.
an arrow or placing an arrow to indicate that there is only that particular frequency component. And finally, when you multiply, we had seen that there are two frequency components, FC plus FM, and the other one is nothing but FC minus FM. OK, so you can see that for here. So this is 10 kilohertz.
plus one, so that is 11 kHz, and then followed by FC minus FM, so that is 10 minus 1, so that would be equal to 9 kHz. So essentially it says that don't worry about, you know, the negative frequencies. Negative frequencies are present only for the sake of mathematical representation.
But we can observe here that the signal, which is of 1 kHz, it is translated around the carrier frequency, which is nothing but fecfdbd. So here we can also calculate what is the bandwidth. That is, only if you transmit these two frequency components. Of course, you know the negative as well. Only then we can record back the message signal.
This is also the concept of the bandwidth, so if you consider the total width of the frequency region, you can see that 11 - 9, so this is equal to two kHz, so 2 kHz is the bandwidth, or else you can also say that for single tone signal it is nothing but twice FM, so this is nothing but bandwidth.
So, these are the essential components which are to be transmitted in order for recovery of the signal.
No, this is where I had concluded in the previous class and today we will look at the concept of demodulation. Please note, you need not have to understand each and everything. We'll have to be aware of two important aspects over here, which I would like to highlight, which would be overcome by transmission of some special signals.
can observe that there are two vehicles which are separated by distance D. So if two vehicles are separated by distance D, what is the impact on the demodulation? Whether will we receive the exact signal? Or else, in case if the vehicle is having some relative velocity V,
In that scenario, also, what is the impact of what is the impact on the demodulation? So, this is where we would be transmitting along with the actual signal, some reference signals, in order to obtain the information about the distance T that separates the two vehicles and also the, you know...
the relative velocity also. So these are all required. In fact, if you are adding more number of or if you are adding reference signals quite often, in fact, this can actually impact or this can have a significant impact on the latency of the required message signal because we are adding.
more or else you are transmitting more overhead signals as compared to the actual signals that are required. So this, if latency is more, then this will significantly impact the time gap that can be maintained between the two vehicles. So everything is connected. If you think this topic is irrelevant, that may not be the case. So let's see the.
demodulation under ideal scenario, then we'll consider the demodulation under the actual or what would happen in case if there is no, when you know under other scenarios. Okay, now I think there is a mistake in the slides which I'll correct. So let's consider that you have received the signal.
or the modulated signal as it is at the receiver. So this is the block diagram at the receiver.
Now, what is the what actually we do? Essentially, the received signal or the modulated signal is multiplied with the actual replica of the carrier signal, which means to say that the carrier signal is the same carrier signal is used at the transmitter, and then the same signal is...
Used at the receiver, it is like, you know, as though there is an invisible wire which is supplying the carrier signal to transmitter and receiver. Let's see, you know, how, you know, why and how and all later.
So essentially, we will see two different signals here. One, the output of the multiplier, and then the output of the integrator, or which is also nothing but a low pass filter, which essentially is actually a replica, or probably it is trying to estimate the signal. So as you can see here, so this is the signal, S of T.
Here, and then the carrier signal, which is used the same version which was used at the transmitter, so you can observe. So, this is the carrier signal, or sorry, this is the modulated signal, as you can observe here, S of T, which was earlier also the product of M of T times C of T.
Next, again, here we have, you know, this term is coming from the carrier signal at the receiver. You may be, or else most of you would be having questions as to how this is being done, or else how you can say that we can use or why is that we are multiplying it by the carrier signal even at the receiver also.
That will be clear in the moment. In fact, this technique is called as coherent demodulation.
This is called as the coherent demodulation. So, finally, after simplification, you can see that, so this is a C square AM cos 2 pi FMT followed by cos square 2 pi fecfdbd. Please note, when you see multiple multiples of you know costs or multiplication of cost terms.
The primary aim here would be to convert that to sum of cosines or sum of sinusoids, so we all know what is the formula for cos square. OK, so this cos square theta or cos square of X is nothing but half one plus cos 2X, so this is what you would be saying, and finally, if you simplify...
You'll obtain AC AM by two cost 2 Pi FMT, then again AC square AM cost 2 Pi FM and then cost 2 Pi FCT. See, I'll leave the math simplification for, you know, for you to go to. OK, that is not our focus in this particular course. So finally, upon simplification.
So you can see, you can see the following three terms in this. Now please comment in the chat box, what is the required signal here and what is the signal which is not required.
This is the simplifications that we get. Please note here, essentially all the multiplication of cosines are converted into sum of cosines, which is essentially by using simple your PUC math formulas. So now what does we intend to recover? We intend to recover the message signal M of 2.
or the audio signal, let's assume that you have pressed the piano button and then that would be represented by a single tone and which is of some amplitude AM, okay, and then cost 2 pi FMT. So here the amplitude is similar to the volume which we can provide gain and then we can increase its volume.
But essentially we cannot lose the information about the frequency content FM. And this is what we are primarily trying to recover. We want to single out this term cost of FM. Now comments in the chat box, which of these terms have to be remote and what are their particular frequencies?
So there are three terms, T1, T2, and T3. So what are their frequencies? As you can observe, the term T1 has a frequency which is equal to FM cos 2 P FMT. Then next, this is having a frequency which is 2 FC minus FM.
Next, this is also having a frequency which is equal to two ffe plus f. Now, if you want to visualise the output of the multiplier, this is how the output of the multiplier appears. You can observe, OK, this is the time domain multiplication point wise. I can observe here that...
Let's focus on only on the positive frequencies. Let's not worry about, you know, the negative frequencies. Anyways, we are not doing any math over here. So you can, as you can see here, we have the frequency FM, which is equal to 1 kilohertz, and that is what we intend to recover.
And then we have the other frequency components which are positioned at 19 and then 21 kilohertz. Now, how did we obtain this? This is obtained by the other components which is observed here, so which is 2 fecfdbd minus FM, and then the other frequency component here is...
2FC plus FM.
So these are the frequency components here. Now comment in the chat box how we can, what are the wanted and unwanted frequency terms. At least this should be clear based on our explanation. Please note the ultimate goal here is to get back M of T
and which is nothing but the frequency content reference. Any comments in the chat box?
FM is wanted, so you'll have to remove all the other terms over here. OK, now how to remove all these terms? Any comments in the chat box?
How to get rid of the...
These high frequency terms, any idea of the hardware component which is used for filtering out these components?
Use a low pass filter, or else use a philtre which will remove all these particular terms. OK, so we can just get rid of all these terms. So, basically, you are using a low pass filter. OK, so this is what, you know, essentially we are doing to remove high frequency, you know.
See, you are using exactly here and low pass philtre like this. So you'll just philtre out and then remove the high frequency terms. You are just letting FM to pass through. So essentially what it means to say is that we are getting rid of these particular terms, T2, and then this is nothing but T3. And essentially this is the...
Output of the low pass philtre that I have indicated over here, the one in the red gets filtered out, and then the one in the green is retained. Now, we can see that this is directly proportional to M of T. Of course, this is multiplied by AC square M divided by two. Now, where you would be bringing in the concept of...
Free space propagation - any comments in the chat box?
where you would be bringing in this free space of propagation over here. Say for example, if I consider the received signal power.
for two vehicles separated by distance, an R, where you would be bringing, you know, this R or you know, this term here. Now, this particular amplitude factor here would be directly proportional to the received signal. In this scenario, we have just ignored this particular propagating distance.
I hope this should clear your understanding, or else it should be clear to you.
Now, what actually is the problem? You know, see, this technique is actually called as coherent demodulation, where the exact carrier signals are used if, you know, both at the transmitter and receiver. Now, this exact word is, you know, quite misleading when...
when you know it is quite misleading. When I say exact, you might say that you know exact amplitude, exact frequency, or it is exact phase. Now what it means to say is that the same carrier signal which is increasing or the same trend it is showing, the same has to be used at the receiver also. Even if there is a slight change, that will actually cause a problem.
OK, the same carrier signal. If I consider the sum, say, for example, time instant T, which is equal to, for example, here 1 millisecond, we know that the one millisecond the carrier is taking a peak value. Same trend has to be followed at the demodulator also. That is the meaning of.
the coherent demodulation, the trend has to be followed. It is not only exact replica. The exact replica is quite misleading. It might see that, okay, I've used the same amplitude, exactly the same amplitude, I've used exactly the same frequency, I've used exactly the same phase, but even then, you cannot say it's an exact replica. The trend has to be.
followed. Okay, now what actually is the problem with this? Now say for example, this is again the derivations are not important as such here. It is more important for you to understand, you know, why there are certain special signals that are used in the frame of the DSRC.
are probably within the CV2X also, which we'll be considering. Now, do you think when we transmit the signal S of T, let's consider the scenario here.
Of that of two vehicles, which are separated by distance D, let's assume for time being the relative velocity is equal to 0. Now, if relative velocity is equal to 0, and if South of T is travelling some distance D, do you think this particular signal will reach?
at 0th instant at the receiver. So this is the transmitter vehicle and this is the receiver vehicle. Do you think it will reach at this at 0th time instant itself or else it will take some time to reach? In fact, we have discussed earlier also.
Do you think it will reach instantly or else it will reach after some time?
Or else will there be some time delay?
And if so, what is that time delay equal to?
Okay, so in this case, the signal S of T, when it is transmitted at 0th instant, it will not reach immediately, but rather it will reach after some delay, which is called as tau. And then how do you calculate that particular delay? We all know that it is calculated by the distance of separation divided by C naught.
Which means to say that if two vehicles are separated by a distance of 300 meters, then assuming the value of propagation velocity is 3 into 10 power 8 metres per second, the incurred delay or the delay for the signal to reach is 1000 nanoseconds, or it is equal to...
One microseconds, so that delay is represented as south of T minus tau. So, this is the mathematical way of representing. We need not have to worry as to how we got this. In fact, I would have explained in the previous class also, but that is not required. What this you have to read this mathematically, it says that the signal is delayed or it is received after.
Tau seconds with comparison to the actual signal, which is south of T, so we will have to read this particular equation. OK, now, OK, so you may ask, what is the big deal? Say, for example, if I consider south of T, which we had considered in the earlier case for demodulation, and south of T minus tau.
which is shown in this particular diagram. Do you see any difference between these two diagrams?
or these two waveforms. Please note that the time is in terms of, you know, the milliseconds or time period is in terms of milliseconds, but the delay is in terms of 1 microseconds. Do you think will this have any impact on this one? I think, you know, the answer is already clear. It is actually the phase delay. In fact, you know, that is true. So.
You can see here that I've just magnified both these waveforms for from zero to 1000 seconds. You can see this particular delay is highlighted over here. OK, so you can see the signal here. It is starting at some other time instant, or else T is equal to 1 microseconds. Now, the delay always...
corresponds to the face layer. That is our problem. OK, that would be the problem. Let's see, you know, what or what this will have an impact on this one. In this, I'll have to write one equation. Again, we need not have to remember this particular equation. Only the physical significance is more important. Say, for example, when I write.
S of T is equal to C of T into M of T. OK, so I'll be giving explanation based upon money content. Yes, there will be an amplitude loss here, so that is where I said earlier the amplitude loss would account due to the free space.
propagation loss, or else it may be because of reflection and free space propagation loss and diffraction also, but that we are not considering. See, in this analysis, we consider one problem at a time. If you consider, or else if you try to address all problems at a time, then you know probably will not be understanding anything.
It's like, you know, countering one enemy at a time. First, we will consider what is the, you know, effect due to free space, then what is the effect due to diffraction or reflection, then what is the effect due to noise, then consider all these particular effects together, then also the effect due to delay.
Consider one at a time, and then you say that, OK, so this is effect is all total effect is sum of all the effects. I hope this clarifies your answer. Now, next, we are interested in...
Getting S off.
minus top.
Airs off.
T minus tau. So how do you get this S of T minus tau, the delayed version of this? So I'll write this in terms of, so this is nothing but C of.
T minus tau and M of T minus tau, which means to say that essentially the product is also getting delayed. Now, let's try to understand this delay relative to the time period of the signal. Now, what is the time period of the carrier signal, and then what is the time period of?
the message signal. Any comments in the chat box?
See, you know, 1 kilohertz means what FM? I think I should have taken even better. See, time period of this is nothing but one millisecond. OK, then one by 10 kilohertz, it does, you know, the time period here it is equal to.1 millisecond, so...
One millisecond miss, so this is equal to 1000 microseconds and 0.1 milliseconds, so this refers to 100 microseconds. Now, if you compare this with the delay tau, in which of these terms we can actually neglect the delay? Any comments in the chat box, because we are taking...
Tau is equal to 1 microseconds.
In which of this, essentially we can ignore the tau value or else you know the delay value. Any comments in the chat box? We know the time period of the signal TC is equal to 100 microseconds and then the time period of the message signal is equal to 1000 microseconds. It essentially shares that I think it has rightly mentioned in the chat box.
We can ignore the time delay that is for the message signal. This is always true. This is always true. But the same thing cannot be considered for the carrier signal because the time period of the carrier signal and then the time delay is almost comparable. Essentially, I can write a song.
T minus tau, this is approximately equal to M of T, so we are not seeing any change in that particular delay, followed by C of T minus tau. So, essentially, please note here when I write the south of T minus tau here, so this is equal to...
M of T and then C of T minus tau. Now, let's compare this with the earlier diagram. In the earlier diagram, we knew that so this is M of T into C of T. We observe that both the C of T's are exactly matching over here. The carrier what was transmitted.
is being multiplied with the same carrier at the receiver.
essentially the concept of core and demodulation. But now do you see the same or do you observe the same thing here?
Are they different versions or else are the same versions of this one? Comment in the chat box.
Are they same or are they different? And then do you expect some trouble to be caused because of this particular tau term over here?
Okay, now I think Manmohan Singh, you know, had earlier mentioned that this will actually cause phase delay. In fact, this is true. We will see how it is. Now, if we have C of T signal, we would be writing this as AC cos of 2 pi FCT. So this is the signal.
Now, if you write C of T minus tau, OK, so just replace T by tau, T minus tau, so this is equal to aec cos of two pi FCT minus tau.
And if you expand this, so this is equal to...
A/C cause of.
Go bye.
FCT minus 2 pi FC tau. This is an important parameter not only for the reference signals but for your examination also. Now what do you think is this?
What do you think this particular term can be called as? Any comments in the chat box? See here, this is FC into T, which varies with respect to time. Assume that the distance between the two vehicles is fixed. If it is fixed, then the time delay is also fixed. Now, what is this particular term? Any comments in the chat box? First.
It is fixed, or it is a variable term.
First, it is a fixed or a variable term.
It is a fixed term. So what name can we give this to this particular term?
Yes, it is fixed.
Not time constant. You can see 2 pi FC tau. You may have C2 pi. What is a unit of 2 pi? 2 pi is radians and then FC is Hertz and then tau is nothing but time. So both, don't you think both will get cancelled? So what will be the time unit of this?
It is nothing but radiance or else it is nothing but phase. Perfect. So this is nothing but phase. So essentially I can write the signal here. So this is equal to AC cos of 2 pi.
fdfde plus phi. So now do you see the phase difference between C of T, what was used in the transmitter, and then C of T minus tau? Is there a phase difference? If there is a phase difference, what does that phase difference equal to?
What is that phase difference equal to?
The phase difference is equal to phi. This is what is important for us. And phi, this is equal to minus 2 pi ffe top. So this is actually the culprit over here. So this, finally, if you go through all these particular multiplications, finally,
I'll not get into the details of this map. This is not required because, you know, I would rather spend this time on explaining the topics of software defined vehicles and in vehicular networking. So this is the term that would be getting. Now, what is the additional term that you have got as compared to the previous case?
In the previous scenario, you had observed that it was just a square AM by two cos 2 pi FMT, but you can observe here that it is scaled by a factor which is called as cos of phi. OK, so this is the only difference, because this results from the relative phase difference between...
What was what was what we are receiving, and then we are assuming that C of T? OK, now this is phi, and then what is the value of phi? So, this is nothing but two pi ffe tau. OK, now is there a solution for this problem?
One is not discussing in this particular course, but that will not help us in countering. That will help you only in getting maybe getting more marks, but it will not help you in understanding the topic. So how to counter this particular problem or else how to obtain information.
About.
ffe.
Aganti.
Receiver, any comments in the chat box?
how to obtain information about the phi or else this extra phase at the receiver or else how we can adjust the phase at the receiver such that we can get rid of the scaling factor. Any comments in the chat box?
This is done by transmission of the special signals.
These special signals are also known signals, so that is before the actual transmission of the data. Before the actual transmission of the data, we transmit these special signals, get to know what is the value of phi, and then adjust accordingly. So these special signals has to be transmitted.
within certain intervals. The more you travel, the more you transmit these reference signals, it will actually add to the latency of the actual signal that we are supposed to receive. So you cannot get rid of these special signals because special signals will help us in proper demodulation of the signals. But if you are transmitting more often, yes, it will help us in
improving the reliability of the link, but the question is, it will, you know, reduce the, or else it will increase the latency. So these are all, you know, pointing in different directions. Does it make sense? Is it clear? Should I explain this topic once again? Say for the examination, all you have to remember is.
And let me highlight this one.
See, all you have to remember is...
OK, so this equation, and then followed by this equation, and then highlight the value of phase five over here, so that should be more than enough.
Clear, any dots or anything?
Okay, if there are no doubts, I'll just move on to the another concept here. So that is a small concept and then we'll move on to the in-vehicle networking. So now, do you think the relative velocity between the vehicle will be always equal to 0? So this is the question mark.
Now, we have earlier we had considered some finite distance. Now, we are considering some relative velocity. We will focus on the relative velocity. So, comment in the chat box: Do you think you know the relative velocity will be always equal to 0 when two vehicles are communicating, or probably when a vehicle is communicating to an infrastructure?
Or else probably when vehicle is communicating with the network as well.
Is my voice audible or how is it? I see portal silence in the chat box.

CHANDAN KUMAR PAUL .   42:35
Set not equal to 0, always.

Shree Prasad M .   42:39
Yes, it is. It is not equal to 0. Say, for example, one vehicle may be travelling at 30 kilometres per hour, then other vehicle is travelling at 40 kilometres per hour, then the relative velocity is equal to 10 kilometres per hour. OK, now if this is the case.
If I transmit the signal at FC Hertz, do you think the signal is received exactly at FC Hertz? Please comment in the chat box. With relative velocity being not equal to 0, if I transmit the signal exactly at FC Hertz, do you think the signal is also received exactly at FC Hertz?
This is the question mark at the receiver. Now earlier we had focused on the phase, which arises because of some finite distance. Now we are focusing on the carrier frequency. Please note that I think this answer is now mentioned in the chat box. At the transmitter, if the carrier frequency is ebcf,
Then, at the receiver, OK, the carrier frequency will be slightly offset, which is called as plus or minus fdfde. Now, what is this fdfde called as? Any comments in the chat box?
There is some slight shift around the carrier frequency.

CHANDAN KUMAR PAUL .   43:53
It restoration frequency.

Shree Prasad M .   43:56
No, this is actually called as Doppler frequency.
This is called as Doppler shift.
or this is also called as Doppler frequency. Now, probably I'll encourage you to try to find out how to calculate this particular Doppler frequency. Okay, you can try to find out this one. In fact, this is the main culprit in high-speed communications. So this is the reason why you cannot...
communicate or you can see communicating or downloading YouTube or watching YouTube videos or high definition videos when you're travelling on train are probably at very high speeds is actually difficult. It is because of these reason. Okay, the speed is the culprit. Again, you can observe here that.
Along with this particular message, there is another term which is added over here, which is nothing but cost 2 pi fdfde, and because of this, the received M of T will fluctuate, and then this can severely impact the reliability of communication. So far, what we have seen is...
You know, either these are the only two terms that you'll have to remember. One is cos phi, then next is, this is nothing but cos 2 pi fdfde, and how, what does cos phi equal to? Phi is equal to minus 2 pi.
FC, tau, and fdfde. We'll have to calculate, or else I'm leaving this as an assignment to find out how to calculate fdfde. So, if this particular problem is asked in the, you know, what you call as in the examination, what of course there will be a problem based on this for your EC to.
I'll be doing a revision where you can, I'll be explaining how to prepare for your EC2 examination as well. There, you'll have to mention, you'll have to mention about, you know, the coherent demodulation, and then finally you'll have to only mention about these two parameters, that is cos pi and then cos 2 pi fdfde.
I'll not be asking all these particular derivation. This is not required. This was only for people who are interested in knowing how actually we get this particular terms. If this is clear, we'll move on to the next topic. So slightly now, we'll have to go to the intervicular networking protocol. This is also.
Because of the kind of signalling schemes that I will be considering now, is it clear?
Any dots or anything? So this is what, and in order to extract information about this, the phase and then the Doppler, you know, shift, we will be explaining or else would be getting some reference signals.
OK, or else we'll be transmitting, you know, reference signals before the actual transmission of data. OK, let me explain this particular transfer once again. So, as mentioned earlier, there are two vehicles.
Okay, so which are communicating with each other and there is some relative velocity we are. So let's assume that this vehicle is transmitting or travelling at 50 kilometres per hour and the other vehicle is travelling at 30 kilometres per hour.
So, we all know that the relative velocity is equal to 20 kilometres per hour. So, this is the mechanics of the vehicle. Now, a transmitting frequency of the first vehicle, which is trying to convey some information that is dfc, it may be transmitting at some 5.9 gigahertz. Now,
So this is the frequency at which the V2V messages are transmitted to the vehicle 2 over here from vehicle one. Now the question is, is the carrier frequency, will there be any change in the carrier frequency at the receiver? The answer is yes.
There will be slight offset. It is plus or minus fb. So in case if V1 and V2 are moving away, then it will be difference. If V1 and V2 are approaching with towards each other, then that would be nothing but positive. Now, what is the impact?
of, you know, what you call this slight change in the offset, or you know, because at the receiver you will not be knowing, you will be still assuming that, you know, the signal is transmitted at carrier frequency of C and then you will be multiplying. So what would happen is at the receiver, as we have seen earlier, for M of T, it would be multiplied by class phi in case of.
Finite distance, but here it would be multiplied by cos of two by the Doppler shift fdfde times T. Now you can see that the signal will actually fluctuate with respect to the Doppler shift. Now, as you all know, higher the Doppler shift.
OK, you know the higher Doppler shifts specifically will arise when vehicles are transmitting at higher velocities. Does it make sense?
Is it clear? Okay. Now with this, we'll move on to the another topic. See, I'm just, you know, converting all these slides here. So it will take some time so that it will be easy for all of you to understand. This, I'll be sharing this entire slide deck by probably by Monday or so. Okay.
I'll stop sharing the screen for a while, and then I'll start with the topic of the digital signals.
And with that, I'll get into the following topics, which I...
I'll the previous slides, and then let me just get into those details.
In meantime, if you are having any doubts, please let me know in the chat box.
M.
Just a minute.
Okay, I'll do that.
Just a minute.
How to get to know the special signal values? That I'll consider when we consider the frame format.
We'll have to at least wait for at least another five to six contact sessions until by the time we reach that stage to understand all these topics. Okay, so let me just go ahead now. So far, what we have considered is all with respect to the analogue signals.
Now, you all know what an unlock signal is. I think I've already mentioned. Maybe I'll be including the animations also, just like the way I've shown.
For everyone to understand, so I'll be sharing that part. OK, so now we all know what an analogue signal is, which we had considered in the previous classes. In case of analogue signals, a typical example of a voice signal at the output of...
The at the output of the microphone is an example of an analogue signal. We have M of T and then T. In fact, this can represent any sensors, Automotive sensors. If you consider IMU sensors, that is inertial measurement units, either it can be accelerometer.
Or it can be a gyroscope. OK, so the output is nothing but an analogue signal, because the signal is defined for each and every time instant. It is defined for each and every.
Time instant.
And also, so this is continuous along not only a long time axis and also within the finite range. Say, for example, if you consider or if you take the example of an accelerometer, it can vary from, say, for a, you know, it can vary from zero or probably from.
Some max negative value till a Z to minus a Z also over here. Any comments in the chat box? What is the standard value of what would be the measured value of a Z on an accelerometer?
Any comments in the chat box? What does the standard if you consider an accelerometer mounted on a car?
What is what will be its standard value? Will it be equal to 0 or is it not zero?
It will be measuring the acceleration due to gravity and then it will be 9 point metres per second square. Now, in this range, the accelerometer can probably, not only in this particular range, it can show any value in this amplitude range. So it is, say for example, it can take 9.8888
or probably it can take 8.9.8111 or probably it can also take 8.11 depending upon the way the vehicle is moving, whether it is going upward or downward. Okay, so it is taking or else it is defined for each and every time instant and then it is defined for across all the values along the amplitude axis.
Now such a signal, I'll be showing the block diagrams or the simulations later. So this is given to a block.
Okay, so which will convert this to a digital values.
This will be converted to digital values. When I say digital values, these are sequence of one and zero. Okay, now what is name the block which converts this analogue signal where it is continuous time and then continuous amplitude to just a sequence of ones and zeros or zeros and ones.
It's called as ADC, and within the blocks.
Within ADC, what are the sub-blocks? Within ADC, the sub-blocks are sampling.
The procedure I will explain later is sampling, and it is called as quantization.
Sampling will measure the signals only at discrete time instance. Don't worry about this now. It's not required. And then the quantization will convert, you know, to only finite amplitude levels.
Finite amplitude levels, so this you need not have to worry now. OK, but this is how. Now, finally, these ones and zeros are nothing but logical values.
logical values and this is what the ADC within the ECU, that is the electronic control unit, or the ADC of the sensor will be throwing out. It is a sequence of ones and zeros. Finally, this may be used by the application layer. Say for example, the electronic stability program.
or the anti-lock braking system in order to decide the actuations or else these are the values which will be transmitted or these are the logical values which will be shared with the other ECUs in the network. Okay, share with, it can be used or it is shared with.
Other.
A SUHAS.
In the...
Network.
There with this now, again, in this case, you know, let's consider, or else let's see how this logical values.
Up.
Converted.
Two digital signals.
See, what is the definition of a signal? Any comments in this chat box?
What is the definition of a signal?
What is the definition of a signal? A signal is an entity which varies with respect to time.
which varies with respect to time. It can be any, it need not have a fixed frequency, but it varies with something which varies with respect to time. The information that is stored in the memory, that is in terms of logic ones and zeros, those are logical values, but these are not digital signals. So this is one.
clarity that one should be having. Now, say for example, we will convert these logical values to digital signals, mainly depending upon, you know, we'll see, you know, multiple such formats, and we will try to see, you know, how that would be represented in CHANDAN, LEN, and then.
Automotive Ethernet, also that would be a focus today. OK, now say, for example, today we will now we will consider.
One scheme with this, we'll try to understand. OK, before I, you know, enter or consider this topic, I have to ask this particular question: Is there a difference between bit rate and baud rate?
Are or are they one and the same?
Is there a difference between bit rate and baud rate?
Are they same? Because often with respect to CHANDAN networks.
Will be saying.
And a baud rate. So what it is, they are different. This is what we'll try to understand in today's class. Probably some may not be aware of this. Let's try to understand this one. Say for example, let's say that the incoming bits are 101.
Now, the way we can represent a simple way of representing this is one is represented for a duration of...
TB seconds. OK, then again, zero is also represented for a total duration of TB seconds, so that is 2TB, and this is represented by some voltage. OK, so say, for example, I can say that you know this is represented by a volts, and as you can see here that...
Why this is actually a digital signal? Because one, you know, one either it, you know, here also you can observe that it is taking only two voltage levels, either it is 0 volts or either it is nothing but three volts. Now again, this is nothing but three TB. Now, how to define this TB? Any comments in the chat box?
How to define this particular bit duration, bdb?
Any comments in the chat box?
It is called as bit duration.
This is called as...
The bit.
duration. Okay.
Now, we also define a term called as data rate. Now, what actually data rate refers to? Any comments in the chat box? In fact, it is written as data rate is written as RB. Now, what does this data rate RB refers to?
What does this data rate R be referred to? OK, now what it says is that if one bit.
Requires.
T.B. or as you know, I could, if you know, I will write it the other way around if T.B. in in T.B. second, if I'm just using one bit.
OK, then, so please note this is in time duration. Then, in one second, how many bits can we transmit? How many bits are? I'll just write how bits, so try to understand this, how many bits? So, and this let us denote this as RB, and now we can see that, so this is an inverse relation.
RB is equal to 1 divided by TB, which indicates bits per second.
So this is the concept now.
Another question, so this is how we obtain the definition of the data rate. Now, another question that you should ask is, is this the only way that we can represent the bit in terms of a digital waveform or a digital signal? So I can, I'll just call this as D of T. Any comments in the chat box? Is this the only way or?
Are there any other kinds of representations as well?
Are there other kinds of representation as well? See, this type of representation, in fact, will be relating this to digital modulation as well. This is called as unipolar.
Coding scheme. OK, when what does unipolar? Polar means it refers to the parallel polarity. Unipolar means it's only single polarity. Either it has positive voltage and then again this is going to ground or which is equal to 0.
On the other hand, you can go for polar coding scheme. Also, the polar coding schemes works like this.
So, this is...
In case of polar coding scheme, one is represented by positive voltage, and then zero is represented by negative voltage. OK, so this is how we are representing, so this is equal to bit duration, so this is equal to by the bit duration, this is.
to raise the bit duration. This is of course the time access and this scheme is called as polar coding scheme. Okay, now another follow-up question where there are chances of...
Zero being easily changed to one or one being changed to 0, because essentially when we consider the reliability of the links, in fact, in one in CHANDAN networks, we consider what is called as bit flipping, or you know, bits getting flipped, or we also calculate a.
important term which is called as bit error rate we'll consider. Now why do you think that say for example the one here will change to 0? Any comments in the chat box?
Why one here can get changed to zero?
there will be noise in the system. You cannot avoid this particular noise and then there will be interference in the system as well. You are EMI, EMC, right? So there will be noise in this particular system and then you can see here because of the noise in case you know it can get flipped to 0 easily because now we can observe here that.
The distance between the voltages of one and zero is only 5 volts, so the chances of 1 getting flipped to 0 or 0 being flipped to one is very much easier. All I have to do is take the mid value as threshold here during decision. I'll say that if the voltage is greater than 2.5, I'll...
decide in favour of 1. If it is less than 2.5, I'll decide in favour of 0. So the noise can actually increase or else it can create disturbance in such a way that the time instant where you are deciding can make or can create error decisions. On the other hand here, if I represent this as 5 volts and this as...
minus 5 volts. Now you can see the distance, total length is nothing but 10 volts. Okay, so the noise has to be significantly high or else you know the noise strength has to be significantly high in order for us or the bits to get flipped from 1 to 0 or 0 to 1.
But even this is not sufficient when we consider an environment or an high interference environment like in vehicle where CHANDAN will actually go for a different type of signaling, which we will be considering in a moment. OK, so this is called as you know.
Channel coding, specifically, or not channel coding, this is all as line coding where the bits are converted to the waveforms.
Okay, now if this we have defined here as you know what is called as bit rate and we have defined here bit, then what is a symbol? Please comment in the chat box.
What would be a symbol then? How do we define a symbol?
How do we define a symbol there?
Yeah.
How do we define in multiple bits? Yes, there is some good answer. So essentially what we do is, rather than considering individual bits, so please keep this thing in mind, rather than considering individual bits.
OK, we will consider a group of bits, or else we will just start grouping the bits. I say, for example, instead of representing one and zero by individual voltage levels, we will say that, OK, I will group the bits zero and zero, zero and one, one and zero.
and then one and one. This is an example I'm giving. Of course, now I'll consider this one of the coding schemes for Automotive Ethernet, which is nothing but 100 base T1.
I'll be considering for this the coding scheme for this, so that you know you'll feel you know whatever is discussed is not having any practically relevant. Now, what essentially I can do is, OK, one one is represented by plus three words and 10 is represented by, say, for example, one word.
And 01 is represented as minus one word, and my 00 is represented as minus three words, so if there is a group of bits.
So this is the time axis and the bit sequence I'll write. So this is bit 11 and then 01. I'll just show for only four bit combinations. Okay, now here you can observe that we'll divide or else we'll consider this as one symbol. So 11 is represented as.
plus 3 volts and then 01 is represented as minus 1 volt. But what is the duration for which these bits will be represented? So again, since you are going for two bits, so this is 1 TB, this is again another, so this is 0 TB, this is...
Two TB, not sorry, yes, zero TB, yes, yes, this is.
This is TB seconds, this is 2 TB seconds, and this is 3 TB seconds, and finally this is nothing but four TB seconds. So, since you are combining 2 bits, you are considering two bit durations. So, here you can see that this is...
Three volts, so you can observe that the signal level is not changing for every bit duration, but rather it is changing for every two bits, where you know it is defined for, or else you are combining 2 bits. Next, again, 01 is represented by -1 volts here, so you can see that.
The signal is not changing quite often as compared to, you know, the signal which would change for every bit in case of where you when you don't go for grouping. Now here this group of two bits, okay, it is called as a symbol and the duration is called as the symbol duration.
Okay, so this entire thing is called as the symbol duration and the signal changes for every PS seconds. So this is the symbol duration. Now we can define the baud rate. The baud rate here is nothing but the inverse of the symbol duration that is
One divided by cf, and then this is given in terms of symbols per second. Earlier, it was bits per second, and then this is called as the symbols per second.
Now again, there are different choices of coding schemes over here. So now we will consider one such example, which is the case of 100 based even. Okay, now another important aspect to understand here is that if we have or if you are combining 2 bits, how many voltage levels are there?
But any comments in the chat box?
If you are combining 2 bits.
How many voltage levels are required? Totally 4 voltage levels, so that is 2 to the power two bit combination, either zero or one, so this is equal to 4. In case if you are going for two to the power three, so this is equal to 8 voltage levels are required, and here the symbol duration.
is equal to 3 times the bit duration that is TB. This has to be kept in mind.
Okay, now, well, before we get into how we know the in-vehicle networking and then you know what CHANDAN is all about, what, how it is represented, let's consider a typical example of 100 base T1. Any comments in the chat box what 100 base T1 refers to?
This is irritating.
What is 100 based even and what to what kind of technology it is referring to? We'll understand, you know, the TIMBLE scheme or the coding scheme for this part.
What is 100 base T1?
Okay, so 100 refers to the speed.
which is in terms of megabits per second and what T1 refers to.
I think one of you have mentioned it is referring to single twisted pair cable.
Okay, now this is also a technology based upon Ethernet. Now, why these things are explicitly specified? Why one has to explicitly specify it as T1? In what way it is different from the Ethernet cables that...
We use at our homes.
Any comments in the chat box? In what way it is different?
How many twisted pairs cables would be there in a normal Ethernet cable?
Only 2 pair.
Only 2 pets, is it really?
It can go up till four pairs.
when I say it is totally 8 cables. Now why can't we use directly this cable inside an automotive environment?
Why can't we use?
This directly in inside an Automotive environment.
It adds weights and then of course, you know, that's not an economical way because it increases the field usage. Now, in what way the bits in 100 base T1 are represented?
Okay, so let us try to understand that part. I think in a single pair, I'm not sure, Manik and then, you know, Alex, you know, we can consider this later, Automotive Ethernet 2 pairs. Maybe we'll consider this later, how it is and all. Okay, now what is the coding scheme that would be used over here?
Okay, now this particular coding scheme, it is actually referred to as, you know, there are, I'll be explaining this in detail now. Please refer to this one. Okay, in this particular case, you know, this is, this particular coding scheme is called as, you know, say for example, the coding scheme.
or you know the line coding scheme. This is referred to as, okay, definitely we'll be getting a question based on this one.
OK, so this is named as.
Four-bit.
Two 3 bit.
Okay, so this is called as 4B and 3B.
OK, then comma again.
Three-bit.
Two.
to ternary pair, it would be clear as to what these terms are. I'll also share the references as well.
OK, so this is 3B and two D. These are all, you know, single terms, and then three level pulse amplitude modulation, and...
Three-level pulse amplitude modulation, or else this is also called as PAM 3. It will be clear in the moment what actually these things are. OK, now let's start by considering an example here. I think that is the best way to understand. Let's consider this is the data that we're having. In fact, I'm just...
directly taking this from the TI reference sheet. I'll just share this with you all. So we have.
You know, this is the stream of bits which is being transmitted. Okay, so this is the stream of bits. Let's see how the stream of bits are given.
Okay, of course there is a concept which is called as media independent interface and then media dependent interface. I'll be considering these talk.
details of Automotive Ethernet. Now, simple question, if we want to transmit or else if we want to have a data rate of 100 megabits per second, okay, 100 megabits per second means 100 into 10 to the power 6 bits per second.
What should be the bit duration? This is a simple calculation that I'm giving.
What is the bit duration?
What is the bit duration?
Simple, you just take, you will have to take the inverse of this one, so that would be equal to 0.01 into 10 to the power -6 seconds, or this is equal to 0.01 microseconds.
Does it make sense?
LOKESH, is it clear? Just take the inverse of this one. OK, it is 0.01 microseconds, but this is not.
Advisable.
To transmit at such low data rate, because you know it comes with one, it, you know, it will lead to a lot of EMI EMC complications.
It will lead to a lot of EMI EMC complications, so that is why.
The we are, you know, the Ethernet before you know when it provides the bits, it will give 4 bits in parallel.
If it is giving 4 bits in parallel, let's see what is the clock rate at which it will be working. Now we'll have to take it for granted. Yeah, if it is transmitted at if the data rate is 100 megabits per second, then the clock frequency.
Okay, I think everyone should be knowing, you know, without clock, even a watch or the quartz crystal will not work. Okay, what is EMIMC? EMIMC is electromagnetic interference and then electromagnetic compatibility. See, when signals, when it changes at very faster rate, as you can see here,
So in this case, you know, the symbol is changing every TS seconds, or if the signal is changing every TB seconds, that means to say that it is changing for every 0.01 microseconds. It is almost in terms of, you know, the nanoseconds. This will actually cause, or else in this case, you know, the...
PCB lines will itself act as an antenna. It will start radiating and then it will start interfering with the other components in the automotive network, which can significantly impact, you know, the critical decision making processes of the other issues and all. So this is EMI EMC, okay.
I've tried to clarify as much as possible. Now, so rather than, you know, see if it is 100 megabits per second, then the clock speed would be equal to 100 megahertz. All you have to do is just replace bits per second by hertz. That's it. Okay, so this is the clock rate.
This is the clock rate. OK, now 1 Hertz is equal to 1 bit. Now, if I'm providing 4 parallel data streams or 4 parallel bits, then what will be the data rate on the individual lines? So.
You can assume there are individual lines like this. OK, so if you are providing 4 parallel data bits, what will be the clock rate then? Should it have to be 100 megabits per second or 100 megahertz or less than that?
Seriously, will it remain the same?
Less than how much less?
OK, let's assume that there is a single line.
There is a single line now.
If I, if I consider one second at the input, if this is one second, then I would be actually obtaining 100 megabits.
100 megabits per second. OK, I think I've got the right answer in the chat box. Now, instead of using single parallel line or single line, I'm using...
Four lines.
OK, in one second.
OK, see all I want is 100 megabits.
So at the input, or else here I'm setting the clock to 100 megahertz and then I'm getting 100 megabits per second. So now if I have 4 parallel lines, do we expect the input clock rate still to operate at 100 megahertz?
Not essentially, each of these individual lines are providing at 100 megahertz software, 25 megahertz, 25 megahertz, so each of these lines are providing 100 megabits, sorry, 25 megabits, and then 25 megabits, and then finally when you aggregate, or else finally when you...
add sum up so that will be equal to 100 megabits per second. So that means to say that please remember that.
At the media independent interface, which will be considered later, the T in 100 base T1 will have 4 parallel lines, and each of these particular 4 parallel lines are operating at 25 megahertz. Does it make sense?
Okay, so if this is clear, so now we have considered 4B. Now this 4B has to be converted to 3B. In fact, this is what we have discussed over here. So this 4B has to be converted to 3B. So essentially...
First time instant you have got 0101. Yes, done. Next instance you get 1000 and next instance you get, you know, 4 ones. So these are a different time instance that you would be getting. Now we will categorise this into or else we'll just group 3 bits at a time. So you can observe here.
So I'm just grouping 3 bits at a time and then I'll write these three bits separately. So this is 010. This is one set of three bits. So this is 1100, 01 and then followed by 111. So now we have converted from
4 B.
Two, three, B.
Okay, so this is what we have done. Now, this particular 3 bits will be converted to the voltage levels. See, earlier we were directly mapping, you know, the bits, you group the symbols and then you directly group it into bits or, you know, to voltage levels. Now, here we are converting from 4B to...
Three B, and then you are mapping this 3B to the voltage levels. Does it make sense? Any comments in the chat box? Is it clear?
Okay, now what is the table for this to map this? Okay, the table, this is again given by standard. So let me write this here. So always writing helps, you know, three bit data and these are the voltage levels. So this is nothing but TA.
And then, TB.
It will be clear in a moment what actually. So if the bit combination is 000, then the voltage levels are minus one and zero. If the bit combination is 001, then the voltage combination is 01. If it is 010, then it is.
Minus one and one, if it is 0111, then it is zero and one. This is not that I'm just, you know, fabricating this table. This is what is prescribed by the standards. I'll have to refer to the standards.
Okay, so now again, we'll go for the rest other four signals. So that is 1001 and zero, then again followed by 101. So this is 0 and -1 and then 101 and -1.
And this is triple one, so this is nothing but zero and -1. Please note, these are not logical values, but rather these are voltage values. These are voltage values. Now, if you remember in the earlier slides, we had referred.
to unipolar as one polarity. Okay, so this was one polarity, maybe plus 5 volts and zero. Then polar scheme, we had referred the voltage level as plus or minus 5 volts. Now here, how many voltage levels are present? Please comment in the chat box.
Here, how many voltage levels are present?
Two, three.
three voltage levels. What are those voltage levels? Those voltage levels are one, zero, and minus one. So these are what are called as tertiary or ternary voltage levels. In fact, you know, this is what I've measured or mentioned here. Two ternary, 2 ternary means you are representing.
Two voltage values which can be either zero or one, so you can observe here there are two voltage values which can take any of the three levels, either it is -1, zero, or one. Now, for 010, you can see here it is represented as...
The voltage level, so this would be equal to...
Minus one and plus one, 110 would be represented as one and minus one and 001 would be represented as voltage levels zero and one, and then followed by this.
Triple one would be represented by zero and minus one; does it make sense?
K.
Now, please comment in the chat box, what is the symbol duration or what is the duration of this each voltage level?
What is the duration of each voltage level here?
Comment in the chat box.
See here we know each of these 3TB, okay. This is where you know, at least it's a good attempt, but it's not 3TB. See each of these lines are operating at 25 megahertz.
Okay, so the aggregate total we know that it will be equal to 100 megahertz. Now when you convert this to R is when you map this to three bits.
What will be the clock rate? Let's go in terms of clock rate such that it will be still.
100 megahertz.
See, it doesn't matter for every bit I have to be obtaining 0.01 microseconds. Is the question clear? See, what I'm doing is...
Four lines.
The aggregate data rate, see here, each of these, you know, on data lines, we are pumping the data at 25 megahertz. Finally, Sachin, I'll come to this question in a moment because it is not that related here. So now this is converted to three lines over here.
Now, each of these particular 3 lines, finally, when it will be converted to bits or whatever it does, again, still, you'll have to maintain an aggregate data rate of 100 megahertz. So what will be the clock on the individual lines? Please comment in the chat box. See, here it was 100 megahertz. You just divided this by 4.
Same thing here also you need to be having 100 megahertz so what will be that? 100 divided by 3 exactly so that is 33.33 megahertz.
33.33 megahertz. So at this particular point, you have 33.33 megahertz, because when you multiply it by three, the aggregate data rate is equal to 100 megahertz. So to answer, you know, Chandan Kumar's question, see, it is not that straightforward here, getting, you know, the bit rate or, you know,
the data rate or the bit duration. Now, with this, if these two bits...
OK, see this entire.
That is where it went. What was the OK, 010, sorry.
Now, comment in the chat box.
What will be the duration of these two bits?
I know this is slightly confusing, but we'll have to deal with this because these are all the automotive concepts. And then I'm trying to explain this as simple as possible. Now, if these three bits are clocked at, see, please note that all these three bits are provided.
or it is operating at 33.33 megahertz, or else it is provided at 0.03 microseconds. Thanks for helping me. Okay, please note that every, you know, 1 by 25 megahertz are receiving 4 bits, which essentially means to say that when you convert 4B to 3B,
For every 0.03 milliseconds or microseconds, you are receiving 3 bits, so all these three bits are actually mapped to two voltage levels.
All these three bits are mapped to two voltage levels, so that being the case, what will be the duration of each bit?
Comment in the chat box.
Let me write this.
1 divided by 25, yes. So here this is nothing but 0.04 microseconds. Every 0.04 microseconds you are receiving 4 bits. Then again, in order to maintain the same 100 megabits per second or 100 megahertz, even these all these three bits are clocked at.
33.33 megahertz, or else if you take the inverse.
So that is 0.03 microseconds. So in 0.03 microseconds, you are representing 3 bits, or essentially you are representing two different voltage levels. So that being the case, if two voltage levels are represented, then what will be the bit or the symbol duration?
Please note, we are grouping the bits. So in this way, this will be exactly equal to the half, that is, 0.03 microseconds divided by 2, which is equal to 0., you know, what we call as 015 microseconds.
Does it make sense? Is that clear?
So this is, you might ask a question why it is done so. That is something which we'll have to find out in this particular case. You can see here that all the four bits are converted to now what you call as.
Three bits. Again, the three bits are represented by two voltage levels, exactly going by half. Please note, this will be an important question that would be asked in the examination. OK, is it clear? So this is about the Automotive Ethernet. In here, I have not discussed about, you know, two important concepts here. One is...
Media independent interface, and then the other one is...
This is a good question, Shashank. Here, as per the standards, what we have done is for each of these particular 3 bits, we are going for ternary voltages because of some obvious reasons or maybe there are reasons for doing so, not anything is done at random.
So, here we are going for three different voltage levels, that is, one, zero, and one. This is completely different. Earlier, we were just representing it by 8 different voltage levels. OK, single voltage levels. Please note, please note here that single.
Eight voltage levels for each of these combinations, but in here a particular bit combination, 3 bit, is represented by a group or two voltage levels.
Two voltage levels - does it make sense?
So essentially what you are saying is you are representing 3 bits by two voltages, minus 1 and minus 0. So essentially this entire duration of 1 symbol here is nothing but 0.03 microseconds. Does it make sense, Shashank?
Okay, now, as a natural transition, I know I could have actually gone into digital modulation techniques, but rather today, I'll get into the networking protocol. I'll discuss, and then later on, we will move on to, we'll come back to, you know, what you call as a.
the CV to X communication, whereas V to X communication. So I will write in.
OK, and also I will highlight in what way this would be different from the future software-defined vehicles, because this time this is the theme of this particular course. You know, you know, can of course it will it is here to stay, but...
You know, this is the modern day architecture and then it perfectly makes sense to teach this particular topic for doing justice for this particular course. Now, what do you mean by in-vehicular networking protocol?
See, in case of inter-vehicular communication, we were using vehicle-to-vehicle communication essentially in the form of basic safety message to ensure that, you know, the decision as in the decision of a particular vehicle is conveyed to all the other vehicles in the surroundings as well.
Now, why do you require in-vehicular networking inside or within the vehicle? Any comments in the chat box?
Let me consider a scenario.
P.
There is a vehicle.
OK, say, for example, let us consider an ECU.
The ECU's responsibility is to switch on the lights depending upon the status of the door. That is, you can call this as front right, this as front left, and again, this as rear right, and then rear left.
Now, as per this explanation, we have considered one ECU. ECU is nothing but an automotive grade microcontroller, which will come with some aluminium casing and then the harnesses which will not allow the water to easily enter inside the ECU. Okay.
or inside the microcontroller. So what all you require in order to realise this system? It's very simple. If any of the doors are open, switch on the lights.
What all do you require? Any comments in the chat box? What, how many sensors and then how many actuators are required for this purpose?
Cool.
Yeah.
Okay, there would be 4.
Sensors, and what kind of sensors are required? The sensors here are nothing but the switch. OK, now how many actuators are required?
Only one actuator. What is that actuator? It is nothing but the light which is nothing but an LED.
Now here, what we can do is the sensors here on the front door, it can be connected directly to the front ECU. And then even the LED is also connected over here. I'll just indicate like this. This is the LED here. And then we can also, for the saving,
money we can also connect the rear left door.
You know, sensor to the ECU, then again the rear right doors to the ECU. Do you think this is a good strategy?
Do you think this is a good strategy?
Okay, yes, it may be, but it will save money. Now, if I'm interfacing...
the switches to the ECU or else in general any.
Say, for example.
Maybe I'll just...
Call this as a sensor. OK, it can be switched or whatever it is. OK, now what is the protocol that is used to connect a switch or a sensor to an issue? Any comments in the chat box?
What is the what are the protocols by which you know the sensors will be communicating? Yes, ADC. Probably, you know, the sensor itself can house an ADC, in which case it will be throwing out the data either through spy protocol or else it is nothing but ITC protocol. OK, MANIKANDAN not.
So the sensor will always communicate with the ECU. In case if sensor is having an inbuilt ADC, it will be communicating either through SPI protocol or else through an A2C protocol. Now these are please note short distance communication.
In this case, you know, the wires connecting the sensor to the ECU will not go beyond maybe even centimetres also. Okay, it will very few, probably 30 centimetres or whatever it is. Of course, you can argue that in house environment we can go for long cable lengths, but that is not advisable.
Okay, so in case, imagine you have been seated in the car and then if the switches of the rear left door and then rear right door are directly connected to the ECU where the cable length will be in the order of meters, even if the doors are open, you may not be able to detect that one because of the long distances as the signal propagate.
It, the signal strength reduces, and also to add, you know, there will be interference and then the noises as well, so that is why a good strategy and specifically, you know, the status of, you know, the doors are also required because as soon as you switch on the engine, the doors has to get locked.
So, these are all, you know, important terms or important, you know, sensors or actuators. So, what I will do here is that I'll bring in another.
ECU and its responsibility is to read only the status of the rare doors. So this will be through this PI protocol or you know probably I2Z or whatever it is maybe depending upon the sensor. Now the status of this door will be communicated from.
the rare ECU to the front ECU, either it can be through a CHANDAN network or else through an LINT network. Essentially, yes, even for boot or also you can go for a different ECU.
You have to go for a different issue. If, as I've heard or else probably as I've discussed, in case if the length of the wire is beyond certain 1 metre or 1 metre means it is almost 3 feet, it would be half the length of the vehicle. So under those instances, it is better to go for.
CAN or LIN protocol, so essentially one CAN and LIN are also exchanging, or as you know, it is transmitting the sensor data, but it is for the long distances. Is this clear? You know, one of you had mentioned CAN and LIN. Manikandan, is it clear? OK, if the sensor is placed in the...
close proximity, then we go for SPI or I2Z. If the sensor is placed at some far distance, then add another additional ECU and then communicate that particular signal. Now, in case of the in vehicular networking protocol, in case of traditional network, the skeletal architecture is fixed. Say, for example,
Even I can go, I'll be showing those lights, you know, in the next couple of in the next class. Now, this particular wiring architecture is fixed.
This.
Wiring.
Architecture is fixed, that is.
Before you know when we launch the vehicle or before the vehicle is designed, we know how many issues are present, okay, and how the issues are connected and the signals that are connected or the signals that are exchanged across the issues are also fixed.
OK, so this is what you know the traditional networks.
Art Design.
One more note, this is what I said. If you, you can use, nobody stopping you from using a single issue for friend. See, that is, you can place the issue like this also. It doesn't matter. But it is the decision of the OEM and then also depends upon the issue.
Vehicle dimensions, also OK.
Now, this is what you know the traditional.
You know, architecture is where the number of issues, the connexions among the issues, and even the signals that are exchanged by the issues are known a priori. We'll be discussing this one, OK? And we, I think you would have already, you know, described, you know, the canned dcb file creation. Is the canned dcb file creation done in your...
as part of your lab exercise.
Okay, then I think it should be clear. Anyways, I'll be going through that also in little detail. So in that, you know, you would be defining all the ECUs, the signals that are exchanged by ECUs, and then also you will define, you know, the communication matrix as well. Now what the communication matrix will inform us is
What is the signals that are transmitted and which issue would be received? And before that, is this a broadcast network or is it an unique cast network? Any comments in the chat box?
Is it a broadcast or is it a unicast? Okay, so now I think you know this I'll have to mention because these are all important topics because I see still there is some confusion as per the chat box. See, can is in.
You broadcast network.
It's an because this is 1 difference between.
If I have a network of ECUs, so this is 1 ECU and this is another ECU.
And this is another ECU. All are connected to a common network or a common cable. And this is a single pair.
Twisted cable.
Okay, now if this EC1 wants to transmit any information, it just transmits.
It's just transmit to ECU 2 and then ECU 3. Now the question here is 2 things. ECU 1 transmit independent of whether ECU 2 or 3 wants it or not. Okay, so this is one of the biggest problems, you know.
Of course, you know it was suitable for the earlier networks. So in here it will transmit whether ECU 2 or 3 requires it or not, and then it is transmitted to all the ECUs.
all the ECUs will hear to this message and then it will receive also. In fact, all these ECU 2 and 3 ECU 3 will receive that message also, but it will get filtered at a particular junction where it will decide whether it is required or not, depending upon that particular message.
Communication matrix, and this kind of network is called a signal-oriented architecture.
This type of network is called a signal-oriented architecture.
Okay.
Is this clear? So those who have answered unique cache, is this clear? So it's a broadcast. ECU1 announces, hey, I've got this particular message. It could be received by ECU2 and ECU3. That is for the ECU2 and then ECU3 to decide whether it wants to retain that message or discard that particular message. So that is called as scan filtering.
We'll be considering that as well in this, probably in the later part of the course, or probably in the next couple of slides. OK, is this clear to everyone? So this is single pair twisted cable. So this is a broadcast kind of communication. Now, how good does this particular architecture, or how easy?
Or can I say, as this is this architecture scale level?
Because often we hear this particular term, CAN is not scalable. What do you mean by a CAN network not scalable?
Either this, what I have mentioned here, this either can be a CHANDAN network or else it can be a LIN network also. It doesn't matter.
Now is this architecture scalable?
Any comments or how easy it is to add an ECU which is already on road or on a vehicle which is on road? Any comments in the chat box? So this is the new ECU which I intend to add.
This is what is called a scaling. That is adding a new issue. How easy it is. Hard to add because again, we'll have to define the dcb file. We'll have to call back all the vehicles. So the scalability is an issue here.
The scalability is an issue here, so that is why, and also these are all microcontroller based.
Architecture.
Now, one is scalability. Now, how easy it is to update.
Vehicle software.
Now say for example, there is an issue in your vehicle. Can someone update the vehicle remotely? Just like in our mobile phones get updated to different versions of Android or probably some different, you know, some different.
I know some extra add-ons or patches or whatever it is. You are saying yes, but with this microcontroller-based architecture, is it doable?
It is not possible. All you have to do is take the vehicle to the service centre and there the technician will update or else you know he or she will connect to the OBD port and then the vehicle software gets updated. Okay, now can we address these two issues by changing?
The architecture.
So that particular architecture here is called as service-oriented architecture.
This is called as.
Service.
oriented architecture. This is also called as a SOA. Don't get confused, both the acronyms finally end up being the same signal oriented or service oriented architecture. In case of service oriented architecture, at the centre of or the of the core component is.
in high performance computer. Now this high performance computer, is it a microprocessor based or is it a microcontroller based? Comment in the chat box. High performance computer.
It is microprocessor based, just like you know the processors that are used in your PCs.
That is what makes the vehicles to be updated remotely, firmware over there updates. Now, this particular HPCs are all connected to the second layer, which are called as zonal ECUs. Again, this is also a microprocessor based.
So, this is also a zonal, this is also again a zone.
zonal issue and this is also another zonal issue. You can see here there are connexion from HPC to this particular zones.
And also, each of these zones can communicate with each other.
So this is more or less predominantly a microprocessor based architecture. Now the question is, what is the role of a microcontroller or the ECU?
the microcontroller or ECU, or where this particular CAN network would come into picture. Now, what I have written in this blue coloured line, these are mainly Automotive Ethernet based networking. Now, for these particular zonal ECUs, we have what are called as 8 GCUs that are connected.
But.
You can see here, so this is 1 edge ECU.
Or else, I'll call this as an ECU, and to this particular ECUs we have the different sensors and actuators connected, so this is the architecture of, or you know, the software-defined vehicles.
Okay, and I did want to mention another thing also. So this is the current architecture of software defined vehicles. And this is where, you know, when we have ed GCUs, okay, we have when we have ed GCUs, this is where you have, you know, a CHANDAN network or a LIN network or depending upon the cost, but.
The rest of the places, you know, it is all, you know, Automotive Ethernet based. Either it can be 100 page T1S or probably 1000 page T1S. And then inside this HPC, all your algorithms will work just like your ADAS or whatever it does. OK, now in here, this is actually called a zonal based.
This is called as the zonal-based architectures.
Okay, so this is we are dividing the ECU based upon where actually it is located. I'll be showing this particular diagram in the later classes and this is where in fact it will enable us to have you know what is called as the firmware over the air updates or building an end-to-end application just like the application that we build for.
Our mobile phones, and in fact, this is what I'll be, you know, explaining in the class as well. You know, there are two alliances, or there are two working groups for this. One is called as COVISA.
In fact, that I've already said explained in the previous class, and then the other one is nothing but Sophie. Now, with Kovisa, what we can do is we can convert the CAN signals to some standard signals, which the applications which are deployed on HPCs can understand, and Sophie is what it will help us in.
developing an ECU for an application. Okay, so this is what will be first what we will do is we will get into the details of the CHANDAN network and then LIN network and then followed by this. So this will be the flow for next couple of classes and then after CAN and LIN and grade.

ABHISHEK KUMAR .   1:57:06
B.

Shree Prasad M .   1:57:25
We will get into the details of, you know, the OSI layer model, and after discussing OSI layer model briefly, that is the physical layer, the MAC layer, the TCP/IP, and those protocols, we'll get into one important topic which is called as...
Some IP.
Service-oriented middle, we'll just get some IP and then followed by that we will get into this, you know, just a brief of how we can build an end-to-end application in HPCs using Velocitas. I'll not explain, you know, how to build an app, but where this will have a role to play and also the importance of...
Cooks are here. These will not have implementations, but it will be just discussed as a use case or case study. So next class be ready for CHANDAN and Linn. CHANDAN, since you have already gone through dcb, I'll just briefly go through the steps, but Linn, I'll be explaining the creation of Linn descriptor file in.
Detail. Everything would be considered in detail, which we will be doing in the next class. OK, so if you have any doubts, please let me know in the chat box or else probably we will meet next week. Probably we take class next week in between some 7 to 9. I will keep you all informed.
Any doubts? All good?

ABHISHEK KUMAR .   1:58:51
The.

Shree Prasad M . stopped transcription