Automotive Vehicle (S1-26_AELZC441)-20260829_081814-Meeting Recording

August 29, 2026, 2:48AM

2h 1m 51s

Srinibas Tripathy . started transcription

Srinibas Tripathy .   1:57
But.
Hello everyone, good morning. So right now I'm going to share the screen.
Could anyone please confirm that the screen is visual?
And am I audible?
Okay, ABHINANDH NANDAKUMAR. Thank you. Okay, so today's session is going to be very interesting. So we'll thank you. Thank you, Venkata. Yeah. So we are going to discuss in the last session. We are going to thank you guys. So we'll discuss
About the engine, and we'll see what the importance of the engine as a prime mover in the automotive in the propulsion system also, and then we'll move forward towards the advanced combustion engines that are currently being used by different OEMs.
So, this is this is not mostly devoted towards the what you can say.
Not related to the...
theoretical or mathematical person. However, it is mostly focused towards the conceptual. You should have known this thing, then it will be a means what you can say that today's session is the heart of your Automotive course, I can say, because the engine is the prime one and we are going to cover all of the engine parts.
Maybe in the latter 2 sections, we'll see what we're going to do, so just a recap of previous section. So, in the previous sessions, we have people see right now, so first we we have covered the a vehicle problem.
system and then after that basic propulsion system we have to cover the introductory part where we have found the how to get a home location the process of home location certifications all the introductory part okay
Then later we move towards the chassis and body, or we can say the vehicle structure.
Okay.
So, it is structure where we have mostly version, cec, and body. OK, so these are the three things we have covered in the last previous sections. Yeah, in the last sessions we have covered, so today you can see...
If we go to the slides here, go to the handout. You can see we have reverted up to not three. It was actually four and plus one. One is the, it is completely introduction of.
introductory to the subject course, nothing to the technical side. So for the three conduct section, we have devoted 4 sessions till now and total I have 5 sessions are completed. So those who are joining late, so you should have at least idea that what you have studied till now.
I am getting emails that how I am going to cover some students of doubt. Either they joined late or they have not in good health condition. They have not attended. So you can go to the recordings and follow the slides. Okay. So we have covered this up to this portion.
So, today's from today onwards means another three sessions you can say 4, 5, 6, we devote mostly to the IC Engine fundamentals. OK, so these sessions we're going to be covered, so the introduction, JC and Wadi and from Powertrain fundamentals have already been covered.
Going to the IC engine fundamentals, we focus mostly to the fundamentals of IC engine and then the fuel conversion techniques, torque characteristics, emissions. Okay. So whatever it is in the handout, what we cover more than that, of course not less than that. So we are, even if electric vehicles are gaining the momentum nowadays, but
As a automotive engineer, you are working in the automotive side, or you are interested in the automotive side, so at least you should know the basics, so you will know the what, how the engine is working, how the engine works, then only you will fundamentally or means theoretically also you will have a better idea to.
extend your knowledge towards the other type of innovative technologies that has been currently used like fuel cell, hydrogen, and then EVs. So now coming to the what is internal combustion engine and what is the role in the automotive.
So, if we'll see the power train architecture, OK, the power train architecture, so mostly it looks like this.
OK, let's assume that I have drawn the top view.
So, though, who is a place like this, and uh...
Of course, it's not because it is far away, but I've just drawn for your presentation, then let's say these are the axials. OK, so the objective is, let's say it is moving like this with a velocity B, so it is moving forward.
So, we need, are we discussed that we need the tractive force, OK?
This is called tractive force to move the Vehicle, but how to get that tractive force?
Destructive force is coming from a.
Power source.
Or you can say.
Or you get the powerful for the prime mover, OK, prime mover, OK.
You can write it like prime mover, yeah, so the objective of the prime mover it will act as a source of power, OK, where the power or torque will.
Transmitted towards the will, so through different chain, uh, it will reach actually, so we will not go.
Right now, we are not focusing this portion. What this portion includes? This portion includes your clutch.
Your works.
Propeller shaft or drive shaft also you can say and then later
you can say the differential, then axial so like this. So what this prime movers include? This prime movers it may be a engine, it may be a electric motor, it may be a fuel cell or it may be a hybrid also. For the objective is to provide the
rotary motion okay like torque the torque will be transmitted here and later it will be converted to the linear motion because we move linearly not rotary okay so the prime over is the we will see we will see rotary linear rotary reciprocating again
we need here linear okay we are moving linear but here we need the rotary to transmit the torque again we inside the engine again it was reciprocating because the piston moves up and down so it was linear okay.
So the initial power is producing linear then converting to rotary again that rotary is converting to linear through mechanical components over here. So without delay let's focus on the prime over spot.
Uh...
This is how we have also discussed few things in the last class, but I will not consider that one as a part of the engine, so you can just means start from here, so from a first class, from the first few students have joined also, so it will be good to start from here. OK, so what the engine means, the...
OK, before engine, we should have known that what is a heat engine? OK, you know this thing, the objective is to convert the heat into...
Mechanical work, OK.
How is it doing? How is it doing? It is doing by combusting the fuel and air, so mostly it is 2 types: one is external and one is internal.
Whenever the...
Whenever there is air and fuel is burning inside the closed chamber, we call it at internal combustion engine, and then whenever the air and fuel is burning outside, then we call that the external engine, so if we're coming to the...
example yes the example is like steam engine and internal combustion engines are like we have a day-to-day petrol and diesel engines are whatever we are using in our day-to-day life this is comes from the internal combustion engine. So now without the
We are not focusing on this set. Our object is to, ohh, sorry, the this portion that this is, you know, this thing, this is very common, so...
So, we focus on the combustion engine part. So, before coming to the combo working principle, I thought, because you are coming from different backgrounds, someone is coming from the electronics background, so for a mechanical guy, it is very common to identify and tell which component is what.
From a different interdisciplinary or cross-disciplinary section of the class, so let's first identify the components. So, how it is like this? Let's say we need a rotary motion. OK, we need rotary motion means how it is a.
Going to the rotary, and how the talk is being converted? That's the our objective is to convert the...
The objective is like the...
so this is the well let's say it is the fuel is in the tank so which energy it is taking producing it is love chemical energy so the carbon hydrogen or carbon hydrogen bonds are there so in the carbon hydrogen bond so mostly the
You can say thermo chemical energy, whenever there is a exothermic reaction is occurring, so you will get enthalpy of reaction, and then the energy will be released, and that energy will release means that chemical energy is converted to, let's say, through intermediate.
then it will be converted to thermal energy and then the thermal energy is again converted to mechanical and mechanical you can say reciprocating.
What is reciprocating? Reciprocating means the piston moves up and down linearly, and then that mechanically it was converted to...
Rotary.
You can say rotary, how it is rotary means the this is in the crank.
Up.
OK, so before knowing this very basic working principle, we should have known that what are the components.
So, I if I if there means having doubt then I can draw a very neat and can simple diagram here. So, of course, to hold the piston and the movement of piston should.
Like this, or you can say, OK.
So, what this part is called as? This is called as piston.
Ohh, yes, this one. OK, how the person will move? So, person will move up like this, up and down. OK, so who will do that job? It is...
Done by the, of course, the chemical energy, then thermal energy, then mechanical energy, then it was putting a force, then it is moving up and down. Who will take this one, OK?
This is done by a.
Connecting that, so this this one's called as connecting.
OK, then the connecting load is again connected to to the sorry again connected to the.
Cranks up, OK.
This one crown top.
Here it is, and this is called as crank. OK, and this was connecting that, so it will draw it so it looks like this.
The path of this one crankshaft is the means it will be like this. OK, crank this one connecting that, so this is the bottom portion. You can say this is the bottom portion.
You can clear it like this. Very bad diagram, but just for understanding. So it will like this. Of course, there are some paths should be there for the global to avoid the pressure developed inside the crank case, also called as this one crank case. OK.
So now, for how the upper portion looks like, the upper portion have so many things. So I do it like this. The first thing is that we need the air-fuel mixture to come inside this space.
What this space called as? This space is known as combustion.
This space is known as combustion chamber where air and fuel mixes. OK, so from where it is air is coming, air is coming from this direction and it is controlled by a throttle. What is this one? This one called as throttle. OK.
So, now, if it's a spark ignition engine, we'll study later, we should have a ignition source, and if it's a digital engine, we'll have a different mechanism, like we have a fuel injector, so you can say either it is a what is called either it is a spark.
It says spark or an inductor.
And after the combustion, we should have the adjust should move away of the from the combustion chamber, so this is called as adjust.
And.
Again, sorry, so this one called that adjust, OK?
And this one, intake.
So now at least you know that what are the different components so you can see. And to control the movements of air and fuel, we have valves over here. This one integral, this one exhaust valve. Here it is an example of a spark ignition engine. So here's
Park is there, and then in tech valve.
It is the integral and this air and fuel, this is not necessarily to be here. So in diesel injector, the fuel injector are here, so no need of fuel comes over here. So this is for a general perspective, which is like a carburetor engine, we can say this type of, this is a very layman.
saying that, okay, air and fuel is coming outside the engine, not inside the engine. We study everything today. Very interesting things are there in the upcoming slides. So, okay, so just we go from the basics. If you know, then it's okay. Just keep patience, so we go forward. And if it's new to the system, then just...
Try to gain something from this, and this is the share.
a cylinder and then this is the piston then the pin there is a called pin guardian pin. What is the role of guardian pin? I can ask directly in the exam the role of guardian pin. The guardian pin connect the piston and the connecting rod. So the connecting rod is there and then this is the connecting rod and this is the crank pin. Of course the crank pin connect the connecting rod.
and the crankshaft, okay? So this is the crankshaft. This is the crank case, the entire holding zone, this one is called as crank case.
Okay, so this is the sum of the lubricating oils and everything are present over here. We see whenever we go for the lubrication system, okay? So these are the, I think you are now familiar with the components. So whenever I use the terminologies, so you should not have any confusion about the components, okay?
Now, coming to the classification. So before you were explaining that how a combustion engine works, you should know that what are the present, what are the different types of engines. There are so many types of classification of the engines.
And.
that the most important is the four stroke engine and the two stroke engine okay so in the what happens in the four stroke and two stroke engine it is that we need two cycles okay or the four stroke it is two cycles it is like piston
A.
We still will come from top.
To bottom.
Is the one stroke?
Let's say first, then piston will go up.
From the bottom, let's say this is the bottom.
And this is the top, and these are the liners, OK?
So, this is the second.
Stroke, of course stroke. What is stroke? Stroke is the moment from one extreme end to the other extreme end, also called as dead center. If it's completed one dead center to other, it is called as stroke. We will study later, okay?
Just remember, so forced to.
OK, so piston is coming first downward, then upward, then downward, then upward. OK, so as the is like 4 four times the piston is coming up and down, so it is mostly common commonly known as four stroke.
Engine, so how the crank crank looks like, so it will connect to any connecting rod through crank.
So, whenever it moves from this portion to this, your crank will rotate one.
80 degree, OK? Similarly, 180 degree here.
Similarly, 180 degree over here and 180 degree here, so for a four stroke it needs 720 degree crank angle. What is crank angle? This is the angle created by the crankshaft. OK, let's say connecting rod is like this, and then a crankshaft is this.
So, you can say your crank angle.
Is 0 whenever it is, let's say it is making like this.
So, whatever angle it is, 10 or 180 degree also, OK?
If it is at the, let's say it is 0 actually, so if the connecting rod looks like this and the crankshaft is over like this, then at that time it is 180 degrees. OK, so this is the most common working principle, not working principle. I think the reason why...
It's called as four-stroke, and then the two-stroke cycle, and based on the source of means air and fuel induction, we divide again into the spark ignition engine and compression ignition engine. OK, so now you can see the...
Different.
a cycle of a four stroke engine. So this is the past we can tell it like a suction stroke and this one a compression stroke and this one a expansion stroke and this one a adjust stroke. So what happens in the suction stroke? In the suction stroke now we draw here.
A the suction stroke, the initially the piston is at the top, OK?
And the piston moves downward.
Then piston rechat button.
So, whenever the piston reach at bottom of course it will act as a syringe effect whenever the piston has restarted this point. So, you will have a vacuum over here when sub atmospheric pressure. So, what will happen because of this sub atmospheric pressure the air or air or fuel
It will directly means to the to the help of a this one total.
Whenever the driver presses the pedal, so it is controlling the amount of air and fuel, so it is coming into the cylinder. Then in the compression stroke, again piston moves from
This one suction.
Then, in the next row, what will happen?
the piston will go upward. So whenever it is going upward, it is compressing the air and fuel. So what will happen? Whenever it is compressing air and fuel means its temperature and pressure will rises and through the help of a spark plug, whenever the piston reaches at top dead center, so all the compressed air and
whichever its whose temperature and pressure have already raised to a significant value, then through the help of a spark plug, the ignition has given. So what will happen? It will create a blast. Whenever there is a small blast will occur, it will push the piston downward. Then the piston will move
Downward, then what will happen? The piston will move downward and the expansion stroke is occur. OK, this stroke is called as expansion stroke. Again, the piston will reach at this portion, so this one compression.
and this one expansion. So in the last row, the piston is already at the bottom, so it will go forward fine. Whatever the exhaust products is present in the exhaust row, whatever the right now it is exhaust, okay.
So, at this point, you can say it's like...
Combustion product.
Okay.
And here you can say the compressed charge.
What is charge? Charge is like the air and fuel mixture we call that charge in English.
And here we have also charge what is this is fresh charge.
So, initially, the press charge is inducted during the suction stroke. During the compression stroke, the press charge is compressed to a significant value, and in the expansion stroke, with the help of a spark plug, the combustion takes place, and the blast occurs, and it pushes the piston downward.
And during the exhaust stroke, all the gases exhausted like this, so through this animation you can say it is inducting now, compressing spark plug, then exhaust now, suction, compression, expansion, exhaust.
Again, you can feel like this.
One more time, suction valve opens. OK, compression, expansion, and then adjust. OK, now you have a clear cut idea. This is very simple, but if you don't know, then you just recall your concept of previous, if you know previously. OK, so right now we have just studied that what is the four.
Stroke spark ignition engine. Now we will start the four stroke compression ignition engine. Here also we will see the same first instruction and compression and then expansion and then exhaust. OK.
So the question is like that, how the petrol, the spark ignition engine is different from a compression ignition engine. So during the, and why the name is like this. So here what will happen, let us say in the suction stroke, the piston
From top to bottom, the only difference is that during the suction stroke fuel is not there, only air is coming into the cylinder, but in the petrol engine air and fuel both comes into the picture.
Function to the cylinder, so this is substance stroke.
What will happen in the compression stroke?
The compression stroke, the piston will move upward, and the air will only be compressed. OK, so what will happen towards the end of compression stroke when the piston reaches?
This is the compression.
OK, towards the end, the air has been.
Compressed to a very high significant value, so that the temperature and pressure reaches at a very high for a very significant value. So, what is happening now at this point, instead of a spark plug, we have a injector.
Now, we can see if we inject the fuel, what will happen? Just look closely, what will happen? The air molecules are like this. Whenever there is a fuel molecule comes, so what will happen? Let us say this is the fuel molecule. So, what will happen?
Uhh...
It will like this is the.
Zoom view, this is the air, and this is the...
SUNIL.
What will happen? The air temperature, let us say it has some something around 800 Kelvin, but the fuel is injected at a very lower temperature, so of course it has injected by the help of the injector.
So, what will happen? The the hit.
Transfer will occur from the air molecule to the fuel molecules, OK?
Once the heat gained by the fuel, what will happen? The fuel extract the energy from the air and vaporizes; it vaporizes instantly, OK?
So, like this, millions numbers of highly atomized fuels present over here get means vaporizes suddenly or instantly producing a blast. OK, like explosion, so the combustion is taking place.
Then, during the expansion stroke, you can say the piston will move downward and again in the exhaust stroke, the piston will go upward. So, you can see in the animations also, now air is coming only, you can see suction stroke, compression, the fuel is...
Now, you can see the blast occurs.
We shall we go downwards, expansion stroke again, exhaust, you can see the wall was opened, OK, suction.
All open, then this one compression.
This one expansion, and of course, there is a just stroke which has not shown in this animations. OK, now it's clear, so this is the very fundamental things that you know also I have explained, so now it's a means lots of doubts are there, so you can ask in the comment section.
Will clear, will I try to address all the questions, whatever you will ask, like why the petrol engine are lighter, why the diesel engines are heavier, why the diesel engine needs to work at 1000 bar, whereas the petrol engine works at only?
5 bar, 6 bar of maximum fuel injection. Why the speed of petrol engine is so high where the diesel engine has very less? Why the diesel engine has compression ratio is very high where the compression ratio of spark ignition engine is very low? A lot of lots of doubts are there.
You can just ask, so forwarding to the next, we will have a working principle of two-stroke engine. So, like this, I told you the four-stroke working principle. Here, two things will happen: once piston will come from the top to...
Bottom.
And then it will go from the bottom to top, complete.
That's the way, nothing like 3-4 stroke we have to do, so it's it means that the crankshaft is a crankshaft. If you do do the crankshaft notation, it will come from top to bottom, and then the next it will come like this, so the total is 180 degree.
Plus 180 degree, so in two stroke the crankshaft will rotate only 360 degree.
It's only one cycle, okay? You can say one cycle. So now we will see how it is happening. You can see during the, if you see the, also the components, it is different from the post stroke. You can say in the post stroke we have valves.
But in the two stroke, there is only ports are there, you can see.
Okay, spring loaded valves, so here, whenever there is a piston, you can see in the animation also, it is very clear. Whenever the piston uncovers this one.
You can see air and...
Fuel, it's a SI engine spark engine engine, so the air and fuel will come into the crankcase and it is called a suction stroke, and whenever this suction is occurring, you can see the piston expansion stroke is also occurring simultaneously whenever it is going.
Upward, you can see through this, so whenever it is only the uncovering of the ports done by the piston, all the things have been happening. You can see air and fuel, this valve, this valve controls, air and fuel comes into the lower chamber, or you can say crankcase, then...
Then it is sending to the top whenever this port port uncovers, whenever it is compressed, whenever it is going upward, it will be compressed and it will be ignited by the spark plug, so in the expansion stroke piston will comes downward and whenever it uncovers this portion.
This exist will go away, which is called that AJEET.
So, if you see the process, the...
If you see the process, the air and fuel, you can see.
Air and fuel, it's coming like this, and simultaneously the...
Exhaust is leaving means air and fuel is coming and exhaust is leaving. OK, so this process of gas exchange is called as scavenging. You should know this term actually, OK?
The scavenging - what is scavenging? Scavenging is...
No.
Proceso.
Exchange up.
Gases, what gases? One gas, you can say air and fuel, it is entering to the cylinder, and one is the exhaust.
So, what should be our ideal condition? Our ideal condition is that, like in the four stroke, your exhaust.
Fruit all empty.
Are all should be evacuated first, then the air and fuel.
Air plus fuel, it should occupy the cylinder.
or combustion chamber. This is ideal. If it occurs like this, then you can say your efficiency of scavenging means the process of gas exchange is taking place in a good way. If it is not, then of course, but in two stroke you can say there is a dilution.
always the air and fuel mixing with the exhaust. So you could have tell me like why it is like this. So if we are diluting the air and fuel with the adjust, so of course it cannot.
Occupy the.
means occupy the full volume. Of course, there is some dilution is occurring, but we cannot do anything. This is the principle of two-stroke engine. Instead, we take it for the advantages for the, we called it as a internal exhaust gas recirculation technique. We will study later.
So, this will be help to reduce the emissions. OK, this is a two-stroke and four-stroke working principle, so...
So, based on the design, you can see either we have an inline, so in the inline you can see this structures are like this. So, in EU, the pistons and cylinders will be like this. In the V, one piston or cylinder arrangement will be like this, another cylinder piston moments are this. OK, this depends on the what OEM is technique.
which type some is X type, radial, H, opposed, opposed piston, opposed cylinder. So these are a few concepts we study also. Another type of revolutionary concept is called as rotary engine, or you can say one kill engine. If anybody is working with this one, then please let me know.
So, here you can say the air and fuel air plus will each incoming here, so it is compressing. Whenever it is compressing, you can see here by twin spark the spark is occurring and the exhaust product it is swiping away and again exhaust is going in this way.
So, this has very limited use in the production vehicle. In the production vehicle, we might we mostly see these are the configurations of these are the engines, OK?
So now, before coming to the going to the next thing, first I would like to explain about the components of Q Engine. OK, how it normally looks. You can see it will see how it looks. So, what is this one?
Anyone can say, what is this one?
Just type in the comment box, we'll see a.
This is fly hole.
Okay.
Because, after that, this, this, this...
A rotary motion, this rotary motion should be connected to the...
Clutch, what part is connected after that? Of course, it will be connected to the clutch, then to the GB to the gearbox, so it will be it will be like this, OK?
So, this is the crank shaft.
Sunita.
Thank you.
And you can see.
This is the connecting rod, OK?
The main crankshaft, you can, I put the individual crankshafts are over here also.
So, connecting rod, and this is the piston.
Why the shapes are like this?
This is, of course, obvious.
We'll talk about this one, because...
There is a to proper mixing of air and fuel, we need the three things: what do we need?
We need one.
We need tumble.
I'm doing it, squish.
What are these three mechanisms?
The mechanism, sir, I will not divert, I will stick, I will come to this one later. OK, so now you will see these are the valves and these are the cam loops, or you can say this one is the...
Crank shaft. OK, what will happen whenever this is the...
Cam controlling mechanism.
So, whenever there is a.
Rotary motion, so what will happen? This this crankshaft rotate like this, so whenever there is the big end of this one, then it pushes the spring-loaded valves and valves are up and down.
Motion occur, OK, valves up and down motion occur like this, OK? So then we got the suction, compression, expansion, and adjust, OK? The timings are matched based on the two sensors, mostly CKP sensor on the CMP sensor. This is called as CKP sensor.
And here, there is a another type of sensor is called as CMP.
The role of crank position sensor and this is the cam position sensor, so the objective is cam position sensor and this is the crank position sensor, so this is the objective is to track the.
TDC. I just tell what is TDC is the top dead center means objective is the sensors objective is to detect where my piston is at a particular time. Let's say time is T equals to T1 my piston should be at here or here or here or a bottom or top.
Where it is this sense of usually helps based on that only spark is ignited.
Or fuel is injected in case it says CI engine.
So, the now you can see these are the fixed, everything is fixed mechanically. We will see what is the limitations afterwards. OK, so this is the camshaft and these are the valve mechanism, OK?
So.
This is a multi-cylinder engine, so...
Now, we just look into the individual components, how it looks. You can see this is a cylinder head.
Cylinder head, this one bottom.
This one from the top, how top means what is the role of cylinder head?
Cylinder in the cylinder head, there is no combustion is taking place, OK?
The cylinder head it applies the valves.
These are the valves.
Okay.
These are the valves for the first cylinder, this one for the second cylinder, this one for the third cylinder. These holes you can see, these are the...
Pulling jackets.
Used for the water or any type of coolant tool should be extracted through the engine, so to extract the heat, you can see the spring-loaded valve mechanism are used over here, OK?
Now, this is the what is called cylinder block. Here, the piston will be there, and the up and down piston motion center, and above this, the cylinder head is there. OK, so this is the bottom portion, this is the cylinder head is in the top of this one, and just assume that this...
This, I will keep it and put it over this one, then cylinder head and below this one cylinder blocks are there. The role of cylinder block is to hold the piston and all the pulling jackets and everything, and the role of cylinder head is to hold the valves, pulling jackets, and the...
Manifolds, intake ports, intake one, adjust ports, OK.
Here, one interesting thing you can see, why though, just I'm asking, you can see this diameter and these diameters are different. You can see this is a little bit smaller, whereas this is a little bit larger. Anyone, anyone in this?
Today's who is attending this today's session can.
So, this thing, let's say this is the cylinder head, let's say one is small.
And another is B.
So, how do I know which one is intake and which one will be adjustable?
Anyone?
So everybody will think, okay, the big one, the big one should be the adjust or big because we need to pump so much amount of adjust products. So what will happen?
So, it just needs much more area to evacuate, so we we we just did a big valve for the exit and a small for the intake, but let, but literally this is not OK. Why?
The region is like that, this one actually.
Big one is the...
Intake, and the small one is the...
Exist, okay.
So this is normally a normally happens why this is if you will see the working principle itself.
Whenever this is, let's say this is a remember this one, this type of question will come in the exam during the suction air and fuel is entering into the cylinder.
How much should be the outer pressure outside the pressure? Of course, it is the atmosphere, so one, but because of the syringe effect.
How much is the pressure over here? Anyone? It is mostly 0.6, 0.5.
OK, it means, or 0.34 maximum, because of the syringe effect, we will get a vacuum over here; it is be sub-atmospheric, so it will be like, let us say, 1 minus 0.5, so your total difference is 0.5 bar, because of that only your...
Air fuel is drawn into the cylinder through this.
Small person.
But just closely look to the exact one.
What is happening? The piston will move upward.
This one exist.
Just see and just try to understand just before the adjust stroke, it has completed the expansion stroke.
During combustion, of course, whenever there is a combustion, what happens? The pressure on cylinder reaches somewhat very high value. Let us say in the expansion stroke, pressure reaches around 150 bar.
and temperature reaches something around let us say 1500 Kelvin. What will happen? The charge initially entered at 1 bar let us say at 25 degrees Celsius now reached to a value of 150 bar and 1500 Kelvin because whenever in the exhaust
Stroke, the piston goes downward, this 150 reduces.
But reduces in such a way that it has not reduced to a value of 0.5; it reduces around 5 bar or three bar. Let us say 5 bar. OK, whenever the piston is at the top, whenever we just open the exhaust valve, you can see outside each.
Of course, one bar and inside is something 5 bar, 10 bar, 15 bar, so difference is always 5 or 10 or 15 bar, so even if in the exhaust we make a very narrow hole, the all the exhaust will be driven away.
Because of this narrow process, and this is because of a.
high pressure differential.
But here it is a very low pressure differential. That is the reason why we need a larger value, larger valve area just to compensate. We cannot compensate, but at least we can compensate to some extent and to increase the volumetric efficiency we will see.
Later, OK.
So, at least you got the point that, so what is the material mostly used? Either aluminum or cast iron. So, aluminum, which you know, is our thermal conductivity is very high to increase the heat transfer, but it is not mechanically durable. So, C, cast iron is another option where...
We implement, so here we just quickly go through the components. Here you can see the this is the crank.
And then this is the piston; this portion is called a cylinder combustion chamber. What is this one? Combustion chamber.
What is occurring over here? What occurred from the...
The air fuel mixture and it pushes the piston over here. OK, so what is the role of an engineer? The role of an engineer is to select the material of the piston cylinder head and based on the mechanical stresses, formal stresses. OK, we will do a design and we will see how much pressure.
it adjust and how the impact of loading. First, you just have an idea that how the component looks and how the role of these components. Now, coming to the air and fuel coming from here.
Okay, how? Through this?
What is called manifold? It may be a intake one.
Or exist one, okay? So here you can say the intake manifold, or it may be a exist one. It's better to open your car one day and see all the components today itself. I think today you just open your car and try to find out.
Whereas the intake, if you know it's okay, but if you don't know then just open it and just check which one is my spark plug, which is my intake manifold, which my air filter, how it is coming from the airfield part, where the exhaust stroke is going on, where is my turbo.
Ohh, that will see everything in the today's class only, so we try after the end of this class, at least you should have a Automotive engineer, or as you are taking this code, you should have identified each and every component. OK, right now I have told you 3 things: one is a swirl, one is a tumble.
And another is the squeeze. OK, what this exactly indicates and how it impacts the it impacts the air and fuel mixing. OK, so just imagine during the suction stroke, the air piston moves downward, the airs are...
Air molecules are coming straight like this.
Through this port, let's say I make a straight port like this. What will happen? Air will come like this.
Do you think it will mix the fuel properly? No. Why? Because you do not have an ample amount of tumble or turbulence you can say. What is turbulence? The degree of randomness or degree of mixing. You can say here, you can see here.
Just, I will change the intake port shape to this one, which is related to this manifold itself, OK?
OK, now what will happen? Air molecule will directly come here and it will make a turning moment over here, so whatever the air and fuel it is there, so it will try to give a extreme high turbulence over here, so it will...
Help for a.
Homogeneous.
Mixing, got it, so the air and fuel will homogeneously mix.
Because our objective is to extract the chemical energy, so as good it will homogeneously mix, we can extract the energy in a greater way. OK, the tumble tells that.
This one actually tumble means, let's say this is the vertical axis, the movement of air around the vertical axis, this is called as tumble, whereas swirl is also some air molecules will.
People take the vertical will see from the top swirl, which means the swirling motion. This is the tumble means turning motion. It will see from the horizontal axis the movement of air molecules rotations about its horizontal axis. It is called as a.
And.
Swirl, OK, and this motion is called as tumble about the vertical axis and what is squeeze? The squeeze are like this, so if you see the piston is not exactly flat. What do we do? We just make a shape like this. Sorry, we make a shape like this, OK? So whenever it is going upward.
What will happen? Let's say my piston is saved.
Is like this.
What will happen? This one, this one will give a motion like this, this one will give a motion like this.
OK, the air molecules will, what will happen? There is a.
A circulatory motion it can create if we make a small section like this. So this also helps improving the what is called the.
Mixing up, it helps the mixing. OK, so this wall, tumble, and squeeze depends on, as an engineer, how well or how good you are designing your manifold. The shape determines the coefficient of discharge, the amount of tumble created inside the cylinder, and the air and fuel mixing inside the cylinder.
Everything depends on this one, OK?
So, you should know about the we will study, we report one class only of the parameters: volumetric efficiency, thermal efficiency, brake mean effective pressure, indicated mean effective pressure, brake specific fuel consumption, specific power, specific piston mean speed.
We found a combustion efficiency, every parameter we take, and as an engineer, you will design the engine components based on this torque analysis, or you will say that how much the size of an engine should be needed or should be developed to.
design the prime mover, at least you should have knowledge on that one. So now we will just quickly go through this one the piston head, these are the oil rings. What is the role of oil ring or compression ring? These are used to
Because you can see the pistons under.
Okay.
If I zoom on this person.
Actually, the piston, this is the liner now.
This is liner, so this is the piston. So how it looks, how it looks like the piston, you can see here.
So the combustion is taking place. The combustion is taking place over here. Just now I told that let's say 150 bar and 1500 Kelvin.
Do you imagine?
Just below this portion, there is one bar in the crank case, and here it is 150 bar, so you can see there is a extreme high level of pressure differential. So, what will happen? The gases that is expanded over piston, it will try to leak through this one.
force it will and this process also we call is as blow by. This process we know is as blow by. Blow by is the losses of or you can say the escaping of combustion gases.
Through to the crank is to the crank is.
To the crank case.
Through this narrow hole, so our objective is to design a piston in such a way that it should not allowing to the gases to come downward. OK, this is the role of compression rings are there to give a airtight or to also the lubricating oils are there, so...
the lubrication of piston should taking place in such a way that the oil rings are there. It is giving in such a way that the oil and the gases should not mix for the oil ring gives a very good flexibility to the pistons. This is called as piston script.
and this is connecting rod and this is connecting rod bolt and this is the rod bearings. This is the bearings and the bearings have you will see here here it is only the role of connecting rods. OK here only the guardian pin you can see the guardian pin we fit over here.
just to connect the small end of the connecting rod and large end of the connecting rod over here. The piston and connecting rod is connected through a guardian pin over here. So connecting rod, this is the upper connecting rod bearing, this is the lower and this is the connecting rod cap, connecting rod bolts. So these are the few components you can just have a look.
OK, now, in a closure view, you can see these are the compression rings and these are the oil rings, so that the oil rings are used to prevent the oil leakage and the compression rings are used to prevent the gas leakage. OK, this question will come.
Now, we have already discussed about the, this is the crankshaft, crank segment in the top, not in the bottom. Bottom is the camp sub. Don't over confuse this one camp sub and this one, sorry, sorry, just three. So this one the camp sub, so in the below is the.
Crankshaft, okay, so just don't confuse, so this is the crankshaft and this is the camshaft which is controlling the cam mechanism or controls the valves here. Crank means it is providing a rotary motion over here. Okay, so the crankshaft, this is the main journal and this is the counterweight for the balancing purpose.
You can see the crank webs are there.
You can see the positions are different here, each one, here each one at the same time, here it is at the bottom, here it is at the top. So, why the why this this is like this, because we know the sparing sparking order, let's say it's a multi-cylinder engine.
So, let's say this is the first cylinder, second, third, 4th, so the pistons are not arranged like this, OK?
Just imagine if we arrange the piston like this, means at a.
Similar position, so all will go upward. So, at a single time, all the at a single time means at a particular time instant, you can say you will get a spark. What will happen? It will give an impulse amount of force so that the entire mechanical system.
will vibrate. So, what we do? What we do? Let us say here we are getting the spark at the same time in the second cylinder we allow it to do the suction.
Again, whenever there is a suction, you can say another in the third line, the piston is at the middle. And in the 4th, how we can arrange? We can arrange it like this. It is going to be a spark. So just to giving spark.
Giving for.
The print.
Timing.
OK, let us say it is a four-cylinder engine; we normally do one, three.
to 4, like first spark will be given to the first cylinder, then it will go to the third, then second, then fourth. So, that at a different time instant the spark will occur, so that the mechanical vibration will be reduced. So, to do that of course we should have a different piston position
That's the reason why you can see one is at the lower and one is at the upper and because here only it is connected crankshaft so here only the from the crank we get a connecting rod and then we have a connection which is going on okay.
I think it's clear.
Now it comes out.
So, cancel.
OK, this is the.
Crankshaft and here the pistons, everything is there and it is connected by a belt or a chain mechanism. Mostly chain mechanisms are there to transmit the power from the top and bottom. Crankshaft and camshaft are very important. The cam loops, you can see the loops are there. Lope means the designs are like this.
I was not like you for that.
Like this, so it has a very top end. Whenever the top end touches the valves, then what will happen? Your valve will open at a higher value.
I tell the wall timings in discussion how the wall looks like, so this is a you should identify that how the components now, at least you should have idea that.
Now, how or or means the working principle and everything for now will what we can do or we can.
Now we can at least completed the working principle and the major part of the components. So now you can see this is the nomenclature. What is the nomenclature? You can see the piston.
Whenever each are the extreme ends.
OK, either of course, whenever the crank and connecting rods are become straight, of course, the piston is at the extreme end.
This is called extreme end or extreme top. What will happen? Beyond that, your piston will not go up.
But your cylinder head, this is the cylinder head, and this is the...
Piston top.
So, beyond that, we can we not go?
Similar position will also happen whenever there is a.
Trunk at at the means the big end of the connecting rod is that here, what will happen? The piston is at the extreme.
Here also, extreme end.
Okay.
But here it is, bottom.
So, whenever the piston is at the extreme end at the bottom, we call that, we call it as a bottom dead center. Whenever it is at the top, it is called as top dead center, means a dead center. It's dead. It cannot go live and move away or move forward. OK, so...
Now, in general, we can say the dead center. The dead center is the position of the piston and its associated moving parts are the instant when the piston reverses its direction. Of course, it cannot go forward, so it will change the direction and it will come downward. Here, it will go upward. OK.
So, now you can see if it is completing from one TDC to BDC or BDC to TDC, it is completing one stroke. What is stroke? Stroke is the completion of or movement of piston from one extreme end to upper extreme end. So,
Lecture piston is are the.
Stop and then the next it is coming at here.
So this is the stroke length or we can say this is the stroke and the length how much it is travel this is called as L and whereas the this is the diameter diameter of the piston. So if D is equals to L we call it as a.
Square engine, if T is less than L, means diameter is smaller, but length is larger, stroke length is larger, means we will get a piece like this, OK?
You can see diameter is smaller, but of course the stroke length is larger, so L is greater than D. This T is the diameter. This is called the under square. And what is the over square? You can see if the pistons are like this.
Of course, it has varied.
Small stroke length, whereas the diameter of the piston is larger, so this is called as over.
So bigger the based on the applications, the OEMs have designed the piston differently. OK.
Now, you have a clear clarity on that, what is my bottom dead center, and this is my top dead center, and the remaining portion which is available over here.
This is called as clearance.
Volume, OK, so the piston I just mentioned now, whenever it is at this portion, the piston is at the 0 degree, the crank angle, whenever it is moves then 90 degree, then it will go to 180, then 270, then it will come at 360, then again.
One 8180, then it will be covered up for 720 degree, so...
And now this is the bottom portion.
If we look at the top, you can see my stroke is from one experiment to other experiment. What is VD? VD is the sweep volume. How much the piston sweep the air fuel mixture? Of course, whatever it is travel, that travel distance is all that sweep volume. You can see VD.
So, what is ideally the VD should be? It should be pi by 4 D square into L whatever the pi by 4 D square this area whatever let's say this is the piston and mostly it is a circular one so the diameter is this one. So, if we see from the top we will have will look like this so pi by 4 D square into
What about the stroke length it has travelled? This is called as swept volume, so pi by 4 D square into L, and what is called clearance volume, where the piston does not allow to move forward.
Okay, dead end. This is called as clearance volume.
And this one, short volume, OK, you can see.
Top dead center, it is the dead center position at which the piston is farthest from the crankshaft. In vertical engines, it is designed as TDC, while horizontal engines, it is referred to as inner dead center.
OK, in the BDC also came, but it's a let's say it's a horizontal engine, we can say is a IDC or ODC inner dead center and outer dead center for a vertical engine, it is called as BDC and TDC. OK, IDC.
IDC or ODC, but here it is called as BDC and TDC. So displacement volume, you now have a clear. So now you can understand whenever you are going to purchase a bike, you are saying that it is 110 CC.
And whenever you are going to buy a car, they are saying that it is 1.2 litre. This is exactly what they are saying. Are you getting? They are saying the soil volume. Cube C means cubic centimeter cube, cubic centimeter. It means that 110 centimeter cube.
It is mostly for a Splendor. If we take the Apache RTR, there's a 160, so there's 160. It was 180. Okay. Some higher end bikes have also 350 CC.
Also, 650 CCM BMW and 40 bikes, OK? If we to the bike, it is 1.2 liters, 1.8 liters. OK, so this is for the so how much liter you can have hard in your life in truck? How much liter you can say? Let's say a truck engine.
We will hard like 13 liter, 16 liter liter means it's a.
Unit of volume, OK.
So now you got the cubic capacity. So first calculate the short volume of 1 cylinder and multiply with the number of cylinder. Let's say it's a 4 cylinder engine. You have just one, this is the two, this is the three, this is the four. So this is the diameter. I will do pi by
4 into D square and what is the stroke length? Of course it is fixed once the OEM sells your car because it cannot change because of mechanical limitations connecting the crank positions.
So, you just calculate the pi by 4 D square into L, that is volume, and multiply with one, two, three, 4, OK?
Now, this is the clearance volume. So what is compression ratio? Now we can see, if we go back to this slide, you can see how much it can compress. Right now, you can see it is compressing one, two, three, 4, 5. This is theoretically, you cannot visualize if we go to your engines.
this type of configuration we can kind it is only though means just visualizing just means expressing or we can experiencing this type of fundamentals just like that.
Instead of 6, if I will maintain, let's say...
In.
Or 12.
Or 16, so what this compression ratio tells instead of 6, I will make 16 layers. Of course, the piston will compress 16 times the air and fuel.
If it is 6, let us say compression ratio is 6, so what will happen? The piston will compress 6 times, so this is very obvious that if the piston will compress 16 times, so what will happen? Your air and fuel mixture will be extremely high, reaches at a very high temperature and pressure.
That is what we want. We have to extract the chemical energy of the fuel and convert it to the thermal energy. The thermal energy is again converted to the mechanical energy. OK, are you getting or not? So, that is the reason we have to define one parameter. That parameter is known as compression ratio.
So, how it was denoted? It was denoted as the total volume
At the bottom, head center to the clearance volume. OK, it is saying that, let us say this is like this, when the piston is at this, so what will be this volume? This volume is...
Clearance volume.
And whenever the piston will reach at this point, and this distance is L.
Will get this one.
Fat volume.
So, what will be my total volume?
My total volume will be V clearance plus V swept.
So, it is saying that the ratio, what is the compression ratio? It is saying total volume with respect to minimum volume or the clearance volume.
Oh, B South, so one plus B South by bc, OK?
This is extremely critical parameter for your engine design. Now, you can see if my VC, if my VC will increase, what will happen?
Prasad, if you come, what will happen?
Compression ratio, if we reduce, sorry, compress clearance volume, if we reduce, then what will happen? Then this term will enhance your compression ratio will enhance.
This is very obvious. Now we'll go back to that slide. Now we can see as much as I can reduce this clearance volume, my piston can go upward and it can compress more and more fuel. OK, so that's the beauty of higher compression ratio. It can
If the clearance volume is very less, it is meant that the piston is reaching at extreme top ends, leaving behind a very less clearance volume, but it so the piston have come sorry.
If bc reduces compression ratio increases, so a very less compression, very less clearance volume means very high compression ratio. If we make the reverse, let's say I do it here.
No.
So let's say I will do it here. OK.
Let's say the Piston is at this point, OK?
And we have a clearance volume of this much. OK, what will happen? We will get a.
bc.
We see higher compression ratio will reduce; it means piston is not reaching at the extreme end, so it is it is it can able to compress, but it cannot compress because of the mechanical limitation, so we are making lower compression ratio engine, so compression ratio directly influences your thermal efficiency.
Combustion efficiency, performance, combustion emissions, everything. OK, so now I think it's clear we take some questions and then we'll add towards the...
M.
Next one, okay.
Okay, so that's good, that's good.
Okay. Sachin is asking to take the questions over here. Okay. So we'll take questions right now. What you have told in the last class, we first go through the last class. Okay.
No, don't do it.
Okay, so...
He has asked that how fuel gets burnt in the diesel engine. OK, so now imagine in the petrol engine, air and fuel, your petrol and air both simultaneously mixes and go to the engine. That's the reason why you cannot compress the engine to a very high extent.
okay why because the fuel will get compressed and get auto ignited okay but in diesel engine we are compressing the air and air is highly compressible whatever you will compress you can compress it but you cannot compress the
Fuel, that's the reason why diesel engine has compression ratio up to 12, sorry up to 16, 18, even if some cases in biodiesel cases it is going up to twenty-two, but in case of petrol engine you can hardly get the compression ratio of nine, 10, 11.
Even if it's a GUDDI engine, then you can reach up to 12. I'll tell you, okay, I'll tell you. Okay, so high pressure fuel injector inject to the cylinder, correct, and to control the air flow based on throttle. No, no, no, don't confuse about the throttle. In my PhD, I developed a throttle-less engine.
So, how I have developed a throttle-less engine? If you can go to my Google Scholar profile and check my publications, you can read the concept how I have developed a throttle-less engine. The objective is to the air and fuel will come into the okay in the diesel engine. Do you think that there is a throttle?
No diesel engine air is coming unrestrictedly; only the load is controlled by injecting the amount of fuel injection. Are you getting me or not?
OK, so your concept should be clear first.
So, the throttle is used to control the air in case of diesel. If it's a petrol engine, it is used to control the air and fuel both. But this is also not true for the petrol or spark engine because why? In GUDDI engine.
gasoline direct injection. I just now I tell this concept. I just teach you through very excellent animations and very excellent slides that what are the advanced technology currently being running in the market. OK, you will get the point. OK, why SME principle we cannot.
do okay why the same principle right properly man why the same principle we cannot do for petrol engine of course we cannot do the petrol engine because in petrol engine air and fuel both is getting compressed so petrol will get ignited okay air use whatever you can you can do compressed it okay
So, next is why the difference is required in petrol and diesel engine structure? Correct. Why? Why we need the different type of structure in petrol engine and why we need there is a different structure for the diesel engine? I think it's a self-explanatory.
Question answered, you should have now you can tell me, you only tell me why, because in diesel engine we need highly atomized fuel to be injected into the engine cylinder.
So, to get the highly atomized in micron level dimension of fuel, we need extreme high pressure. You know, the CRDI Common Rail Diesel Injection Engine works at the pressure of 1600 bar.
Right now, you are at the atmosphere, so the atmospheric pressure is exerting on our head, so the pressure is hardly at one bar.
The diesel engine operating at 1600 bar 1600 means it is 16 times higher than the atmospheric pressure, so you can imagine how high pressure it is impacting on the.
Fuel to get it atomized, so that it will be injected into the cylinder and extract the energy from the air molecule and get the explosion.
Work at 1600 bar, if we put your hand also, it will cut away because the velocity is extremely high. To maintain that pressure, we need a lower pump, low pressure pump, then a high pressure pump, then another high pressure pump, and the mechanical integrity should be strong enough.
to get it highly strength so that the diesel should work at very high pressure. So mechanical integrity, complexity increases. But in petrol this is not like this. Petrol engine works at three bar, only three times higher than the atmospheric pressure. Are you getting me? So that is the reason why I am saying
In petrol engine, we don't need that much, and also petrol is volatile and the compression ratio is very less, also very less, like 9, 10, so we don't need to mechanically strength, so you can see in the diesel engine you can hear the noise, but in petrol engine you will not hear any noise while you are sitting, or if you are driving a city drive.
Then I would recommend for you to go for a petrol engine. If your job is to go from, let's say, from Pilani to Delhi, Pilani to Jaipur, let's say 200, 300 kilometers, I will recommend for a diesel engine, and that's where you can also petrol engine part very high, and diesel engine means...
I mean, the speed is very well, less also, okay?
OK, so why we cannot bring same principle? OK, this is and so why cannot use instead of diesel in compression petrol? That is, you know, the petrol self ignition temperature is very less, it will get self ignited and knocking will occur, detonation will occur, pre ignition will occur, you will find very complexity.
OK, and now, Sachin, sir, good. OK, question. I, if you have Sachin, edit out, then please let me know: during such some states, how does the piston move from top to bottom? This is an excellent question. MANMOHAN SINGH has.
Rest an excellent question that, sir, you told that the piston will come from the top dead center to bottom dead center, but you have not told that how?
OK, this is done by a self-cutter.
Initially, as there is no power available in the system, what you are doing? You are just waking up in the morning and going to the office, just take the key and then just start it and you just push it. If a very sophisticated car, auto self start like this, you just...
Push what we are doing, but just whenever it is starting initially, there is a starter motor. That starter motor is allowed to rotate first 7 to 8 cycles, let's say 15 to 16 cycles, so piston will up and down will takes place after a certain time.
The piston, it is a self-propulsion vehicle, so it will develop its propulsion power, you can say, by self. It will not depend on a self-starter. Self-starter is used for a starting portion initially, just to make the...
To initiate the suction from compression, expansion, exhaust stroke. OK, why two-stroke is not better than four-stroke? Who told you two-stroke is not better? Have you seen the man? Man is a company, MANMOHAN. Have you seen the two-stroke engine of Man Energy Man Company? It is like...
I think it's a three-story building height, the two-stroke engines are like 30 feet height, a huge giant marine engines are two-stroke engines. Who told you two-stroke is not good? Two-stroke is good, but based on your applications, OK?
So, good, fly will, good, good, good, crank, Bala, ABHINANDH, ABHINANDH, good answer the questions, fly will correct. What is the use of the gear, this gear for OK, which gear? OK, OK.
This we are.
Compass on the skill.
Which get?
I think this is not a gear. This has maybe wrongly placed. The gear should not be there. OK, I just re work on this figure and we can get back to you while updating the final slide. Should work in this air as well. OK, please explain swirl, tumble, squeeze once again. OK, he's asking, Sachin is asking to explain swirl.
Swirl, you can see the swirl means, let's say this one swirl. OK, so horizontal axis rotation, you can see me. OK, the horizontal rotation of air is like this, so you can say this is a swirl, but tumble is like this one.
OK, means let us say your engine is standing like this, you have either air molecule can rotate according to the vertical axis or about the horizontal axis. OK, so whenever the air molecules have rotation of about the vertical axis, we can say it is swirl, and whenever the...
Means motion is like about the horizontal axis; you can say it like tumble, always say that the...
Swirl has a diminishing impact on the tumble. Why? Because swirl will move like this, tumble will move like this. The objective of tumble is to get a turning motion inside the cylinder, so that the air-fuel mixture should mix homogeneously and in a better way. OK, but swirl will try to...
Deteriorated. OK, if you can interested, then you can read the comments. It's working extensively. I worked with comments mostly in the fall, tumble, and the squeeze design, so...
It's good there.
Yes, the IDC and ODC just change, it's vertical, BDC, TDC just reverse it to like horizontally and whichever is closest to the crankshaft, so this is like IDC and whichever the furthest one, this is called as ODC. Can you please repeat compression ratio concept? Okay.
So, MANMOHAN is saying that the compression is OK, so let's say piston is at OK, let's say the piston is going upward.
The objective is to our objective is to let piston go as much as possible, so that it will try to compress the air and fuel, how that how much the compression you will get, how that much more temperature pressure will erase, and the thermal energy can be.
Lecture.
To the mechanical power, OK?
Objective is there is power, there is not power, there is energy. There is energy or you can say the enthalpy content that is available in the fuel. The objective is that your job is to extract that chemical energy. How you are going to extract? You have to compress it.
You have to compress it and extract the energy present in the carbon hydrogen bond and this exothermic reaction will release the energy. This energy will impact on the piston. Are you getting me? So our objective is to compress as much as possible. If it is a lower clearance volume means.
The piston will compress to a very high extent. If the clearance volume is large, means the compression is very less, so you will get a low compression ratio. It depends which one you want to take it, OK?
So, what will happen if we keep both whole same for inlet? OK, of course, scavenging efficiency will be less, your exhaust will not be come out of the cylinder, and just a second, I have to take a call.
Yeah, sorry, sorry for taking the call. Okay, so sir, can you please explain again the pressure different technology at intake and I will talk, I will talk about whenever I will talk about the turbo charging, I will definitely talk. I think it's clear now, so today we didn't get a.
Time to take the break. I'm sorry for that. So, we not take the break. We have only 30 minutes left, so let's visualize some interesting animations and see the what are the advance I say, because the next class I will bore you. I will make you very boring class. No, not boring means we go to the...
performance analysis, then do some numericals. Your fundamental will be more clear. So today is mostly like discussion getting into the engine because heart of the automobile is the engine. And it will sustain for the years and even if all the technology will come, so it will exist.
Good, OK, so.
Now, I will come to the advances in IC Engine, what is called as valve actuation.
So, if we, if we just see over here.
OK, so now just tell me.
I draw, try to draw very neat and clean, which is my dream, but I couldn't be able to do that. It is, I'm an on campus teacher here, so faculty here, so this online thing is not in my cup of tea, so we will highly teach online.
So please scope it. Okay. So this is the...
Mm.
TDC, this is the British, OK?
So, the valves are spring loaded, and it is like this, it is like this, OK, just for your reference time, and the spark plugs are there, so this is TDC and the BDC, OK?
Piston is coming from top dead center to don't tell that in a single cylinder we have two piston, no. This is that one time instant, this is that another time instant. OK, so you will see the whenever it is coming from TDC to BDC, the intake valve should open. How the intake valve should open?
Then, we first take the ideal case. OK, so this is the BDC, and here we will make the BDC. So, what will happen?
Okay.
It will come from TDC to BDC, so this one called as suction stroke. OK, I can write it like suction, so during the compression stroke again, this piston will move forward, so it will come from BDC to...
TDC.
Okay.
Up to this point, it is suction stroke.
Okay.
So, during the suction stroke, what is happening during the suction stroke? Your exhaust valve is closed.
If we closely look.
Your exhaust valve is closed.
And here, the spark plug is there.
And here it is, the intake, OK, because during the suction stroke.
In the suction stroke, only air and fuel is coming, so your valve is open. OK, so the valve opening duration should be start from TDC and it should be close at BDC, means it will start from TDC and close at BDC, so the IVO, the intake valve.
This period is called as IVO. What is IVO? IVO is.
Pintech wall opening.
Then, what will happen? Of course, the intake valve should be closed. What will happen?
Now, you can see.
Now, this valve is closed, the spark plugs are...
Make airtight over here, and this valve is closed.
Now, piston will go from...
BDC towards TDC by compressing the air plus let us say petrol.
Air and petrol or SI engine, so gasoline you can say, OK, and petrol is getting compressed, so this should be a closed chamber, of course, else how the pressure and temperature will build up over here, so the piston will come from BDC to BDC, so this one called as compression.
VDC to TDC. OK, this prior is IBO.
Integral opening and this one called as integral closing.
OK, this point, this point is integral closing. Now, nothing is occurring. OK, in this way, suction and compression is done. Now, again, piston is at the top dead center. Now, we will do another one, then again, piston has TDC. It has to come to BDC.
How it will come from this one, and it will come at this point. This stroke is called as expansion.
How it will occur? How it will occur? Expansion will occur once the piston is at the top. When there is a spark, the piston will, it will push the piston and piston will come downward, so it will have a expansion stroke. OK, then after that, what will happen? Exist stroke will occur. How the adjust stroke will occur and how it looks like?
Of course, it looks like this: your intake valve is concretely closed, your spark plug is there, and...
You can say this piston is pushing the all the exhaust after the combustion, whatever the unburned product that should be swipe away from this path.
So, this is the exhaust port.
Again, piston coming from BDC to TDC, so the somewhere here the exhaust valve.
Should open OK, and when it should close whenever the piston reaches a top point and all the exhaust product are swipe away from the cylinder, then here your intake valve exhaust valve should close at this point and the exhaust valve should close.
And this entire troop is called that.
Exist true, now you have clear that my valve timings should be like this, so if I will say how the valve timing should be, so you will understand, so your valve timing should be like you can say.
Intake valve open.
Intake valve close.
And then, at another section, exhaust valve open and exhaust valve close, this one occurring during suction and this one occurring at the exhaust, OK?
So, to play with this valve timing, we have employed this cam lobe. OK, this cam lobes are there, which is pushing the valves so that you will get a.
valve opening if it is a spring loaded so with respect to the crank angle how your piston how the valve open initially initially the spring takes time to means expanded so you
It looks like China should, OK?
It looks like this, okay?
Your piston should look like this, OK, and this is the lib.
In?
M, mostly tell in mm, not in kilometer or not in meter. OK, it is hardly it opens 6 mm or up to 10 mm maximum the opening means the opening of the closing position and this is the opening position, so if it is zero.
If it is zero, so it is maximum 10 M, it is open. OK, so how the profile looks like? Your profile looks like this, OK?
different.
The actual value profile, OK, the region of why.
You got the same profile for the intake and similar profile for the exhaust also, but now just tell me whenever you are driving, how many scenarios, how many sorry, how many situations you have faced? Let's say in the morning you just put a key on.
Do you need power? No, let's say you forget your means lunch tip in and you called your loved one to take your tip in from the home. You have not started your vehicle, but your vehicle is running, your engine is running.
It is giving power to the all the accessories of the vehicle, but you have not started. You got your TPN and then start for the office. Okay, what will happen? Let's say, just after coming up, let's say your home, you encountered an extreme high busy traffic again.
Vehicle you at stop there.
Then you just cleared the traffic and got into the highway. Now you have to...
Give power, and then you just put the accelerator pedal, and your vehicle should run very fast.
To reach the office in a particular time, what will happen? It is the objective of saying like this: your vehicle encounters different situations where it needs sometimes extreme high power, sometimes it needs to be in the ideal, sometimes it is cruising, sometimes it is accelerating.
So, for different type of situation, of course, we need to open, sometimes the valve should open like this, sometimes we need the valve time to open like this, sometimes we need to valve time open like this.
Or short times, we have to change the timing also, but can we do with this configuration? The straight answer is no, because of this conventional actuation mechanism, you can see this belt pulley diagram; everything is attached to a particular system, so you cannot.
Change the valve positions. OK, as you cannot change the valve position, as you are purchasing your vehicle from the day start to your day end, you have a constant volume profile for your entire life period.
But you need different situation, sometimes high power, sometimes slow. This is the limitation why, while you are going to purchase a car or a vehicle, you can see in your manual also, you can they have given this type of car, you are getting the.
Of course, you need higher torque, high power, but the power or torque is needed at a particular narrow zone, let's say 3000, 4000 only, you will get maximum torque and maximum power, but remaining period.
You have very less torque. Why? This is the limitation of IC engine. Also, this is the limitation why we have mechanical complexity or mechanical integrity in such a way that we cannot change the timings in the conventional vehicle. I am not saying, I am not saying that is the reason why we are throwing the...
Advanced IC engine concept, so...
We are not able to get this one, OK?
This is the limitation why the valve events should. Let's say I'll just tell me where should be my intake valve open. Now you are in a position to of course tell me, OK, at top dead center your intake valve should be open, but let's say it's a very...
Low speed.
L.
Clear.
What will happen, my intake valve?
Should open not at TDC, just for a higher air fuel mixture, it is opening.
Before the T dc, let us say this one 5 two.
N degree before.
Top dead center, got it.
But do you think at low speed I have to keep it this one less or higher? Let's say something like this.
Okay.
Let's say I this is for high school.
Let us say this one at 20 degree around before T dc. Why we are doing like this? Because at low speed, your cycle time is higher. You have simple amount of time means the cranks up, everything is rotate very slowly. So you have higher time.
So, you can close it very, means you can just open the intake valve just close to TDC, but in high speed, let's say engine is running at 6000 RPM, so frequent movement is taking place, very less time is available, so you have to open your intake valve opening here.
Because intake valve opens like this, so it is taking some crank angle just to open for a few millimeter only, later on it is spiking, OK?
Opening is like this, now you can see up to some crank angle it is opening only this side is lift.
This side is crank angle. So lift is very less but after this period you can see it is spiking but up to this point it is very slowly opening. So you have to open such a very before so that air like this also you have to close. Okay I will do another figure to explain.
Your intake valve should not close at exactly BDC; instead, close it at somewhere after the BDC, let's say 20 degree around 20 degree after the.
B this will be this.
Okay, this one V dc for 20 degree absorb. So what is my total IVO? Sorry, intake valve opening timing. So interval open somewhere here, let's say 20 degree, 20 degree. So here is 180 degree. So 180 degree plus 20 degree plus 20 degree.
The total is 240 degree. I am just opening the intake valve just for bit of scavenging. OK.
But this is also conventional, so if you see this diagram I have just explained, let's say it is a high-speed engine, you have to...
Go for yearly opening, if it's a low speed engine, you have to go for a late IPO. But do you think your car is doing this one? No. Your car is always giving one value. Let's say if the OEM have fixed this much means you will get this value in your entire life.
Like this, you can just check with other also. OK, what will be my different like high speed, late closing, low speed, early closing. So hence the valve events needs to be optimized for different engine speed and also I can say different load.
Conditions, okay, for better understanding, okay, so that's the reason why the companies have now started VVA variable ball actuation, but we can do based on our demand.
Can I, instead of this, can I open it like this?
Can I change the intake valve opening timing and interval closing timing, because this is the crank angle, so can I shift this one? Can I shift to this one?
Yes, we can do that. If we will implement this technology, advanced technology, PBI is not advanced, I will tell you which one is advanced, which one is again conventional. Okay, so this one, one is semi flexible, one is fully flexible. So, okay, so these are the eight techniques
We can.
Every technology has some advantages and disadvantages. Someone can open yearly, someone can close late, someone can some means can be open yearly, some means interval can be late. So there are 8 techniques of course, 4 bulbs, so combination is 8. So everyone has advantages, disadvantages.
For the scavenging, internal excess gas circulation for the better performance, lower performance, we implement these strategies using the intake valve, really closing the intake valve late. So, these are the strategies. So, what is this semi-flexible? Why this is name is semi-flexible? The semi-flexible means...
You can, let's say your intake valve opening is like this. Can I change it to like this? Yes, you can.
This is the beauty of semi-flexible VBA system by designing the mechanical system in such a way that you can.
To get either valve smaller valve like this, I told you in the traffic, you don't need much power, but your valve opening area is much higher. Of course, it will produce higher power; your fuel consumption will be higher if, in that case, if you can reduce the...
Val leap, a small air will come and your small fuel will burn, so means less fuel will burn, so you will get a better mileage. So, this type of technique is implemented by using either reducing the lift or shifting the timing, OK?
Means you have just cam fetch, you have just cam phasing and somewhere is cam switching. OK, these are two techniques. What is that? You can see in our let's say Toyota VBTI or if it's a BMW, so BMW Valtronics or the VBTI Variable Vault Technology Intelligence.
Toyota system, you can see one intake volume profile is like this, and then another is like this by using some technology. OK, so the cam phasing technology mostly used to phasing means we have just changed phasing. What is the name?
Caging indicators, caging chains.
Or phaging shift, first page, second page means let's just let's say first after lunch, there is second page before lunch, it's a first page, so like let's just see that you are not doing some work in the first page, you are doing the second page, you are just shifting the workload like this, we are...
Why I told you the benefits I have already told you? Don't ask me that why you want to open it over here. Early intake valve, late intake valve. I just told you the demerits over there, low speed, high speed, okay? You need different opening and closing duration, okay? So phase change means the angle change.
OK, angle change means I will tell you whose angle. OK, now we can see now the cam lobe, whenever it is the big, the like this, it has designed like this, whenever this this end touches this one, you will get a pick like this.
OK, this peak, whenever this one touches this portion, you will get a feel like this, you will get a maximum one. So, how you are going to switch it to this portion? Of course, if instead of touching at this one, if I can just by implementing any technology over here.
And change its page. OK, change its page means can I just give a angle or rotation, a small angle to this one, so that the entire camp shaft is?
Rotated by a small angle, like this small angle, then what will happen initially, which the big lobe was touching this one at a particular instant? Now it has changed; you can see from the figure itself. OK, now how we are doing? We are just changing this position, then your entire duration will...
OK, it's better to just clearly look this animation.
This is the while timing cam phasing mechanism. You can see our objective is to just give this on the cam phaser. OK, it has just gone away. OK, now we just push the what oil now from here. Now what will happen? It will occupy.
At this, OK, now you can see it has given a rotation. Again, we do not we want to shift again, so what will happen? It will release the...
Do it so again, it will enter this portion and it was and it was changed. OK, now how it is doing? You can see objective is to give a rotation like this, give a rotation like this, just a small angle change. If it will change means it will change the cam lobe.
And you will get different technology. I think it is clear. OK, so now what will happen?
We just cam fencing we just completed and now cam switching. Have you heard about this car I VIVEK?
Variable technology, electronic control, Honda, Honda Valt, this is very common and very old. Also, this is camp paging, you will get, as I told you, you will get different camp.
Profile. OK, how you are getting? You can say POAC dual overhead cam VTech, so Honda patented technology VVT lift electronic control lift electronic control VTech. OK, variable.
Lift a variable, just to see, lift the TCOK system uses a cam switching technique to vary objective is like that, so it will vary the valve. How instead of putting a one cab low, they have put a small cam low also, OK?
Are you getting me or not?
If after all this portion is touching the valve and pushing and you are getting a lift up like this. If in case I will reduce this one, of course it will push a smaller height. That's the key concept. How it is doing? You can just loop this animation, everything will be clear.
You can see here.
Now this is at the low speed. You can see this has been not been used. This it is ideal only the small lobe is operating. You can see the small lobe is operating. OK, but at high speed now there is a hydraulic litch. If I connect everything then the big lobe will.
Push the valve, you can see the hydraulic pin is there, then you can see this is the high speed.
If you want, then I can just play it again for your picture. So just see that during the low speed, what will happen? Our objective is to use the smaller cam loop. This smaller cam loop. So this is you can see it has not been coupled. It is ideal. Even if it is pushing, it is not impacting the...
Valves OK, but during the high speed, what will happen? The issue will give the command and the hydraulic pin will attach here, then the entire everything has been coupled. You can see it has no role, only the big one is pushing and your valve is operating.
OK, the objective is to get that high speed. This mechanism is clear, OK?
So I think.
We have cleared most of the things, so we'll go back and take any questions if you have any doubt, and then...
Okay, PATIL, I assume this is the use of that gear. Which gear you are saying?
OK, what is the OK? OK, so you are OK, so Patil is asking, no, no, no, don't confuse this gear is in the crankshaft. OK, this gear is in the crank camshaft and this one the crankshaft. I'll just check and let you know that that's OK.
So, it depends on different OEMs configuration, so this is not the right answer. So, it will not be at the middle of correct, correct. It should not be at the middle of the crankshaft if it's a timing here, so I don't think so. Okay, so Abhinandana has addressed this one, so PATIL is at least try to address that is a very good thing.
Um...
Any other questions you have asked me? Can I please repeat compression? I have already addressed what will happen if we keep both?
OK, let me end up with a different technology at the area to now.
OK, so I think it's now you have cleared, so we actually have not covered most of the things, but in the next class we'll just quickly see that how the VCR technology, while running the engine, you can change the variable compression ratio engine, variable compression ratio technology, and then opposed piston technology.
and then the turbo technology, then the CRDI technology, then the camless technology, we'll see some advanced technologies that have been implemented to the internal combustion engine just to enhance the efficiency. Okay, and later we'll go to the fundamentals of this part and then later we'll focus to the emission part.
What are the emissions, how it is impacting, which technology is used to reduce the emissions? Okay, the engine part will going to be very interesting. Okay, any doubt you just ping me or pitch me also, or any doubt also in your engine side, any doubt you can just tell me will have a very.
Good discussion. OK, so just a small announcement that I have already told you regarding the which test it has scheduled at September 1st, 8 A.m. You will be, you can open it and start your exam and it has.
Given some time duration, I think 40 or 60 minutes, and for it will be open for one.
One week, okay? I think it's open, I think it's clear to all, all the best for your test. It's very simple, just try it and we'll see the hard question in the mid-semester. I'll now prepare, okay? Syllabus for mid-semester, syllabus I'll tell you because right now.
Up to engine, it will come in quiz: theory or numerical? In quiz, it is theory or theory, yeah, mostly theory, whatever you have just mostly engine will not come before engine, whatever sections we have covered, mostly introductory propulsion system, and I think Bala it is clear.
Sachin, good Sachin. Sachin is really doing good. It's active play but shooting. Yeah, AIMCQ. It is multiple type questions, so you can address. I think you have communicated through. Okay.
I think you can conclude it, and have a good day. Thank you. Thank you, everyone. I hope you have enjoyed.

Srinibas Tripathy . stopped transcription