Embedded System Design (Merged - AEZG512AELZG512)(S1-26)-20260829_103107-Meeting Recording

August 29, 2026, 5:01AM

2h 8m 12s

KENDRE SANGMESHWAR SHANKARRAO . started transcription

KENDRE SANGMESHWAR SHANKARRAO .   0:06
Yeah, good morning to all. Welcome to the sixth session of embedded system design. In previous session, we have started the assembly programming part. So before that one, we have gone through the what is the assembly need of assembly programming. And we have gone through that.
Part of what is the instruction set, and then we'll go again, Parsley instruction format and directives assembler directive, and then we'll start actual your instruction set for this one, so...
We have seen we are having the label part and mnemonic, operand one, operand two, operand three, and then you are having comment. All these are, it is not all a part of this instruction is compulsory. Some parts are optional are there. OK, so what I have told in previous, do you remember anyone?
I will make the allow the people to.
Use the mic, so I allowed.
So, what is the use of label? As I have told, and label is given the symbolic which identifies the instruction or data location in simple language. Label is nothing but the it is a meaningful symbolic name to your address in a program. OK, so it it.
location or a data location. So you can give the level for the instruction location means your program memory location or data location means your RAM memory location. You can do that part. Then we have seen what is a mnemonic. Mnemonic very important mnemonic tells the.

DEEPIKA P .   1:50
I.

KENDRE SANGMESHWAR SHANKARRAO .   1:50
Uh, 6 processors. OK, please mute.
DEEPIKA.
Okay, so what is the mnemonic? Mnemonics tells the processor, specify the processor operation, or you can say mnemonics is the necessary processor about the operation to perform. It tells the processor what operation to perform. This is a human readable name for your processor instruction, such as add.
Substract, move, LDR, load the data from the memo location into register, store, store the content of register on memo location. All these are called as a mnemonics. Then we have gone through the operand. So operand is the your operand one, operand two, operand three. This tells the instruction or the data to operate.
Operate on operands; it is given operands can represent several kind of information, so you are having the instructions such as you are having the instruction such as you are having a.
You're having instruction as add instruction is there add then R0 comma R1 comma R2. So add is the your mnemonic. It tells the processor you have to do the add operation. And these are 0, R1 and R2. These are the your.
Operands are can be represent several means these are the registers. So here it will represent R 0 is the destination register. This is the first source. This is the second source. Or sometime you are having image data also. You are giving the constant directly in the instruction sheet.
Instruction itself, so you are giving this one ad.
R0 comma R1 comma hash 5U. So hash this is immediate. It indicate that this is immediate value. Add the value of add the content of R1 with the file and store the result into R0 register.
R 0 register. So here this is a destination, this is a source one, this is source two. So you maybe you are having the image that operand or maybe operand contain the data in terms of registers that register having the information regarding the your data. Everything is related to data. Sometime you are having such operands are there. We are going in detail regarding this one.
LDR load the content of memory location into the register R0 comma in a square bracket you are having a R1. So what is the R1? R1 is a register which contains the so it is in a square bracket means it act as a pointer so you are accessing the memory location.
So what is the content of R1 is nothing but the address of memory location. So it in point, it act as a pointer. So you can, you are accessing, so you are loading the content of memory location pointed by R1 into the R0 register. So R1 having the content that is nothing but it is address.
And at that address, what is the data available? That data we are copying into the R0 register. So this such type of your different types of operands are available. So we are we are we have seen that one that gives the information regarding this operand can describe the registers, can describe the constants.
Memory addresses, offset labels, all this combination is nothing but the your operand. If you understand this part, please type one into the chart, OK?
If you understand the operand, please type 1 into the chat. It's a very, very important part.
Then we have seen what is the assembler directive indirectly also we have gone through that one. So out of 64 only four to five peoples are giving the replies. Please give the reply. So what is the assembler directive? Assembler directives are now the processor instructions.
These are the not processor instruction. Keep in mind, this is not add, this is not your subtraction. Please keep in mind, this is a so ARM Cortex M processor does not execute this one. I have enabled the mic, DINESH.
I have enabled now, check.
Okay, so these are the operand which gives the information regarding the data, whether data is available in the register, whether it is a you are giving the data in the instruction itself, whether it is a address of the memory or it is the label, miss again is related to the address. So all this information given by the operand.
And then we move to the assembly directive. These are not.
We are going in detail, in the next session we are having that part LDR. So assembler directives are not the processor instructions, keep in mind. So these are very important, okay? In the Cortex exam, these are not executed by this one, but these are the command to the assembler saying that, so whether you have to start as a 32-bit value, you have to create.
You have to create a code section, you have to create a data section, reserve the memory, all these information given by the assembler directive to the assembler. Then comment, it is for documentation purpose as a human reader programmer intention, what is the assumption you can give in the comment.
And program commonly separate as a core section data section we have seen, so whatever the core section data section that that will be.
It will be like what happens.
That will be a.
You can make a select by using the part of this one.
Then you are having the, as I have told, only one section is not there. Multiple sections are available in the your memory, your code. So you are having the instructions which are available part of your program, and that is it is a part of your RAMA memory or flash memory variables.
stack, heap, all these are part of your SRIRAMAN memory. Then as I have told, depend upon the tools, what are the tools you are using. So these, your syntax are somewhat different. You are writing the programming, writing the mnemonics, operand, syntax, instructions.
And these are smartly varies with your tool chain for that. OK, so this is regarding the...
your part of format and then we have moved towards the assembler title. So this part I have taken in a previous session. So this area, so you can make a block of data, make a block of code, you can define this is a area, this is you are creating or selecting the code area or data area.
in that you are writing. Entry, entry point where program start execution. I have given a simple example of addition in assembly programming. I have done that part previously also. Then align, you can align the data or code memory towards the four byte boundary or two bytes boundary.
And I was saying dcb, dcb, okay, dc, and related to the floating point, these are used for defining the your number of bytes you are storing, you are allocating for the memory location. As I have told, in C language, you are writing directly INTA, okay, and then you are assigning some.
Your data to that variable, and then this is a compiler's responsibility to assign the memory location to that one. OK, and it reserves the space for that one. Now, when you go to the lower level language, it is your responsibility as a programmer's responsibility, so you have to...
Assign, allocate the memories for the your variable, so for that one you are having dcb, dcb, dc, dcb, dc, dfc, fecfdbd, dfc, fb, so you can allocate the number of locations, memory location, dcb, which means byte, 8 bits are allocated, W means upward.
16 D is double word, that is a 32 bit. It is a floating single precision folding double precision point. Space you can allocate as a block of memory, you can fill the data, you can give the equate numerical constant, you can give with the help of EQ, RN, instead of any register if you want to give the symbolic name you can use.
RN export symbol or any data you want to refer into the other programs for that one. So whatever you are using, so we have used that as under score under score vectors. This makes that symbol is visible to other object or models for that purpose or reset and we are used mostly in reset handling.
Import is a similar way. It is, you know, another language we have discussed is language if you want to import the data, so whatever the declare symbol as export you can import into your program. That one include gate, proc, miss the subroutine, end the subroutine. Last was that should be end and this is end of the your.
assembly source. So these are the important assembly directive with the help of this one you are writing the program. This is a related to the kill assembly directive. Keep in mind this is a we are discussing with the software which is related to the kill one. So it varies with the somewhat your varies with the your tool chain. OK.
Now we are going towards the actual instruction set. Now we'll go one by 1 instructions. We'll discuss that instruction. Then I will take one program based on some of the part of that one. Again, it is a very fast. It is not possible to complete all the instruction, all the programs in assembly, but I'm giving the introduction to assembly programming. Okay, keep in mind.
Otherwise, it is again, you can say as a 30 or 50% part of assemblies, you are having 50 sessions of that one. Okay, so first we are going for the data movement setup instruction. So we are going for the move instruction and MVN instruction.
So this move instruction as move is copy or you can call it as MOV, move, move a value, but it is not actually move, it is a copy of value. So you transfer the immediate or register derive value into a destination register wherein encoding permitted. So this is a data movement instruction. You can move, you can copy.
Any arbitrary data from one member location directly into the another one or one register to another or immediate value can move into the another one, so the immediate or register derived value you can move into the your destination register. This is a move. Now, what is the MVN? MVN is it is a similar to move, but what?
It produces the bitwise not of its operand.
It produces the bitwise not of operand, and that value is moved into the your destination, so it what it does it make the inversion not of that every bit not created, and then it again moved into the your this one, so these instruction do not it is a generally memory to memory copy instruction. Keep in mind.
Whatever the ARM instruction is called as a load store architecture, load store architecture. Okay, so all the operation, all the CPU performs the operation on the content of registers. So if you see the previous traditional some processor 8051, the 8051 controller or...
They directly work on register as well as memory, but this ARM processor always having the load store architecture means there is a separate instruction for loading the data from memory into the register, storing the data from register to memory, so all the operation.
performed by the CPU ARM CPU on the content of register. It does all these are on a content of register for that one. So what we have seen, all these are derived, all these are working with this one. So we have seen these two instruction move. So MV1 is a not.
top operation not with this will be used for masking the your data for that purpose it is used. It is not for general memory to memory per transfer as I have told only load and store instructions are used in that also load multiple instructions are there and store.
Then, operand and encoding restriction determine which constant can be represented directly, so you cannot go for directly all this one constant, so now every constant will not fit into the mu. OK, so this operand I need to control it, which determines what constant can be represented directly. OK.

YASHWANTH V H .   15:30
Master.

KENDRE SANGMESHWAR SHANKARRAO .   15:32
So you cannot give the 32 bit data into the MU instruction because your instruction size is also 32 bit. So you cannot give the 32 bit data into the MU. So we can go for that one. Different variations are there. Then your operand, as I have told, assembler may use a different instruction sequence or pseudo instruction mechanism for.
Large constant. If you are loading a 32 bit data, if you use a instruction to load the 32 bit, it will not a 32 bit because your data is 32 bit and for encoding the instruction again it will take a some bits, so it does not hold that 32 bit data into the instruction that is not valid. So for that one you want to use a.
load instruction, so that is called as a pseudo instructions are there. We are going in detail regarding that one. So with the help of this one, you can load the 32 bit constant value into the your register. You can move that one. So that will be available. So then a flag setting forms are there.
that will be also distinguished from non-flatting sets. So such instructions are available, such as you are having the instruction add. This is a simple add instruction, and you are having a add S instruction is there. So this S is the suffix which plays the very important role.
OK, for deciding the whether you want to update the flags or not. OK, so this yes, if you put a yes in suffix to the instruction, then processor condition flags are updated by this instruction. If you don't put the yes.
Suffix into the instruction, then it will not update the your conditional flag. Conditional flags, anyone NZCV negative 0 carry overflow. All these flags are not updated by this instruction, so this is update condition flags.
So, with the help of this, yes, your flag setting will be done. OK, so this this is called as a non-flag setting, not non-flag setting instruction. This you can call as a flag setting instruction. So, due to this, yes, you can set the flag of the your conditions. OK.
So this will be there. So if you are having carries generated, if you use only S and if you.
If you use AS, then carry flag is not set by that addition. If you use ADD, yes, then carry flag is set by the addition. It will be reflected in a carry flag. OK, this is not reflected in a carry flag. These such instructions are there. I will take this example and I will show with the help of that one.
So this flags is very important, okay, to determine the your condition of the system. So how you can control the your complete flow of your code with the help of these instructions. What it any doubt up to this one? Any doubt up to this one?
Then...
Clear this page, OK?
Is it clear? Are you there? Please confirm.
Then you are having the data movement, which is also important. The first steps you have to prepare the operand for arithmetic and addressing, and then you can do it. OK, 532 variable cannot be used in move instruction. All the instructions of arm is a 32 bit size, size is a 32 bit instruction.
So in a ARM, this ARM Cortex M4, you are having two types of instruction. That is called as ARM 16, thumb 16 bit, and thumb 32 bit instructions are there. If you use a 32 bit data, and if you want to move into the instruction, so instruction also you are converting into your code, operand, operation codes. So it will take some bytes.
and your data is also 32, it goes beyond 32. So you cannot move this 32 value, this immediate value into your move instruction. So there is a 16 bit you can move, okay, such size you can, so that is encoding representation is there.
Whether you have to go for encoding, that means whether 255, if you are having range of 8 bit data, yes, you can use 8 bit data for move instruction directly. You can use, but 32 bit constant, what will be generated, whatever the encoded instruction generated, that is larger than your 32 bit.
So always this will be done. Otherwise you have to use another options are there so you can change and all these one facilities are available. Okay, got it.
Then, uh, we are having the part.
So these are the loading constant into the registers. Such instructions are available. All these are related to constant, means immediate data you can move. So as I have told, now Aleph is also asked related to that one. So move Rd. is a destination register, and then you are having a hash that indicate.
Whatever the data here, it is a immediate data, so you can move the immediate 8 bit data. 8 bit data means 0 to 255. You can move into the your destination register. MRUNMAI means it inverse bit wise inverse the your data and inverted data is moved into the destination register.
OK, so such instructions are available, which will be more into the your systems, unless it will be more into the your data.
Then you are having the MOV Phi T is there and MOV Phi West is there. Phi West means it is used for immediate 16 bit data. You can move MOV T move the 16 bit immediate value top half of this one will be moved. You are giving a 32.
16 to 31 data you can move into the your Rd. register bottom half remain unaltered so bottom half if you are giving it will not use for this one so you are having the instructions such as.
And here you are having, it will move the zero to 15 and top up up, it is not moved. So you are having the instructions such as you are having the R0 content as 0X00001234. So with the help of this instruction, 1234 only it is moved into the R0.
Okay, so this 0 to 15, 0 to 15 is 16 bit data is moved into the your destination register and here higher means 16 to 31 that data will be moving to the your your move VT this instruction, but it so for that one 16 and 16 bit you can move move top.
T means top, move top and you are having the W move the top value and move wide, W means wide, that is your 0 to 15 data you can move into the register. And by combining you are having the effect of moving the 32 bit data. But this such.
instructions are not there, but you can make construct, you can constant 30 bit, 32 bit constant by using these two instructions. You can use these two instruction and you can move a complete 32 bit value. I have got it, but cannot maximum 16 bit, it will be at a time it will move the data.
Okay, then very important instructions are coming. So as we have seen, ARM is a advanced risk machine. Okay, advanced risk is a risk miss. What is a risk? Anyone?
We have seen in the first sessions, what doing by this, yeah, YASHWANTH.
Yes, we have seen what is a CISC, what is a RISC is also. Yes, you want correct. Reduce instructions and computer or computing. So having less number of instruction and most of the instructions are executed in a single cycle. So if you are having a two instruction, it will take more time to execute that one. Two instruction, 2 cycles are there. Instead of that one,
Why should not go for the one? OK, or any combination is available in the instruction which will move the data? Yes, fine. Simple move is not possible. You cannot move 32-bit docker because machine instruction has limited encoding space. OK, you cannot move this one.
More.
R0 comma hash image data 0X12345678. This is your 32 bit data. You cannot move because this is the 32 bit data and this after encoding it should be this instruction should becomes 32 bit.
Okay, so your encoding space is limited, so you cannot do this one. So instead of that one, you can move using this one, you can move. One, two, three, 4, you can move by using MVT, top, and 5, 6, 7, 8, you can move by using MUW. In this way, you can completely move the 32-bit data into the any register. That is possible.
Another way is there by using the LDR instruction, OK?
By using the LDR instruction, you can move the dot. OK, this is the problem, Chris.
By using the LDR instruction, also you can move the data. OK, here you are able to see LDR, RT is your target registers, and then you are having an equal sign, then you are having immediate data, so it is equivalent to move, but this is a assembler assistor constant loading.
This is a assembler assisted constant loading. This is not a instruction. This is not a instruction. OK, that is a call as a assembler pseudo instruction. These are called as a assembler pseudo instruction. So, what do you mean by this one? So, here we are given...
You can move equivalent. This is called as a pseudo instruction, so you can move the this is a 8 bit or you can move directly 32 bit. OK, so these are called as a pseudo instruction. What do you know? It is a look like your LDR memory instruction. We'll see later on memory instruction, but here we are putting a equal and you are putting a constant value.
You are putting a constant value, which forms a pseudo instruction. What do you mean by pseudo instruction? Anyone?
Anyone? Those are from a electronics background and those have worked on S32 or ARM processors. Maybe they're familiar with this terminology for doing my pseudo instruction. I have enabled the microphone also.
Anyone, please give the reply.
Yes, you will.
Prashant, LOKESH, anyone?
I forgot the names. Those are familiar with this one.
Ganesh, BALA GANESH.
What do you mean by pseudo instruction?
Venkat, Venkat.
SUHAS.
Anyone? SIVA?
This one day, are you from mechanical or electronics?
Are you there?
Yes or no? Please give reply as a thumb.
Yes, okay, KARTHIK again, do you know this one?
Yeah, LOKESH, thank you.
So these are called as a pseudo instruction. Pseudo means.
Not directly correspond to machine. OK, no problem. OK, no, no. I'm explaining that one. So it is a look like a similar to it is look like short. It's not not. So it what what do you mean by pseudo instruction? So conceptually it means that assembler.
Please arrange for this constant, which is you are placing as a 32 bit into the R register. Use a suitable instruction sequence. You are giving the requesting the assembler with the help of this instruction. Please use the suitable instruction set of.
ARM means your ARM Cortex M4 instruction set of instruction to implement this thirty-two bit data moving sequence, so this instruction is converted into a similar by two instructions, and this will be then.
used for transferring the 32-bit data. So what this is a pseudo instruction which will be converted into real load instruction from the memo location and then assembler assign that one and it will convert into that one. So we'll see that later on how this will be converted with the help of that one. Okay, so this is a.
set of instructions are available which will be used. So what do you mean by pseudo instruction in ARM? So this is a important, very important concept of the pseudo instruction. Pseudo instruction can be written in assembly program, but it is not directly supported as a machine instruction by the processor. This is a very, very important.
This is not directly supported by the processor, so these are given the instruction to the assembler. OK, so assemblers assembly file you are writing, you are giving in that you are having a pseudo instruction. Assembler decide how to implement that sudo instruction. It is assembler's responsibility. Keep in mind.
Assembler decide how to implement that pseudo instruction. One or more real machine instructions are used for implementing that pseudo instruction. Keep it so it is a provided mainly for the programmer convenience and improve the code reliability.
Got it? What is the use of pseudo instruction for the programmer's convenience purpose and for improve the code reliability purpose? These pseudo instructions are used during the assembly. A similar translate this pseudo instruction into one or more.
Sewel machine instruction.
So it will translate, it will translate this pseudo instruction into the your actual machine instruction. The source shows LDR and ADR as important pseudo instruction example. We'll see that that one instruction. I will give that one example in the one program. So all this one within one program I will explain it, wait it for that one.
LDR can be used as a pseudo instruction to replace an immediate constant or an address into the your register. So 32 bit if you want to move the immediate data or 32 bit address if you want to move into the your any register, you cannot use move instruction for that one you are having LDR.
or you are having ADR, another instruction is there which will be used for that purpose. The actual machine instruction generated depend upon value being loaded and whether it can be represented directly or not. It is a assembler, it will decide how it will be converted that one.
Okay, keep in mind. So therefore, one assembly source statement does not necessarily correspond to one machine instruction. Keep in mind. So one assembly instruction not convert into one instruction, but if you are using pseudo instruction, maybe you are having a 2 instruction or one instruction, multiple instructions are.
They are available, OK?
So, if you are moving towards the one, so such instruction is called as a LDR. So, there is a 2 instruction available: one is an actual machine instruction available, that is called as a LDR instruction also, and you are having a pseudo LDR instruction. What is the difference we have to discuss? LDR means load the...
your memory contained into the register. LD means load. R means register. Load the content of memory location into the register. Content of memory location into the register. This is a directly you are having this load register and transfer the data from memory into register. This is a normal.
instruction that is a machine instruction which will be executed. So this is a example of real memory access instruction. You are accessing the memory as previously also I have told LDR, R1, comma, R0. So whatever the R0, R0 is register, register content, 32 bit, 32 bit register.
R0 contains the memory address and processor loads the value stored into this memo location into the R1, so this is an actual, you can say, as a MHETRE or real instruction which is supported by the ARM Cortex M4 processor, but what is the difference between?
Your pseudo-instruction and the your.
Actually instruction. So here, LDR can also appear as a pseudo instruction when there is a notation, equal and notation. If you are using equal notation into that instruction, if you are using that instruction, you want to consider as a pseudo instruction. What is the instruction? LDR.
R2 comma equal sign and then you are having a 32 bit value any constant or any value you are giving any value you are giving so that is a 32 bit value you are max 32 bit value equal sign and if you are giving 32 value then this LDR is a not a.
Native machine instruction. This LDR is a pseudo instruction. OK, so this LDR comma your destination register comma equal to constant. So this is not a general microprocessor supported instruction. It is a pseudo instruction. OK.
If you got it, so here intention is to place this constant value into the R2 register and this assembler distinguish assembler distinguish to form from the operand system syntax by using the operand syntax, so it is equal then 32 bit value here you are having directly your data in memo location.
You're having a member location, it pointer, it pointer, R0 register pointer, the address of the memo location where data is available, that data is moving to the your LDR. If you understand what is the difference between pseudo LDR and real instruction LDR, please type 2 into the chat. Now we are going for the.
Another pseudo instruction.
Another instruction is called as the LDR instruction. Out of I think pretty few participants are there. 59 participants are there.
Yeah, Chandan Kumar.

CHANDAN KUMAR PAUL .   35:47
Hello, sir, for the pseudo instruction, that is the constant value. This constant value also the store in a single register, that register will be which one?

KENDRE SANGMESHWAR SHANKARRAO .   35:48
Yeah.
Yes.
So, this is a R2 register; this register is your internal register, R02; you are having R02 R15, so you are having total 16 registers are there, so it is internal processor register.

CHANDAN KUMAR PAUL .   36:03
Huh, yes.

KENDRE SANGMESHWAR SHANKARRAO .   36:11
What is under?

CHANDAN KUMAR PAUL .   36:11
Okay, okay, yes, yes.

KENDRE SANGMESHWAR SHANKARRAO .   36:14
Yeah, so this is R0 is also internal register, but it is having holding the address of the memo location. When R0 is also having content, but if you put in a square bracket, it indicate that R0 it act as a pointer. What is it indicate? R0 contained the memo.
Contain the memory location, so you have to copy the address here, load means you have to load the content of this memory location which is pointed by R 0 into the R1 in this way, got it?

CHANDAN KUMAR PAUL .   36:47
Yes, it is.

DHANUSH S .   36:49
Note the address for...

KENDRE SANGMESHWAR SHANKARRAO .   36:49
Okay.

DHANUSH S .   36:52
Address of R.
It loads the address of R0 to R.

KENDRE SANGMESHWAR SHANKARRAO .   36:57
No, no, no. Content of this memo location, it loads. R 0 content is nothing but it is a memory location. And what is the memo location? In that memo location, whatever the content available, it will be loading to the R1. Okay, I will go in detail. We are going, I'm taking big examples also later on. Okay.
So this is your LDR instruction. Now you are going for the ADR instruction. These 2 are the pseudo instruction. So how we can differentiate the real instruction and how we can instruction the pseudo instruction related to LDR. So in ADR instruction, this is a your pseudo instruction. ADR is a.
purely pseudo instruction. In LDR we are having complexity because LDR may be your native instruction or it may be pseudo instruction. For that one this is easy. So if you are having equal sign in the your operand part and then you are having 32 bit data, this is your pseudo instruction. Got it?
Then we are moving towards the ADR instruction. This is the pseudo instruction for loading the address. So by using these two type of pseudo instruction, you can load the 32-bit value into the your any register. Okay, so it is used to place a nearby memory address into the register. For this purpose, this will be used for nearby.
If you use have a 4K in the range of four kilobytes of memory location or two kilobytes of memory location, then you can use the EDR. It is also a similar generated address. It will do the calculation and that will be loaded into that one, so here...
Unlike the LDR sudo instruction 6, EDR does not use the equal sign. Here you don't require because EDR is a pseudo itself. It is a sudo instruction. So source explain that assembler translate this EDR into add or subtract operation using the PC program counter as a source operand.
Whenever EDR comes, assembler understands this is a pseudo instruction. I have to replace this EDR with the help of your PC and addition instruction or subtraction with content of your program counter. With this one, it will be converted. So EDR effectively calculate an address relative to the current program location.
and it's suitable when required label address which is lies within the applicable address range for that one. Mostly it is used in that area. So it is not, you cannot move the data from data memory into the program. Within a program memory you can use this EDR instruction.
You cannot use because I2X2000, you are having a data RAM, and if you want to copy the data from your data memory into the your any register, you cannot use this ADI. ADI should be within the range that will be again processor to that your your.
system to system it will be defined that one. So this is example add R1 comma R2 comma R3 add the content of R3 with the content of R2 and store the result in R1. So here it is giving LDR R4 comma loop. Loop means you are going to that memory location. Loop is nothing but the.
Label label is nothing but the you are having the instruction, so pseudo instructions are very important. Why you put important because these are the instruction which will be used for convenience purpose of programmers. So, when you write the data, when you write a peripheral code.
when you are giving the addresses of your register object for this one, this plays the very important role. So with the help of this example, I will explain in a coming example all this one. So first, we'll go for that one, and then we'll move towards this one. So first, we'll move, what is the big Indian?
And little Indian, so what is the Indianness? Anyone? What do you mean by Indianness?
Here it is mentioned, Indian means what?
So.
So, yeah, no one is asked, so here, what is the address? As I have told Arm Cortex, this address part, so this address part nearby address memory, OK, so this nearby address memory is nothing but the, as I have told, it is a 4K, 4K size. You can go represent.
offset in the range of 4K byte forward or backward. So if you are having instruction, you can refer up to 4K forward or 4K backward. That is this total range of this one. OK, so this much it will use ADR. Within this range, you can use ADR. Otherwise, you cannot use this per.
Miss above that one, then you have to move for LDR. So represent offset up to your 4KB forward or backward, forward or backward. Miss, you can say within 4K range, you can say within 4K range, this will be used. OK, yeah, as you are saying that one.
What is the big Indian or little Indian S? It is a order of byte, so important to understand the Indian, yes.
Batman, yeah.
Prathana.
Yeah, LSB at a lower mode, correct one. So here, this big Indian, little Indian is important. What is it?
Indianness, Indianness tells how the byte of multi-byte value are arranged at a consecutive memory locations. OK, so here you are having this is a big Indian sign, this is a little Indian sign, you are having the memory location, each memory location is 8 bit.
8 bit, 8 bit. So how these bytes are arranged? So your address, this is a base address, address goes on increasing. Okay, so this is a plus 1, plus 2, plus 3, so goes on increasing. How this data, if your multi-byte values available, how this multi-byte value is the...
Stored is stored into the memory that are arranged into the consecutive memory location that is called as a Indianness. So there is a two types of Indianness. It is called as a Bing Indianness and Little Indian. ARM Cortex M processor support both Bing Indian and Little N and by default Design Little N behavior is a.
By default, but if you want to change, you can use both the big end and little endians are mapping access will be supported by the processor. So, what is the so Indian as I have told byte order of the data element has with the has a multiple byte having then byte order you can store. What is a little Indian? Little Indian means...
Important, low order byte of the number is stored in a memory at the lowest address. So you are able, this is a byte zero, this is a byte one, byte two, byte three. So this lower byte will be stored at a lower memory location. So L, L, you have to understand this one.
Lower byte of the number is stored in the memory at a lower lowest address and higher order byte is stored at the highest address. This is a higher byte. Byte 3, it will be stored at a base address plus three. Then it is called as a little Indian. Little end is LSB to LSB. LSB meme location.
and MSB goes to the higher memo location. So that is called as a little Indian. If you understand the little Indian, please type one into the chat. Very few participants are giving reply. Out of 58, only three to four. What is the big Indian exact reverse of your?
Little Indian. So big Indian means higher order byte of the number is stored at the lowest address. Higher byte. This is a higher byte. It will be stored at the lowest number and lower order byte is stored at the highest memory location.
or highest address that is called as a your big Indian. So LSB 2 stored at the lowest address, little Indian, and you are having big N, most significant byte is stored at the lowest address that is called as a big Indian. Simplest wait for that one.
So, this Indianness plays a very important role. OK. Anyone, what is the use of Indianness? Why it is important? Why is Sachin is asked? That is the current time I am pushing that. Why Indianness is required?
Because...
Huh, MANMOHAN, that that is so if you are having the CHANDAN messages, Ethernet messages, TCP IP, if you are going for this protocol, TCP IP protocol, you are going for CHANDAN, you are diagnostic any protocol at that time, so TCP IP always they use a big Indianness, big Indian.
Okay, whatever the data you are getting that is in a big Indian format, CHANDAN messages are something that you are having a big Indian format. So you must know which type of format they are supported. And with the help of that support, that message, yeah, this DBCs are there. You can can signals and DBCs are there.
We can use this one to map purpose, mapping purpose, how to write the data in which order. Okay, so for that communication specification, you have to must. So software that allow the protocol, that protocol order is defined. So that protocol order is regardless of your CPU.
So whatever you are CPU using.
Regardless of that protocol order is defined. According to that one, it is your responsibility to support, you have to select which top of Indianness will be used and you can convert, you can convert the big Indian data which is coming from that protocol into little Indian and you can store the data. Okay, so for that it is not apply universally. Every processor access in this way, it is not applied.
So it is your, so if you are accessing the data, so you can configure whether it is a little end and bit in mapping, face the instruction, always in ARM Cortex, it is a little endian, then you can convert into the beginning and you can store the data in this way. Okay. Yeah.
Correct, correct. SHUBHAM, it is a by default is a little Indian. Got it? So that that purpose bit big big and little Indian plays the important role, so you have to you must know this part. OK, so now we are moving towards the another some set of instruction which is related to the.
your XPSR, XPSR is program status register. How we can access the program status register. OK, so how can change? We have seen in a privileged mode, you cannot change the content of your control register or.
your program status register. We have seen that in privileged mode only having facility, but on a privileged mode or non-privileged mode, you don't have a facility to change the content of your register. But if you are in a privileged mode.
And if you want to change the registers content, yes, you can change the register content for that one. You are having a special type of special purpose register access. You can use a special instruction for that one. So you are having such instruction that is called as a Ms. move from a special register to general purpose register.
You can copy the data from special purpose register to general purpose register. For that one, you are having this type of another. Also, you are having Ms. R move from general purpose register to special purpose register. If you want to set some of the bits OK of your Ms. R, you can use this one. So this is a.
You can read the special purpose register for any processing purpose, or if you want to write a special purpose register for that one in a privilege mode, you are having Ms. move the general purpose register content into this part. So, here you are able to see, so a PSR, IPSR, EPSR, we have seen, I think, in a third session.
related to ARM Cortex, or you can use XPSR to read all these three registers. By combining, you are having a 32-bit register. Only if you want to make the changes in MZCV, all these flags, so you are having the facility of this one. By using this one, you can make the changes in a particular bit of the status register.
or you want to read it for that purpose, you are having the special instructions are available. OK, so these instruction plays.
In unprivileged mode, you don't have a right access of your XPSR other field except NZCV. NZCV is having your right. Got it.
You can use this one to update the NZCV only condition flags you can update, but other part you cannot update, you cannot copy this one, you cannot do the other part, but condition flag means condition your flags will be changed in unprivileged mode. OK, thread unprivileged mode that will be there. OK.
Now we are moving towards the arithmetic instruction after going through the arithmetic instruction and then I will show one program in a kill how we can write that program for related to that one. So these are the and as I have told it is not possible to cover each and every instruction in detail and all this one. I will upload the.
ARM ARM architecture reference manual. OK, that is we are we are calling as ARM. OK, what is called it is a ARM.
Architectural reference manual. Architecture reference manual. So ARM. So I will upload that one. You can refer it if you want to go through that one. In that all the instruction related to all architecture it is given. OK, so you have to check whether it is applicable to ARM Code XM. It is given the short notes below that one.
And then you can refer this instruction for that purpose. OK, so now we are moving towards the arithmetic instruction, then we'll go for the logical instruction, bitwise operation instruction, then we'll move towards the comparison instruction, then we'll understand the stack, and then next session is related to the enter programming that.
Part we'll see, so here all these are the arithmetic instruction, add instruction, OK, add with a.
Uh, carry ADC miss add with carry. So here I have given the description on operation. Also here it is doing the add operation how it will perform. RN and Rd. Rd. means any destination register. R02 you are mostly R12 we are using. RN also N equal to 02 you are 12 means R02 R12 any register you can use.
And your operand to operand to miss any immediate value, or it may be content of register. In this way, you can you have to consider. So, Rd. is in a destination register, RN plus, so it does the operation of content of maybe it is a register or maybe it is immediate value.
It add with the RN content and store the result to RN. There is no carry will be updated. We'll see that later on. Add with carry, whatever the previous carry carry set, then carry plus carry plus up to plus RN content, then it will be stored into the Rd. So it is called as a add with the carry.
Add with a carry sub, subtract with a carry or borrow. That is a subtract, subtract with a borrow. RSB is there. This is a advantage. RS means reverse subtract. How it is in between this and this one, how can check. So RN minus.
your operand 2 in a reverse subtract Rd. minus here what is the operand 2 minus RN due to that one is called as a reverse subtract. Are you getting this one? So it is called as a reverse here. What is the difference here? RN minus operand 2 here operand 2 minus RN. So due to that one.
Reverse subtract. This is also support supported multiply.
So it is a multiply which results should be within 32 bit. It is called as a short multiplication. We go for long multiplication, sign multiplications also separate instructions are there available. So you are having a multiply, okay, RN into RM and you can all these you can store maximum results should be 32 bit.
In this way, only those multiplication having result of maximum 32 bit, you can use this multiply, simple multiply instruction. And if you are having a multiply of 32 bit by a multiply of 32 max is a 64. 64 bit result will be expected. At that time, you have to use another instruction.
We'll see that one. MLA multiply with the accumulate. So multiply RN with RN and add the content of RA register. So it is called as a multiply and accumulate. Store the result. Accumulate means addition previous data. So it is mostly used in a DSP operation. MLS multiply and subtract. We are having multiplication and subtract here addition.
your M subtraction. Yes, DIV means sign division.
UDIV unassigned division. Miss MSB bit is you have to consider here. Here MSB bit is not considered. Yes, set sign saturate. U set unassigned saturate overflow and all this one. So if you are using that part then it will be used for that purpose. OK.
So some instructions are simple and all these instructions are going for the 32-bit operation. So here it is given shift operation and it will be given the operation for unsigned, signed division. All these are going for that one, but don't go for divide by zero so they use the fault.
OK, you have to consider, so sign saturation, yes, sign saturation will be there, it saturate the value to the range of your N bit sign number, N bit sign number up to that one, you are having a saturation is there, so your result should be your number which will be sign of that one, and unsigned saturation, your range is from...
Zero to two raise to North -1, and if you are having saturation, you are having a range of your data is a if you are consider -2 raise to North -1 to the two raise to North -1 -1. In this way, this range will be for where result will be considered.
For saturated and unsaturated operation, OK?
Then you are having the, what is the impact of this on your condition flag, condition flag? This is very important. So if you are shifting or you are doing some operation, what is the S field? So NZCV, N means sign related most significant result bit will be checked.
and flags are updated upon this one. Ms. bit will be checked and if your bit is set, then it will be updated as a one. Zero, it indicates zero. Your result having a zero, then this bit will be set. Carry, carry represent unsigned carry information for addition and this the architectures no borrow interpretation for this abstraction.
This is a very important. If you are not taking borrow for that purpose, this carry also represent no borrow condition with the help of this carry. Carry is set. If you are, there is no borrow generate or no borrow taken. So taken from this one. So this is an important. Zero flag is easiest flag.
When result is 0, it will be set. Result is non-zero, it is a zero. Sign, you are having this carry also. Carry is also important. Carry means MSB bit. If it is MSB bit is 1, you are interpreting given number is a negative one. Okay, if you are doing sign arithmetic operation, all that's one for that purpose is used for this one.
Okay, and carry plays important role as I have told subtract you are having a subtraction. If you are doing the subtraction, if your carry is set to 1 and if you are doing the subtraction, carry is set to 1 means no borrow occur. Here you are getting it represents the unsigned carry info for addition and the architecture no borrow interpretation for the subtraction.
So carry equal to 1, and if you are doing the subtraction, in that subtraction, no borrow occurred. And if carry is 0, borrow is occurred. It is a reverse. Oh, if you are doing the unassigned carry operation, if carry is generated set, there is no carry, it is a zero. But here, if you are doing this abstraction,
No borrow occurred, it is set as one. If borrow occurred, it cleared, it occurs as a zero. Unsigned that that plays the important role of this one. Then if we flag is there, it is we call it a sign overflow flag. It is a completely different from your carry flag. It is a overflow flag. It interpret the bits that.
Having the two complements, if you are doing a sign arithmetic, and in that, if you answer exceed the sign range, OK, in 32 bit sign arithmetic, MSB bit represent the sign and remaining 0 to 31 bit gives the magnitude, and if you are...
Result goes beyond 31 bit at that time. It gives indicate that there is a overflow occur for sign arithmetic. It plays the important role. Carry and overflow are not same due to that one. I have told carry and overflow are not same that you have to understand this one.
So, processor does not instantly know whether you see variable is signed or unsigned. Software interpret the flag appropriately. Correct condition branches therefore depend upon the intended numerical representation. So, here when you want to go for conditional execution, only you have to check the condition and you have to.
Do the your operation, so for that one, this carry and all these flags are there, so for this one we'll take this bit, we'll take a quick five-minute break, and then I will show the part of an example of how you can use for this one, so many data processing instruction how the you can have a.
condition flag update facility available. If you add a yes suffix, as I have told, yes suffix, add yes, it requests the flag setting behavior wherever applicable. So if you put a yes suffix into the instruction, then if carry or any flag generated, it will update the flag behavior, otherwise it will not update.
So add and add S as I have told, add S can update the condition flag based on the result.
Carry generated, it saved the carry flag. Result is 0, it saved the zero flag. OK, if you are doing sign arithmetic and overflow occur, it saved the overflow flag. OK, so this will be.
Used by this, yes, but add what had yeah, so you add will not update the flag, it will give the reserve it, it will not update the flag, preserving the flag when later instruction depend upon the earlier, so if you are why this flags important if you are doing any branch operation based on previous condition.
Conditional execution, if you want to do then it is responsible to preserve the flag. OK, and some set of instructions are there. We are going in a latter part, compare and state instructions are there. These are intended to update only the condition information. They are not interested in a.
Giving the result, so it will not give the result, only it will update the flex for that one only. Comparance test instructions are there, so we'll go for related to that in a coming session. OK, so as I have told previous, we are having a multiplication. All these are 32 bit.
But if your result of multiplication is more than 32 bit, that may be because 32 multiplied by 32 bit, max result is 64 bit. Everyone is know that one. So if you are having a result is more than 64 bit, then what you have to do? So here you are having a facility, you are having special instructions are there.
So, here we are having sign multiplication.
Unassigned multiplication, MLA, Ms. these are available, yes, division, all these are available, which is all the 32 bit data, but for long, that that's got a short multiplication. For long multiplication, you are having a UMULL and here pair of registers are used for.
Holding the your sixty-four bit result, OK, pair of register, so you are having a RDL 0, RDL 0 means result or destination lower result register, RDHI, your it indicate destination higher.
byte result, it will be stored into RDH. So here you are having 32 multiplied by 32 bit. OK, you are doing unsigned multiplication, RN multiplied by R2, your LSB stored into this one, Ms. be stored into this register in this way. So is the sign unsigned multiplication, signed multiplication.
Unsigned multiply with the accumulate, sign multiply with accumulate, where result will be sixty-four bit, result will be sixty-four bit, result will be stored into the sixty-four bit by using this long multiply instruction. OK, so this will be related to arithmetical operations. OK.
new concept will be there. We'll see that is a barrel sifter. This is a hardware available in the ARM architecture. We'll see what is the use of barrel sifter. Based on this instruction, we'll check, and then we'll go for the example. So if you understand up to this one, please type 2 into the chat. We'll take a quick 5 minute break, and then we'll...
Come for this one.
Are you there?
Yeah, please confirm. Are you there?
By giving thumb or something.
Yeah, thank you.
Thank you. Okay, now we'll move towards the barrel sifters. So as I have given the hearing, it is a hardware assisted shifting. So before going to that one, I want to explain some part. So if you consider only 4-bit, okay, I'm writing here 4-bit data.
So, 0001.
001 OK.
So, I want to shift this data by one bit towards the left left shifting, so I want to shift this data.
By one bit, what happens? This one come here, I'm inserting zero, so what happens? Zero 010? It is a one bit left shifting.
Left shift.
Again, if you want bit left shift this data, so what happens? 1000.
Now, tell me the value: What is the value you are getting if you again one shift by this one, 1000 left shifting means what? Now, tell me here.
Anyone?
In binary, you are able to get this is nothing but the one, this will be your two, this will be your four, this will be your eight.
Ha, so what doing by left shifting? Left shifting is nothing but the if you left shift the data by one bit, two bit, 3 bit is nothing but the you are multiplying the data with two raised to how many bits you are left shifting?
So we are left here one bit if you want to shift, multiplying this data by how many bits you are left shifting. So you are multiplying only one bit left shifted. So preview original data, this one be 1 multiply by how many bits you are left shifting here.
Two bit you are safety, so means you are doing the multiplication by two raised to number of bit set in the reverse way if you go for right shift.
If you two go for right shift, what happens if you one bit if you write shift 0100?
This is 8, now we get a four. If you again write shift towards the one bit, 0010, you will get a two. If you again write shift, 001, you will get one. So what is the write shift? Right shifting means what? Please give the reply.
I have enabled the ID also.
Right shifting means what? What do you mean by? Yes, divide by two raised to number of bits. That number you are dividing by two raised to number of miss. Left shifting is nothing but the binary binary multiplication. Divide right shifting is divide. So this will be the.

SUHAS ANILRAO BOBADE .   1:11:11
Fiction.

KENDRE SANGMESHWAR SHANKARRAO .   1:11:27
You can do by using this shift operator we were done previously. OK, left shifting we are using a I think in a binary that part. OK, this part will be there, but here in ARM processor there is a dedicated hardware available. There is a circuit.
digital circuit or digital model available in the ARM processor that is called as a barrel shifter. This barrel shifter is capable of shifting or rotating. We call shift or rotate both. The binary word by specified a number of bits position effective.
So, we are not this. If you use a there is no barrel shift and then this shift operation will be done by the ALU. So, what we are doing, so in ARM processor they are having a separate hardware barrel shifter. OK, and this barrel shifter is used for shifting and rotating the one of the operand, so operand 2.
Mostly operand 2 that will be sifted by barrel sifter, and then it is given to the if you want to sift. If you don't want to, then there is no problem. OK, you can bypass the barrel sifter if you want to sift. Sifting is plays the important this architectural behavior of and sift operation will be there. So, as I have told a very important example.
So this is a second operand will be passed to the loop through the barrel sifter. And this barrel sifter will does the operation of sifting on the data. So simple, there is a such instructions are available which will be supported this barrel. If you want to use barrel sifter, you can use this one.
So, such examples are there, such as a so, as I have told, this is a parallel sifter concept, which a rapid multiplication of a register operant, you do rapid multiplication, or you can do a rapid division operation, logical and arithmetic.
and rotation will be possible. Sift operations are useful for multiplication and division as I have taken previous example. So if you are shifting towards the left it is nothing but the multiplication by power of 2 where numerical your interpretation permit and if you are division it is divided by.
Power of two, so I have explained this part in a previous example by given, so there is a 2 fundamental packaging and packaging bits are there, and you can use this shifting, carry forward, flag setting for these forms with the help of this register very effectively.
And for that one, you are having a some set of instruction which is related to the barrel shifter, which will be added with your main instruction. LSL and LSR. LSL means left logical shift left means logical. This is called as logical.
Logical shift left LSR, logical shift right. These instructions are used for shifting bits towards the left position or towards the right position. So here important you have to understand shift.
Ting bit forward more if bit position will be there, forward was left, so you are going towards the higher bit position, zeros entered into the vacated lower bit order. This is important.
Zeros are entered into the zero, zeros are entered into the vacated position, so we are done 0001. We have taken example if you left shift 001, but here I as I have to 0 enter into vacated position, zero will be entered, that is called as a.
Logical shift left, and if you are going for logical shift right, right means you are having 1000, then if you are right shifting, it becomes 1000. This is a vacated position, 0 will be there, so 0 will be untowed, zeros will be untowed into the vacated.
Lower or higher order, so unassigned arithmetic left shift by N correspondent multiplication by two raise to north, we have discussed and you are discarded with the bit overflow and all this one shift right is towards the less significant bit we have seen towards the LSB position or towards the right zeros are enter into the.
Higher order positions, unsigned values, right shifting N correspond to integer division by two raise to north. All this part we have seen, so this bit shifted output in plus carry in flex setting form. So when you are doing this operation and along with your yes suffix with your main instruction.
Also, it impact the carry flag.
Another some instruction which will be related to.
Yes, correct one. SHUBHAM correct one, correct.
If you are 8 bit, we are considering only 8 bit data, according to 8 bit. Otherwise, your 32 bit data is there. All 32 bits are there. I'm taking example for that purpose. OK, for that one, that is a logical, logical shifting left or right, zeros are entered into the faceted position. But now I'm telling.
This for this purpose, these are very important for arithmetic shifting. So A means arithmetic shift right. This is a rotate and this is a rotate with X rotate right with extended and rotate right rotate bits are there and that will be also towards the your.
Right, will be there. Here, it is interesting.
Arithmetic shift right, shift the right value, replicating the original sign a bit, replicating the original sign a bit.
So it will, if you are shifting the data, it will replicate the sign with whatever the your data previously available, that same bit will be forwarded as it is. That is important. So instead in a logical shifting, packeted position, we are inserting 0. So here instead of inserting 0 at a left,
because it is your right shifting, arithmetic right shifting, it is a replace or it replicate the original MSB bit as a signed bit. Got what? So it will preserve the previous value as it is previous value. Rotate, rotate the bit right so that bit shifted out.
Enter at the opposite end. OK, so lift, you can go for that. That will be important. Rotation is differ from shifting because here we are not discarding any bits. Here we are some discarding or you are using that one. Yeah, but we are taking that one. So here you are, I am giving this example.
You are able to see, please mute.

YASHWANTH V H .   1:18:25
Right.

KENDRE SANGMESHWAR SHANKARRAO .   1:18:29
Logical shift, logical shift left, so zero will be inserted to the vecketed position. What is the use of this MSB? Then MSB will be shifted towards the work carry flag. LSR right zero will be inserted into the vecket position to from the right side and...
your leftmost LSB bit moves into the carry flag. This is a LS, logical shifting. What is the arithmetic, important arithmetic here?
This bit is copied, this bit will be shifted here, 31 it goes to here and this will be remain as it is. OK, and B0 goes into carrying. That is the difference between logical shift and your arithmetic shift, right? What is the rotation? In a rotation, whatever the bit is 0, it will be copied.
and it will be inserted into the MSB and then your shifting will be occur and whatever the bit zero it will be copied into the your carry so it copy into the carry flag also and it will be inserted into the your MSB flag also miss in MSB bit also. RRX rotate right extended extended miss.
Carry's content will be more into the verbage with the carry, this rotation will be done with carry here without carry, it will be only B0 goes here later on, whatever the content comes here, it will be more here. OK, so that is rotate and rotate X rotation differ from.
Ting it here, we are not discarding anything.
Whatever the bits leave that right end enters it to the left, whatever the leave it enter into the left, whatever the leave enter into the left. OK, so this is a rotate and rotate extension, so it include the carry flag, it include the carry flag through the register, and carry flag is transformed.
Into the quantity, what did BALA GANESH this this presentation this PPT gives all your LSL, LSR, ASR, ROR, and RRX? This explanation related to that one. OK, so if you want to take a simple example, OK, so...
You are having a simple left shift operation if you want to perform, so I will give you the simple example of this one. So, if you consider this example, OK, OK, you are having the L, you want to do the addition, and if you want to do the shifting.
Okay. Previous.
R extended, right extended, right extended means whatever the content of carry flag, it will be inserted into the L34. Here only B0 goes into the carry flag. Later on, whatever the contents are coming, it will be rotated into the your B31 means.
Towards the Ms. P, Ankur got it.
OK, so we'll take a example.
So we'll take a example, simple example. We'll take R1 register, R1 register having a 10 value, R2 register having a three value. And if you want to do some left shifting operation with an, you have to do it addition. So you have to use such type of instruction, ad.
R0 is the destination register, R11 your operand one, R2 operand 2. Now I'm using a barrel shifter for that one. I'm using LSLLSL by two bits, LSL by two bits.
Miss what it does, it first performs the...
Sifting operation, shifting operation will be perform on a operand two. R2 content will be shifted by two times. What is the content of R2? R2 content is a three. Three means 00113. It will be rotated towards the left two times.
So what happens if left by two times? So this one comes here, again one comes here, this one comes here, again one comes. What is happened? 1100. If you are lifting 2 one times, what happens? Here you will get 0110. This is the first.
Left shifting, second lift shifting. OK, so here, here you will get this is the so you are having here 12, so this is a content. Now your R to contain is now 12.
Now, R1 continues 10, so R1.
Is A 10, R2 is a now 12, after shifting initially it is a three, after shifting it is 12, your addition will be performed and R0 will be becomes as a 22.
What to DINESH?
Do you understand this one? So this is called as a left shifting and simplest way. So as I heard it is I have done it to understand as a step by step. First left shift, right shift, but it is a best way. What is the original content? Original content is 3 multiply by how many times shifted 2 raised to two times.
Two raised to two times this multiply by two raised to two, two raised to two means 4, 4 * 3 means 12, so this is the understanding of this one, Abhishek.
So, first barrel sifter operation will be done and everything will be done in a single execution cycle. Everything will be completed in a single execution cycle due to that one. Ali is not involved in this. Ali is involved latter for adding only. Here, this will be done by the barrel sifter shift operation. So, R to contain is 3.
Three miss 0011. You have to left shift by two times. First left shift it becomes 0110. Second lip shift it becomes 1100. This is a second lip that is miss 8 + 412 or I have told simple way. What is the original content of?
R2 is 3, how many times your so left shifting is nothing but the multiplication by power of two, two raised to two, so 12 got it, and then it will be added with the your R1 and then this addition will be performed by the ELBIN does the perform operation, so everything will be done by.
In only one execution cycle, so this becomes very fast, OK?
So before going to the bitwise logic operation, we'll take a fastly program and in that I will explain, I will take another part which will be very important, that is addressing part and then we'll go for the program in that I have taken addressing part also. So bitwise logical operations are there.
This bitwise logical operations are similar to your simple logical operation, only it is a bit, and-ing, or-ing, okay, bitwise or-ing, bitwise XOR-ing, bitwise not or-ing, bit clear, bit field clear, bit field insert, and ME1 already we have gone through the ME1.
Logically negate all bits.
Then you have to move it, so here.
All these operations will be there, so Rd. is a you are having a destination register and content will be RN and operator single and it is a bitwise.
Or operation.
XOR operation, not operation, and not operation, you are having the bit will clear particular bit you want to clear it, reset it, you have to make zero, then you have to use a bit will insert if you particular bit if you want to insert, and that logical operations are there.
Okay, similar way I have told your additions, abstractions are there, which update the your carry or flex will be there. But these are the some of the instruction which plays the important role for only updating the condition flex.
So this call is a data comparison instruction. This very, very important instruction only will not give the any result, but it does the operation or it only set the your flags. So it will not does any other part.
So, means here it is given here, compare RN comma your operand 2 means maybe register or maybe your immediate value. It does the comparison, comparison it does means comparison means it does the subtraction in simple line in subtraction it does the subtraction result will be updating destination result.
your register, but here it will not give the result, only it sets the NZCV flag. Only flag will be set. It will not put the result into the result field, instead of only it will give the, or it will update the your flags.
So, due to that one, these are called as a comparison. OK, so here it does: R North minus your operand 2. Here, R North plus operand 2 means it will compare with a negative, compare and negative with, compare and negative with, so your means addition you are doing, and then it only make the...
Your output means that it will not does the addition, it will it does the addition perform, but it will not store the result. It will not store the result in R. There is no destination, so it will update only the carry flag. So, this due to that one, it is called as a.
You are what you are calling as a comparison instruction, another one test, test RN and your operand to test means it is a logical ending, logical ending operation you are performing and whatever the result of logical ending operation, whether result.
Is the setting any carry or not carry? Only NZCV flag will be updated, although there is no update of any result of this one. Test equal test and EQQ means test for equivalence. Again, here it will does the XORing for this RNN.
OP2 OK, operation 2 and only NZCB flags are updated. Only carry flag, only negative flag, whatever the result will be there, only that flag will be updated. There is no so such instruction. It is generally we can say comparison instruction and test.
instructions. These are used to generate the condition flags without keeping result in a destination register. We are not keeping result in destination register. We are doing only the ending operation, or in operation, ending, or in subtraction.
Addition operation, but we are not keeping the result into the destination. If you understand this one, please type one into the chart. Now, very, very important part is a memory addressing, and after that I will give you the demonstration of 1 C program, which is very important to understand, so...
This is used for relation purpose. Okay, this is called as a memory. So ARM Cortex M processor support flexible memory addressing. They provide 3 addressing mode, pre-index, post-index, and pre-index with update. Okay, and then we'll go one by one example with the help of this one.
Okay, so pre-index means someone in previous session it is asked when I explaining the C program, which C program I have converted into the S32 Design Studio into a disassembly window. In this assembly window, such instruction is there.
Maybe who is X? I don't remember the name. Do I explain this one? So I have told there is indexing, it's pre-indexing and post-indexing. So such instructions are there which will be used for giving the where you have to lower the address and where you have to miss whatever the loading the data or storing the address.
From the base address value plus or minus offset for that purpose, this addressing mode is very important. OK, so this is called as a pre-indexing. So, LDR R1 comma in a bracket R0 comma hash 4 in whenever in square bracket comes means this is a memory location.
Whatever the content of R0 is nothing but the memory location R0 gives the base address and hash for it is offset. OK, so what happens? Whatever the content of this memory location R0 plus one, this memory content memory content location means whatever the content of this memory location.
Will be moved into the R1 register, and whatever the content of R0 means that is the address, it is remain unchanged, is remain as it is, so this is called as a pre-indexing pre-indexing. OK, so this is a along, this is an important order, this is a it is not slow instruction, this is a.
Important to understand: these are actual cortex, your processor instructions are there. OK, so effective address. So here you are calculating the address, which is your effective address for moving the data, and this will be your R0 plus R, your four R0 for R4. This is your effective address of the memory.
And memory content of this effective address will be moved into the R1 register, and after moving the data, R0 content means this base address is unchanged, it remain as it is, so this is called as a your pre-indexing offset you are applied, but are you are not changing the data?
So, this is called as a.
Offset addressing, and here we are not changing the data for this one. OK, so this is offset means offset means it is common. You are having multiple base zero up to four bytes, 68 bytes, 12 bytes. In this way, you are going in a 4 multiples of four, 32 bit words data you are going.
For that one, this R0 on offset will be used for pointing, so R0 points this starting address, starting address, and it will pour it in a data which gives the offset at this actual from this base address, or you can say as a from this starting address.
where data will be available. So maybe this will be important in arrays, in a stack, in IO register. This plays important role. After that, you are having a.
Pre-index with update. So here when you add exclamation mark, if you add the exclamation mark, what happens? Here R0 is unchanged. Now R0 also changed. Means whatever the R0 previously, R0 plus 4, this is a memory address whose content will be moved into the R1 register.
Now R0 contents also becomes R0 plus R1 is address also updated. So due to that one, it's called as a with update. Whenever you add the explanation, both are pre-indexing, pre-indexing. So first point the memory location, move the data, and then you can move the data and then your.

DINESH KUMAR G P .   1:34:06
OK.

KENDRE SANGMESHWAR SHANKARRAO .   1:34:20
Base address will be depend upon you want to update or not update. Base address will be remain as it is, or it will be updated depend upon the instruction. OK, so this will be the your.
Preindex with the URL update. Now we are having post index post indexing. So here what is the format R0 comma hash 4. It is not in a bracket.
It is a non in bracket. Keep in mind, this is very important when you are going for the indexing. This is in a not outside of your closing your square bracket outside. So original R 0 is a memory access. Original R 0 is memory access, which will be updated by 4, which will be updated by 4.
So, here, what we are doing, copy the content of R0 means actual memory location which is pointed by your R0, move it, then update the your R0 base register, means first move, then update the your pointer.
So due to that one is called as a post indexing. Here, first move the data at pointed memo location. Don't update. Here, update. Here, post means first move without R, R0, R4 is not moved. Original content will be moved and after moving, your index will be updated. So index mostly using arrays.
When you are accessing the data, so you are having arrays of multiple your datas are available, so this is not that way, so you are having a arrays of data.
So, one, two, three, four. In this way, you are having some datas are there, and if you want to access, so here you require index. This is 0th index is the first index, this is the second index, this is your third index. In this way, this is the 9th index if you access. So, first I'm giving 0 is a base address.
Zero is a base address. This R0 is a base address of this one, and from this location I want to access the another mem location from 4th mem location, so I want to flip the data, so I will give plus four, so 0 + 4 and from 4th mem location it will start accessing the data, so this will be.
used for accessing the data from stack, from moment location, from the structure, from the IO register. So this is a very, very important for understanding this part. So I have given here with example, you are able to see this one. So with example I have mentioned with slide example.
So R0 having 0X200800. This is your RAM location address base memory offset is 4. So 0X200800. So this is R0 plus offset. It becomes 0X200800. And at this location, what you are doing?
Lower the content of.
R0 plus 4 memo location into R1. What is R1? So here it is a base address is nothing but 0x200. So this data is copied into the R1. So 5B6A7988. It will be loaded into the R1 register. This is called as a.
pre-indexing R0 remains as it is. It is not changing the base address. After loading the memory location, location contained into the register, content will be loaded. It is a 4 byte content. This is the first byte, second, third, third and four byte, 4 byte content into the R1 register.
This is will there. If you understand the pre-indexing, please type one into the chat.
So this base addresses remain as it is, only offset will be there. This will be as it is. This is a data, this is your memory. Now we are going for post indexing. In post indexing is similar way. OK, what it does.
It will not add the offset and it will not point that data. First, it moved it. It quad does LDR, lower the content of memo location pointed by R0 into the R1 and then you can update the R0 content in this way how to read. So what is R0 content? 0x200800.
Yes, R0, this will point here. What is the content have a link? 1F, 2E, 3D, and 4C. So this will be copied into the R1 register. And then your R0 points here, R0 points at this memo location. It points, R0 is now updated, added before, and now R0 is.
becomes 2X208004. So it is used for getting the data from array or from a stack for that purpose. You can use any method. So this is called as a posting. What is the difference between pre-indexing and post-indexing? But in post-indexing, offset will be added and from this memo location, data will be moved.
In a pre-index, in a pre-indexing first offset will be added and on updated data it will be copied and your base address remain as it is, unchanged in post-indexing base address data, whatever the base address pointing, this content will be moved into the R1 and then later on.
R 0 content will be updated. If you understand the pre and post dexing, please type 2 into the chat. Now we are going for the.
Pre-indexing with update only here, exclamation mark will be there. This is addition. If you add the exclamation mark, what happens? Zero R0 is 0X200 at a similar way, load the content of memo location pointed by R 0 + 4, so R 0 is nothing but the zero X200.
Eight 00 offset added plus four, so it becomes 0X2008 from this, whatever the data from this memo location 5A, 6A, 7A, it will be more into the R1R1 loaded with the pointed by the R0 plus offset. This data will be going to the R1 register, but...
Due to this explanation mark, okay, what happens? R0 content also updated. It memo location also updated with, so R0 becomes R0 plus offset. Okay, so in a post ending that update facility available in R0, immediately after transferring the data, it will update.
But here in a pre-indexing, it is your responsibility to update the R 0 if you want to update, then by putting this exclamation mark. So if you understand pre-index with update, please type 3 into the chart. Now we are going for.
Example.
And then we'll go for remaining part. Because this is important because in a pre-indexing, post-indexing, always you are referring the stack or you are referring some structure. At that time, that structure and stack will be inbuilt. It will be referred by this indexing mode. Please type 3 into the chat I have told.
If you understand this part, yeah, send them, yes.

CHANDAN KUMAR PAUL .   1:41:30
Sir, for the R1 you have taken 4 byte means you have 32 bits for that one only.

KENDRE SANGMESHWAR SHANKARRAO .   1:41:37
R1 which one? R1 is a register which is a 32 bit data. This is a memory location. Ha, it is a 32 bit data. This is your memory location. This is a starting address of the memory location. From this one you are having a third your 32 bit data will be there. Ha, this is a 8 bit data. This is 8 bit, this is 8 bit, this is 8 bit. OK.

CHANDAN KUMAR PAUL .   1:41:39
R1R1I.
Ohh, so that's why you have taken.
Thirty-two bit, okay, yes, yes, yes, okay, okay.

KENDRE SANGMESHWAR SHANKARRAO .   1:42:01
Sachin will go through the presentation. So I will go through this simulation with the killer and then I will show that part. Okay. So remaining only we are having a load. So these are the load related and store instruction with offset and all this one. Multiple instruction also you can go and then branch instruction and related to stack and then we'll stop here.
before that one, maybe I will take fastly the example for this one and then we'll go for this is a very, very important, very important. I am telling that one. So I will write to, so previously we have done this one, simple example, okay, of assembly programming, that C programming. Now I will go for the another example.
So what I will go, go to the project, new Micro Vision project. I will go for here, new folder. I will give this is a same example I will do addition, but it is having a ad under score 32 bit. I am giving the name for this project, this one.
Add under score 32 bit, but I'm putting the your operand into the memo location so that I will use this LDR pre-indexing for that purpose and also I will store the data into the memo location for that one. I'm using this part for explanation and as well as well as as well as.
I will use a variable. When you use a variable, variable is nothing but it is a part of your mem location that is your SRIRAMAN mem location. So how we can declare the data memory which is a read write table? We have seen previously is a code part. So we are now will go for the read write memory. Save it.
So, it will ask the your target, as I have told, we are to go for 144 and this will be.
I think you eat.
XXLLXLLX Okay.
So I'm not using any header file. If you, as a previously, also I have told you are having in device, you are having a startup file is there. If you want to use startup file for C programming, you can use it in startup file. We have discussed all your definition means your vector table, everything will be written in a.
dot C file assembly file and then it will hand over the control to the reset handler from reset handle it will hand over control to your main but here we are writing everything from a scratch so it is a programmer so it is my response name as a programmer's response you can both support packages are there we are not using CMSS if you want to use this one.
That is Cortex microcontroller software stack is available, middle will is available. This will be mostly used for makeup portable for that purpose. I'm not using this one. I will click OK. And you are able to see it is empty. Everything is empty here. Nothing will be there. So I will right click on this one. I will add a new.
Assembly file dot S file. I will give the name as a ad.
Add it due to time constraint. I'm fastly I already written the code, so I will use this code and I will show it for this one. You can go through that part, OK?
So same, you are able to see this one. I have given the comment or so, so that you can understand. Save it. OK, so first I have to preserve the eight means. We'll discuss when you are using any stack movement data or you are using a interrupt in your programming. Eight words location will be moved on a stack.
So stack will be arranged in that way. It will be given here. It we don't give that is there is no problem here. OK, it is a thumb already we have discussed. These are called as assembler directives. Then we have put here area. This is a reset area. I am putting only the address of a MSP and reset handler. So here I am designing dcb.
Define constant double word 0X20010. It is a my top of a stack and dcb reset handler. This is at this location we are having reset. So once your microcontroller reset or you can power on, this will be copied into the MSP and this will be copied into the program counter. We have seen that is a reset sequence.
So I have defined here, this is under vector, it is under the set section. Okay, so this is a data read only part, there is no code. And I have shown in a previous session, you know, write a code here, executable instruction, if you write, you will get a error because it is not a code area. So for program, I have written another code area. This is a area.
So this is a one area that is a data, which is a read only. It will not return. It is a part of your flash memory. Another area is called as a program code area. So here you are, I'm written area. This is my code. I have given name for this one, my code. This is a code area. Code means executable instruction, or it is called as a.
dot text section or dot code section. Okay, this is a section. We are calling this one. This is a data section. It is a read only section. This is my code section. It is also read only. Code is also read only. The entry point we have discussed. And this is the first reset handler. This is the first instruction to be executed. Now we have seen that one.
We have seen this instruction, whether this is a these two are pseudo instruction or real instructions. Please tell me, please give the reply. Now we are having only 10 minutes. Give the fast reply.

CHANDAN KUMAR PAUL .   1:47:29
A real instruction.

KENDRE SANGMESHWAR SHANKARRAO .   1:47:32
Chandan Kumar, whenever you are having equal sign, equal signs, so whenever you are having equal symbols, then it is called as a pseudo instructions. Yeah, so all these are the pseudo instruction, not a problem. So please keep in mind this one. What is the SRC? SRC is a label means it is a value.

BALA GANESH B .   1:47:36
Equal symbol is this.

CHANDAN KUMAR PAUL .   1:47:36
Okay, okay, good, a few.
Pseudo, pseudo, pseudo.

KENDRE SANGMESHWAR SHANKARRAO .   1:47:52
It is a SRC check SRC. What is the SRC? SRC is given. This is a data memory. It is also read memory. It is a part of your code memory, which is aligned at a four byte. Means every memo location is aligned with a four byte. At first location, I am having a 0xF002 0 data. Another location, I am having 0x110.
this data. So SRC act as a address of this first memory location. So this address will be moved into the R4 address of this, you are a dc, this 0x fdfde. This will be moved into the R4 register and is assigned to DST. DST means destination. DST I have defined in a.
My data memory. This is a my RAM memory. This is a RAM memory. I have given area my data data read write. I have given here access is a read write. So at which memory you are having read write access? Only SRIRAMAN memory you are having read write access. So I have defined.
This is a your section is called as a data section, dot data section or dot data your segment. OK, this is a data segment. In that data segment, I here reserve the four bytes, dc, and I have given initial by as a zero.
So whatever the result of this 32 bit addition, whatever the result comes, I will store this 32 bit result on my this destination, means in a RAM location and writing this result of addition. So what we are doing addition, these two numbers we are doing the addition and result will be stored at.
This memory location got it, so that memory location we don't know, so we are giving the label and that label's address will be assigned into the R file. OK, but it, if you understand up to this one, please type one into the chat.
One by one.
Up to this.
Please give the reply. And most of the participants are leaving the meeting from very, very important topic I have started. Due to that one, I have kept this is all towards the end because this is a very, very important. Whatever we have studied, that all the instructions are now we are revising here. Okay, so because this is a pseudo instruction, and what is this?
These are the real instructions. Chandan Kumar got it. These are Shivam. These are the real instruction. Now, here you are able to understand this is your post or pre-indexing.

CHANDAN KUMAR PAUL .   1:50:28
Uh, pre-indexing, H, uh, sorry, sir, post post-indexing.

KENDRE SANGMESHWAR SHANKARRAO .   1:50:28
Post or pre-indexing.

BALA GANESH B .   1:50:29
Texting.
Post.

KENDRE SANGMESHWAR SHANKARRAO .   1:50:33
Correct. This is a post indexing. First, move the content of R4. R4 means SRC. Move the first data into the, which register? Your R0 register. Then update the base address by 4, means it will update the base address of 0x100. And later on, I'm moving, I'm not giving here anything.

CHANDAN KUMAR PAUL .   1:50:33
The last.

KENDRE SANGMESHWAR SHANKARRAO .   1:50:55
So the R4 content, what is the R4 content now? Updated data, this memory location content will be moving to the R1. So first data goes into the R0, second data goes into the R1. So here I have used post indexing. Keep in mind this is important. Then I am doing the addition. So here.
Will do two way: first, we will do without S suffix, so I will remove this S. OK, I will save the code.
So, whenever S suffix will be there, your condition flags are updated. Whenever there is there is no S suffix, condition flags will not be updated. We will do, because here you are able to see F 1, F 1, so you will get a carry. So, F 1, F 1, here you are getting a zero, carry will be there, F 1 1, 1.
Carry will be generated. OK, so carry will be generated if you put here only add the carry will not be considered. Carry will not be there.
Okay, and whatever the result available, this result of 32-bit data will be stored under R5. What is R5? R5 is a DST. What is a DST? DST is nothing but the RAM location in SRAM location. This result will be stored. So here you have two types of memory. We are using program memory.

DINESH KUMAR G P .   1:52:10
In the free hour.

KENDRE SANGMESHWAR SHANKARRAO .   1:52:11
SUDHIR.
Please mute your program memory in that you are having.

DINESH KUMAR G P .   1:52:15
Yes, yes, yes.

KENDRE SANGMESHWAR SHANKARRAO .   1:52:21
DINESH, please, please mute.

DINESH KUMAR G P .   1:52:21
You should do it in a closer or closer there.
December.

KENDRE SANGMESHWAR SHANKARRAO .   1:52:29
Okay.
Got it, now we'll save it, build it.
During everything morning, now we'll go for this simulation.
Yes, what we have to do here, I have given directly, you have to go to the option target, use the linker, use the memory layout, go to the debugger, use the simulator. If you use a simulator, then otherwise you require a external hardware. Click OK, build it, and then go for the debugger.
Click on depot. Now you are in a simulator. All the registers are available here. You are able to see all the register here. XPSR, NZCV, QGT, IT, ISR. All these are available here. You are in a thread privileged mode, MSP. Now you are able to see what is the Ms. stack pointer. Stack pointer is 0x00.
One 00. What is the program counter? Program counter is 0X008, so 00 mem location. You are having first instruction. You are having the first instruction. So now this is a pseudo instruction. Pseudo instruction. This pseudo instruction is a replaced by assembler.
By the actual instruction, what I have told PC comma has 16 here, the it is a replaced by the your assembler replace that sudo instruction with your processors instruction. Now you are able to see what is R for nothing will be here changed by default 00. I will execute the first instruction. Now you are able to see.
R4 having a SRC instructions, your memory location data, where your first data is available. Then DST, now what is the SRIRAMAN location? Now you're able to see R5 is 0x200. From 0x200, you are having a memory location. We have seen in a setting I have shown in a previous session.
Now you have to point the memory location which is pointed by SRC, copy the first data and then update it. Now you are able to see R0 should be your 0XFFFF00. So now you can execute this one. Now got it. R0 should be 0XFF0020.
Now, what is R1? R1 should be your 0x111.
R1 should be 0x1111. Now you are doing the addition and here there is no yes and now you are able to see NZCVNZCV all are 00. It is expected that carry should be generated but carry is a generated but it is not updated the flag. So if you update this one.
execute this one, you are able to see there is no carry flag is set. And here result will be 0x10110030. This data you have to move into the R5. What is R5? R5 is your 0x200. Now we'll check in a memory location. I will add this.
0x2000000 and I will check at this memo location. There is no data available. You are able to see all are 00. SRAM contents are all are 00. Now I will execute. So after executing it will store the content this 1011003.
It will be moving to the your SRIRAMAN area. I will execute this one now. You are able to see it is coming into the SRIRAMAN 10110030 so we are moving the data into the SRIRAMAN area. There is no operation. I already an open no operation. This will be there now. I will execute same program. You are able to see.
There is no impact on a carry flag. If you change it, I will I will add a yes here. Yes, it OK, save it again, build it, save, build it, and now I will show what is the impact of this one. OK, now I will do fastly.
We are moving this one. Now you are able to see expand this one. NZCV00. Now when you will execute this ad yes instruction, you are able to see carry flag is set.
Are you able to see this one? Carry flag is set. Previously, carry flag is not set, and it will update this one. The result will be also available in a RAM memory and carry also set here. Carry flag is also set. Result will be moving to the your main location. OK, I will share this program. You can go through this one.
So this is a complete program in that I have defined what is data area, 2 data area which will be having a constant value which is not changeable. Okay, due to that one I have defined this data area and this your data area as a read only. One area is your code area.
Another data area is a read write area where I am storing the my result. OK, that is the SRIRAMAN part. OK, if you understand this one, please type yes into the chart. Now we are moving towards the remaining part of this one. Now got it, Sachin.
Such a money.
Yes, BALA, yes, no, yes means why? Yes, OK, no problem. You can type yes, only few parties out of 40 means I think 60 or 61 peoples are there. They near about 20 peoples left the meeting towards the end and which is a very, very important. OK, very, very, very important. They are left the meeting.
Maybe they will go through the recorded session then and then they will understand. Otherwise, it is a assembly. It is a difficult one for that one. OK, now we'll go for the part. What it and practice it, you are able to understand how this program will be gone through this one. So there is a multiple load instructions are available.
Load, it will be used for 32 bit. You can lower the 8 bit data also, byte, half byte, 16 bit also, that is called a half word. Then sign byte, sign half word. So this will be related to different loading the data from memo location.
You can load 32 bit data at a time, or you can load only byte by using this B or 16 bit data. So B means only byte, 8 bit data. H means half word, 16 bit data. Only if you use LDR, then 30 bit data. Chandan, you are getting this one.

CHANDAN KUMAR PAUL .   1:58:54
Understand.

KENDRE SANGMESHWAR SHANKARRAO .   1:58:57
Abhishek, not necessary everything that will be there. So whatever I have told, if you go through the basic concept of this one, you are able to understand all these things. It is a conceptual question will be there. Whatever I am taking theoretical part and all this one, if you go through the recorded session, if you go through the presentation, then definitely you are able to give the.
Answers of that questions. It is not. It is a professional studies are there. OK, so give the question bank and I'm asking the question from that question bank.
Then you are having load multiple words also. So instead of single 8 bit, 16 bit or 32 bit data, if you want to load 8.
32 bit word location and data memo location into the your registers. Yes, you can have a load multiple. You can load multiple data also.
Sachin, I have told such from session one to session 8, session one to session 8, all the contents are there.
And already contents are already given there in the syllabus. Already it is mentioned in a syllabus.
Store.
Store the content of register on a memory. So already we have taken that one, we have taken this one, store the data on your SRIRAMAN memory, it will be saved. Store byte also you can store 8 bit data, store 16 bit data, store multiple register on multiple memory location.
So this instruction will be available there. So as I have told, loading and storing with multiple registers, you are having a after storing, after loading, you can automatically increment. This option is available. Okay, forward is loaded and forward stored. You can increment the.
Memory address that is called as a incremented by 4. It is an increment after after moving the data you can increment increment before after means first move it before moving the data first increment and then move it. So this is the IB increment by 4 before.
you are loading the data or storing that one. DM is decrement and DBMS decrement before. So if you want, so store and load, all these will be supports increment after increment before, decrement after decrement before. So this will be storing the data in decremented and then it can store.
For accessing and storing purpose, this will be used when you are going for this one. Similar way, I will extend maybe 5 to 5 minutes. I will extend for a session. There is a branch and conditional instructions are there, so...
All these instructions, whatever the instruction we have seen, ARM has a support. All the instructions are executed conditionally. So add instruction is there. I have shown I have given only simple add instruction. If you take add instruction, so we are not putting any condition, but you can put a condition.
with add instruction also. So ARM support all the instructions execution with the condition and all these conditions are supported into the ARM. Equal, not equal, undersign higher or same, undersign lower, minus, plus, overflow, overflow clear.
Unsigned higher, unsigned lower, greater than, less than, greater than, less than sign always, or we will use.
Instruction as a ad.
So, in Advis, there is always AL is present. You don't require to write AL. This is always.
Otherwise, add equal to add. So, what is here here? So, if previous instruction result is equal, equal means zero flag is tested, zero flag is 1, zero flag is 1, then and then you can execute this as.
Add instruction, so for single instruction execution that is a conditional. If you want to execute conditionally, you can use these conditional instructions along with your instruction as a condition. So, if you want to execute this instruction, add if your previous result is equal or 0 flag is set.
Then and then instruction is executed, otherwise it is not executed, so every instruction in ARM you can execute conditionally. That is the advantage of ARM. OK, other other processor you are having a here also having a branch instructions are there, conditional instructions are there, jump carry, no carry.
But here you are having a facility, every instruction you can execute conditionally. OK, that is the importance of your ARM processor. OK, such branch instruction also available, branch to this memo location, OK, without without condition or with condition also you can branch.
So branch if equal, branch is not equal, means 0 flag is set, 0 flag is not set. In this way also, you can branch, means you can transfer the control to particular memo location with the help of branch. So this is also a very important instruction. Along that one, there is a branch L branch with.
Your link register, we have seen link register is all the address of the your return address mostly, as well as your code execution code, link execution code will be stored when you are using a for the interrupt purpose. OK, so BALA means branch with link, BLX branch with link exchange.
Okay, and BX means branch and exchange. So here exchange means you can mostly these are important for which supports the arm and thumb. You can go to the arm to thumb and thumb to arm for that one this plays the important role. Here these are also available.
For saving the data, these instructions are available, so only I will extend 5 minutes for stack, and then we will stop here. Stack is first in, first out data structure. You are pushing the data, means we are loading the data, and popping the data means you are reading the data from the stack, so it is a.
Leaf full means last in first out. Whatever the lastly you are pushing, you are able to read firstly. That will be part on stack is nothing but the it is a memory location. We have seen in a different controller, different memory locations are there. Here in a it is a SRIRAMAN location. If you go to the peak, there is a hard.
wired stack is available, special memory is available. If you need 051, it is a part of your RAM memory location. So here it is a part of your RAM memory location. It is a contiguous region of a data memory, which will be used for processor to hold your data on a stack memory. This will be available.
So can push the data two way it is a having stack. One is called as a descending stack. Descending means your data will you are pushing, you are going towards the lower addressing mode. Ascending means you are increasing the your addresses. So here decrementing, descending the stack.
Tap stack top goes on decrementing, so top will be addresses goes on decrementing. It is a descending, ascending. So when your data push it, stack grows downward, stack grows downward. Here top is available, stack grows downward and here it stack grows upward. So that is called as a push. So when you push.
is gross when you pop it to decrement. In a procedure and all these stacks will be plays important role. Here you are having a full descending, empty descending, full ascending, empty descending. Different types of stacks are there. So out of that, by default, you are having a full descending stack, ARM support.
by default pulled. All stacks are supported, but by default it is a full descending stack. What is a full descending stack means? In this way, you are having a stack base and you are pushing the, this is a stack pointer. Stack pointer is a address which holds the top of a stack. When you push the data, it decrement the memory location.
OK, so it is called as a descending. Again, you can push the data, it will descend it. OK, so here 2 concepts are there in full descending top is becomes base -1 empty base minus and whatever the data you are having, you are implementing the data and you are top empty ascending.
Ascending us, it points the empty memory location. Empty means empty. And if we are having a pull, it points to the last location which will be pulled. OK, that last location will be pushed. It will be pointing that one. So this is a full descending, empty descending, empty ascending will be there. So we'll go again if you want to go through the stack and if you are having any doubt, you can lastly ask me.
So this is a related to today's session. If you are having any doubt, you can ask me. So we'll stop here. Thank you.

KENDRE SANGMESHWAR SHANKARRAO . stopped transcription